THE THIRTY-SECOND MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/32)

(Honolulu, Hawaii, USA, 10-14 May, 2010)

Agenda Item 5:  Air Traffic Management (ATM) Issues

User Preferred Route (UPR) Updates

(Joint Paper Presented by Civil Aviation Bureau, Japan (JCAB) and Federal Aviation Administration)

SUMMARY

This information paper provides an update on the progress of UPRs in the Pacific.

- Introduction

  - The use of UPRs in the Pacific Region began in December 2000, as a replacement for Pacific Organized Track System (PACOTS) generated tracks between the United States and destinations in the South Pacific.  These UPRs are conducted without procedural constraints and have resulted in annual fuel savings of approximately 9.6 million kilograms.

- Discussion

  - Beginning in September 2007, operational use of UPRs between New Zealand/Caledonia and Japan was introduced.  Unlike the United States to South Pacific UPRs, these were introduced with the following Operational Requirements:

- Within the Fukuoka OCA:

  - Aircraft may file a UPR random route east of ATS route A337.

  - Aircraft on a UPR random route east of  Air Traffic Services (ATS) route A337 must join an ATS route by 30 North latitude (waypoints NOGAK, UPDOB, KAKNI)

  - Aircraft west of ATS route A337 must be on one of the established ATS routes.

- Within the Oakland OCA aircraft transiting through Guam CERAP must file reporting points on the Guam/Oakland boundary.

- Within the Guam Control Area (CTA):

  - Aircraft must be established on an ATS Route.

  - Aircraft must flight plan to avoid airspace released for military exercises, as charted or published in an International Notice to Airmen (NOTAM).

- Within the Port Moresby CTA aircraft may file UPR routings on or east of ATS route G205.

- Within the Nadi FIR, aircraft must be FANS equipped.

    - In March 2008, the restrictions within Guam CTA were relaxed.  Aircraft at or above FL310 no longer have the restriction to remain on an ATS route.

    - The annual fuel savings for the New Zealand/Caledonia – Japan UPRs is estimated at 2.1 million kilograms.

  - In February 2008, operational trials of North America to the Russian Trans-East Entry (RTE) UPRs began.  There are several Operational Requirements for the RTE UPRs, however the efficiencies have been extraordinary, with an annual savings of approximately 1.1 million kilograms of fuel.

  - In August 2008, operational trials of UPRs began between Hawaii and Asia.  Additionally, PACOTS tracks were being generated for those operators who wished to use them.  The Hawaii-Asia UPR procedures include the following Operational Requirements:

    - For eastbound flight planning between Asia and Hawaii -

- Operators must file one of the following Oceanic Transition Routes (OTR) and connect to appropriate ATS routes:

  - KAGIS OTR11 DRIVR OTR22 ETRON;

  - KAGIS OTR11 RIPKI/GARRY;

  - VACKY OTR13 SEALS;

  - SMOLT OTR15 MORAY;

  - FORDO OTR17 FERAR; or

  - at or south of MERED G223 TONIK.

- Eastbound UPRs may flight plan on PACOTS track 3 and then diverge so as to remain at least 50 nautical miles (NM) south of PACOTS track 3.

- Aircraft on PACOTS tracks between Asia and the Continental U.S. have priority over Asia-Hawaii UPR aircraft for altitude assignment.

- Aircraft on PACOTS tracks between Asia and Hawaii do not have priority over Asia-Hawaii UPR aircraft for altitude assignment.

- The UPR route must be flight planned to remain laterally north of ATS route A450 or flight plan via A450 between TAAVR and KATHS.

- The UPR shall be planned to incorporate a published waypoint on the Honolulu control facility boundary.

- The UPR shall be planned to avoid military special use airspace when active.

- The necessary information regarding available Hawaiian gateway fixes will be sent to participating aircraft operators prior to the generation of PACOTS tracks 11/12.

- The UPR shall terminate with the appropriate Hawaiian Standard Terminal Arrival Route (STAR).

    - For westbound flight planning between Hawaii and Asia -

- Aircraft on PACOTS tracks between Asia and Hawaii do not have priority over Hawaii-Asia UPR aircraft for altitude assignment.

- The UPR shall begin with the appropriate Hawaiian Standard Instrument Departure (SID).

- The UPR route must be flight planned to remain laterally north of ATS route A450 or flight plan via A450 between KATHS and TAAVR.

- The UPR shall be planned to avoid military special use airspace when active.

- The necessary information regarding available Hawaiian gateway fixes will be sent to participating aircraft operators prior to the generation of PACOTS tracks A/B.

- The UPR shall be planned to incorporate a published waypoint on the Honolulu control facility boundary.

- Aircraft departing Hawaii after 2330 Coordinated Universal Time (UTC) must flight plan so as to remain 50 NM south of PACOTS track F.

- In the Fukuoka FIR, operators shall file appropriate ATS routes and connect to one of the following OTRs:

  - CALMA G344 CELIN OTR9 ABETS OTR11 SCORE;

  - ETRON OTR9 ABETS OTR11 SCORE;

  - RIPKI/GARRY OTR11 SCORE;

  - SEALS OTR13 VACKY;

  - MORAY OTR15 SMOLT;

  - FERAR OTR17 FORDO; or

  - at or south of TONIK G223 ADKAK.

    - The annual fuel savings for the Hawaii-Asia UPRs is estimated at 2.9 million kilograms.

  - In May 2009, operational trials of UPRs began between RJAA and YSSY/YBBN/YBCG/YBCS with the following Operational Requirements:

- The North and Southbound UPRs must remain in the Fukuoka, Oakland, Guam, Port Moresby, Honiara and Brisbane FIRs.

- The flying time between filed reporting points must not exceed 80 minutes.

- Aircraft routes must include filed reporting points for each Area Control Center boundary crossing point in the FPL.

- Aircraft must flight plan to avoid airspace released for military exercises, as charted or published in an International NOTAM.

- Within the Fukuoka CTA:

      - The UPR paper routes will start or end at the Waypoints along latitude 30 North and follow existing conventional ATS, departure and arrival routings north of 30N.

      - Aircraft may file a UPR random route at least 50nm east of ATS route B586, or:

      - Aircraft on or west of ATS route B586 must be on one of the established ATS routes in the Fukuoka CTA.

- Within the Guam CTA aircraft may flight plan UPRs at or above FL310.  Aircraft at FL300 and below must flight plan via ATS Routes in the Guam CTA.

- Within the Brisbane FIR:

- To integrate UPR traffic with domestic traffic operating along the eastern coast of Australia (e.g. Brisbane – Cairns), the following gates will be used for Japan/Australia UPRs:

    - “150 NM Cairns” (i.e. BENKI, MEMIG, PUPEB), HM, KELPI, or SETER

    - Aircraft departing Australia are to plan ATS routes (or CS DCT BENKI) to the gate then UPR

    - Aircraft arriving Australia may plan UPR to the gate then via ATS route (or BENKI DCT CS)

    - The projected annual fuel burn savings from allowing UPRs between these city pairs is 1.9 million kilograms.

  - Operational trials involving the use of UPRs for aircraft that would normally utilize PACOTS Tracks 14 and 15 began in September 2009.  The following is a list of procedures that operators must use when flight planning PACOTS Track 14/15 UPRs between Asia and United States.

    - In the Fukuoka FIR, operators shall file one of the following oceanic transition routes (OTR) and connect to appropriate ATS routes in domestic airspace:

    - KAGIS OTR11 DRIVR OTR22 ETRON;

    - KAGIS OTR11 RIPKI/GARRY;

    - VACKY OTR13 SEALS;

    - SMOLT OTR15 MORAY;

    - FORDO OTR17 FERAR; or

    - at or south of MERED G223 TONIK.

- PACOTS Track 14/15 UPRs crossing 160E prior to 1600 UTC must flight plan so as to remain at least 50NM south of Track 3 or flight plan on PACOTS Track 3 and then diverge so as to remain 50NM south of PACOTS Track 3.

- PACOTS Track 14/15 UPRs crossing 160E after 1600 UTC may flight plan the UPR within the Fukuoka and Oakland FIRs without regard to other PACOTS Tracks.

- The UPR shall be planned to avoid military special use airspace when active.

- The UPR shall be planned to utilize one of the following North America Gateways:

- NATTE

- MITOH

- LATNE

- VESPA

- UNVER

- TRYSH

- SHENU

- FATMO

- DACEM

- CEPAS

- BUTEN

- ALLBE

- AUDIA (Must connect to AUDIA via a route that is south of 37N130W)

- The UPR shall flight plan via the appropriate SID/STAR.

2.5.1	There are no metrics for the PACOTS Track 14/15 Operational UPR fuel savings.  Operators are requested to provide JCAB and the FAA with the savings data form their Track 14/15 UPR flights.

  - Operational trials for the use of UPRs by aircraft that would normally utilize PACOTS Tracks H and I began in April 2010.  The agreement for UPRs in association with Tracks H/I involves Anchorage ARTCC, Oakland ARTCC and JCAB ATMC and includes the following Operational Requirements:

- The UPR must enter oceanic airspace over one of the existing waypoints on the Oakland Oceanic FIR boundary along the west coast of North America.

- Flights must join a NOPAC ATS Route when transiting the Anchorage FIR.  Join the route east of the waypoints listed below:

- R591	AKISU

- G344	CARTO

- Flights entering the Fukuoka FIR from the Anchorage FIR must be established on a NOPAC ATS Route and cross AKISU/CUTEE between 0000Z and 0600Z.

- Flights may flight plan on PACOTS Track F and then diverge so as to remain 50 NM south of PACOTS Track F.

- In the Fukuoka FIR, operators shall file one of the following oceanic transition routes (OTR) and connect to appropriate ATS routes when entering domestic airspace:

- ETRON OTR9 ABETS OTR11 SCORE;

- RIPKI/GARRY OTR11 SCORE;

- SEALS OTR13 VACKY;

- MORAY OTR15 SMOLT;

- FERAR OTR17 FORDO; or

- TONIK G223 ADKAK.

- When transiting the Okinawa area, flights shall use one of the following routes:

- BIXAK DELTA ONC flight plan route to destination;

- SABGU R583 OK flight plan route to destination;

- 23N140E (or south) TUNTO IGURU flight plan route to destination; or

- 23N140E (or south) TUNTO R595 SEDKU flight plan route to destination.

- The UPR shall be planned to avoid military special use airspace when active.

- The UPR shall flight plan via the appropriate SID/STAR.

- PACOTS Track H/I UPR aircraft do not have priority for altitude assignment over aircraft on an existing PACOTS or Central East Pacific (CEP) traffic.

  - Paper Trials for PACOTS Track 1 aircraft has been proposed for Spring and Winter 2010.

  - ANSP Operational Requirements are necessary to manage air traffic operations can create a loss of efficiency for aircraft operators choosing to file UPRs.  ANSPs need to review Operational Requirements as they gain more experience with each UPR set and reduce or eliminate Operational Requirements if they are no longer required.

- Recommendations

  - The meeting is invited to:

    - Note the successful implementation of these UPRs and the significant fuel savings and reduced environmental impact achieved.

    - Discuss UPRs and ways to improve their efficiency.