THE THIRTY-SECOND MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/32)

(Honolulu, Hawaii, USA, 12-13 May 2010)

Agenda Item 5:  	Air Traffic Management (ATM) issues

Considering the implementation of the operational trial of the 30NM Longitudinal Separation Standard based on the ADS between Oakland FIR and Fukuoka FIR

(Presented by the Civil Aviation Bureau, Japan)

SUMMARY

This paper provides the status of consideration the implementation of the operational trial of the 30NM longitudinal separation between Oakland FIR and Fukuoka FIR.

- Introduction

  - JCAB planned in a phased manner for implementing the ADS 30/30 separation standard in the Fukuoka FIR as follows;

Step-1: Trial operation to the Climb/Descent in the Fukuoka FIR

Step-2: Trial operation to the Cruse phase in the Fukuoka FIR

Step-3: The regular operation of 30/30 in the Fukuoka FIR

Step-4: To apply it to the crossing-FIR boundary traffic between the neighbouring FIR.

  - In August 28 2008, JCAB began an operational trial of 30NM longitudinal separation were applied only between RNP4 capable aircraft to enable same direction aircraft to climb or descend in Fukuoka FIR as Step-1. In December 1 2008, the trial was forwarded, 30NM longitudinal separation were applied only between RNP4 capable aircraft to enable same direction aircraft to the cruse phase as Step-2.

  - JCAB recognize that some issues are still existed to forward to the next step, which has intermittent problems with satellite communications due to un-notified or unplanned outage, network load of satellite data link system, and low percentage of flights for capability of the ADS 30/30 separation.

  - The percentage of RNP4 capable aircraft have still maintained low in Fukuoka FIR (23.2 % in March 2010). At IPACG/31, JCAB stated it predicts that Step 3 will depend mostly on the rate of RNP4 aircraft equipage and approvals, and given that it is hard to estimate this, a date for the commencement of Step 3 (assessment/simulation) is not determined at this time. Also, JCAB stated it would be considered, along with FAA, how and when to start introducing seamless cross boundary operations within this trial.

- Discussion

  - At IPACG/31, FAA stated that neither the availability nor the reliability satisfied 99.9%. Also, FAA stated that current data link performance does not justify near-term planning for an expanded application of 30/30 including the operation of 30 NM route structures within the Oakland Oceanic CTA.

  - JCAB recognize that it is necessary to take into account of the availability of data link communications and the unplanned outages not only in Fukuoka Oceanic CTA but also Oakland Oceanic CTA, and JCAB would like to decide the target date to implement ADS 30/30 as a regular operation including 30NM route structures within Oakland and Fukuoka FIR in step with FAA.

  - Originally, JCAB plan to implement seamless operation of the ADS 30NM longitudinal separation between Oakland and Fukuoka FIR after implementation of ADS 30/30 regular operation in Fukuoka FIR. But it would be difficult to improve current situation in the near term, JCAB will consider and implement measures to introduce seamless operation of the ADS 30NM longitudinal separation between Oakland and Fukuoka FIR as a trial before the regular operation.

  - To move smoothly into seamless operation of the ADS 30NM longitudinal separation as trial phase, JCAB recognize it is necessary to study or review prerequisite for the smooth implementation of seamless operation of the ADS 30NM longitudinal separation between Oakland ARTCC and Fukuoka ATMC. JCAB would like to decide the date to implement of seamless operation of the ADS 30NM after making the necessary amendments to the plan with FAA.

- Action by the meeting

  - The meeting is invited to note the provided information in this paper.