THE NINETEENTH MEETING OF THE IPACG FANS INTEROPERABILITY TEAM

(IPACG FIT/19)

(Honolulu, Hawaii, USA, 11 May 2010)

FANS over HFDL Trial

(Presented by ARINC)

SUMMARY

The trial data is showing HFDL to be a viable media for ATC communications.

- Introduction

  - Oakland ARTCC supports the use of FANS-1/A for surveillance and ATC communications within the KZAK FIR.  However, due to concerns regarding message latency only VHF or SATCOM may be configured as the primary communication media; HF should be configured as a tertiary media.  This limitation was imposed because the observed message delivery success rate and latency times failed to meet the criteria in the FANS-1/A Operations Manual (FOM).

  - In addition to the changes made to the HF sub-network, another factor considered when deciding to reevaluate the use of HF as a primary delivery media for ATC datalink communication and surveillance messages was an observation that the previous performance monitoring may not have been an accurate measurement of the media.  This flaw in the analysis was due to way SATCOM-equipped aircraft are configured to attempt delivery of datalink messages.

The typical aircraft configuration for media use: VHF  SATCOM  HF (as equipped).  To date, all aircraft operating in the Oakland oceanic FIR that are equipped with a FANS-1/A package are also equipped for satellite communications.  When the aircraft are out of VHF range they will first attempt to deliver a datalink message via SATCOM.  If this fails the aircraft will then attempt to deliver the message via HF.  When calculating latency times for message received via HF, it is impossible to determine what portion of the time was spent attempting to deliver the message via SATCOM.  This “SATCOM delivery failure burden” means that most messages received via HF have unacceptably large transit latencies.

Hawaiian Airlines has equipped 18 B767 and  with FANS-1.  Unlike the typical datalink capable aircraft, these aircraft are not equipped for satellite communications.  However, Hawaiian Airlines’ Datalink Service Provider, ARINC, does support HF datalink.  The Hawaiian Airlines B767 configuration for media use will be VHF  HF.  This means that when the aircraft are out of VHF range the primary media for message delivery will be HF.  The trial will take advantage of this unique configuration to measure the latency times for ATC datalink messages delivered via “pure” HF.

Finally, as noted above, previous performance has been measured against the criteria listed in the FOM.  Oakland ARTCC is now using the standards set forth in RTCA DO-306 to evaluate datalink performance within the KZAK FIR.  Given the improvements to the ARINC HF sub-network, the HAL B767 equipment configuration, and change in performance criteria a new evaluation of the use of HF datalink for ATC purposes is justified.

  - Description of Trial

Aircraft

Participating aircraft will be Hawaiian Airlines B767 with a certified FANS-1/A package and upgraded HF data radios.

Operations

There are no special operational requirements necessary during the trial.  All ATC datalink operations are to be in accordance with the FOM.  Even if otherwise qualified, the flight plan (FPL) for participating aircraft will not indicate RNP4 capability in field 18 (i.e. NAV/RNP4).

Phases

The trial will be conducted in several phases; Airline Operations Center (AOC) messaging, ADS messaging, CPDLC messaging.

The first phase of the trial will measure latency times of messages exchanged between the aircraft and the AOC.  These measurements will be made by ARINC.  During this phase all communications between the aircraft and ATC will be done via HF voice radio.  This phase was completed in November of 2008.

One phase of the trial will measure the latency times of ADS messages.  These measurements will be made by the FAA.  During this phase, the aircrew will log onto the Oakland ARTCC ground system (KZAK) by transmitting an AFN Connection (FN_CON) message.  The Oakland ground system will respond with an AFN Acknowledgement (FN_AK), a CPDLC Connection Request (CR1), and an ADS contract request.  Once the aircrew observes that ATC communications are established (on the ATC LOGON/STATUS page) they will terminate the CPDLC connection by selecting ATC COMM OFF on the ATC LOGON/STATUS page.  This will terminate the CPDLC connection but leave the ADS contracts active. During this phase, all clearance requests and clearances will be done via HF voice radio.  All position reporting requirements will be fulfilled by the ADS messages. Unless specifically requested by ATC, position reports will not be made by HF voice.  While the standards set forth in DO-306 will be the criteria used when evaluating the success of this phase, there will be no requirement to meet the latency time standards prior to moving to the next phase.  This dispensation is being made in recognition of the fact the use of the HF voice radios can negatively impact the latency of datalink messages.  To minimize this impact, the aircrew should limit the use of HF voice communications.

Another phase of the trial will add CPDLC messaging.  During this phase the latency times of ADS and CPDLC messages will measured by the FAA.  The logon procedure will be the same as that used during ADS phase.  However, the aircrew will not terminate the CPDLC connection.  All ATC communications will be done via datalink.  HF voice communications should only be used for unusual situations (e.g. MEDLINK).  The aircrews should be mindful that a portion of the measurements taken during this phase include crew response to ATC clearances (see Figure 1 and Table 1 below in the criteria section).

- Challenges of the current trial.

  - The trial has encountered operational issues and protocol anomalies that are being addressed. Those items that have already been addressed have yielded significant gains in transit times for messages.  The current standing challenges are outlined below.

  - VHF to Oceanic Media Fringe Issues. This issue is endemic to the gateway points regardless of media involved.  HAL/ARINC continues to explore the VHF to Oceanic Media Fringe Issues. One technique that HAL is exploring is to MAP the fringe areas and apply that map to the CMU to smooth out these transition issues. A certain amount of improvement in data link performance has been achieved with crew mitigation. By procedure, crews will de-select VHF as an operable media on the outbound forcing HFDL to be the primary media. On the inbound – the reverse will occur allowing the oceanic media to operate until VHF attains a quality signal.

VHF reception at the oceanic gateways. Marginal VHF reception at the oceanic gateways produces a mix level of performance. Oceanic gateways that have strong VHF signal allow for VHF to be the primary media for the initial gateway position reports. HAL/ARINC has discovered certain oceanic entry points tend to be less than optimal for VHF operations. The aircraft altitude when crossing this waypoint can make a difference with higher being better.

Tail-to-Station Antenna Issues - It has been recognized that the optimum frequency and channel auto-selection struggles to maintain connectivity with an antenna that is directly aft of the aircraft due to aircraft antenna characteristics.  Any change in heading will restore the signal strength and make that frequency/channel available for use. Program logic and frequency auto-selection may generate temporarily loss of connectivity as the airplane direction changes causing the search, seek and connecting process to work overtime. During these periods, connectivity is lost.  ARINC is investigating using Near Vertical Incidence transmissions to address this issue.

Message Priority – The process used by the CMU to prioritized AOC and ATC messages can make a difference in the latency of ATC messages. By default – ATC messages have some level of priority however if an AOC message is placed in the queue first, this may pre-empt a following ATC messages. When crews operate with CPDLC only, they can by procedure place the ATC downlink out before any AOC message to assure a first-out priority. When ADS is used, crews have no control to outgoing ADS messages which can conflict with other data link communications. The end results – ADS performance  suffers.

HF versus voice – During this trial, HAL has had voice requirements that have interfered with data link connectivity. The fact the voice shares the same antenna as data link and that only one can operate at the same time presents certain challenges. HAL and ARINC continue to explore voice operations that can be handled via data link to reduce the lost connectivity when conducting voice operations. These operations include AOC communications, Medlink and ATC voice communications.

ATC simultaneous voice and data link communications – On occasion, HAL flights will receive both a data link (uplink) and voice (SELCAL) ATC clearance. This presents a dilemma for crews in how to complete the transaction. On occasion, ATC clearances uplinked via data link need to be acknowledged via voice due to the lost of downlink capability. The ARINC COM Center (SFO Radio) seems to handle these transactions very well.

Suspension of ADS operations – It was decided in March 2010 to discontinue the use of ADS and operate with CPDLC only. Data link performance has improved dramatically since that program was instituted.

AFN Protocol issues – The logon process transfers 3-5 uplink contracts to the aircraft after the AFN logon.  The issue is the contracts are sent immediately after the logon is confirmed.  Uplinking each contract is a serial process however the T-0 marker is the same for all contracts.  Owing to the faster speed of VHF and SATCOM technologies, this has not been a problem. However HFDL, being an inherently slower media, the uplinking process takes longer.  If the issuance of each contract were to occur simultaneous; rather each contract should wait for acceptance from the aircraft rather than the ack from the service provider gateway.  The RCP measures the complete path from host to aircraft and back. The current process is not accurately measuring the round trip results because the queue time is not accounted.

CMU Sleep mode – There is a disparity between ARINC 635 and ARINC 618 specifications that during channel changes, the CMU can enter a 10 minute silence for sending messages to the delivery media.  This has been documented at the AEEC committee levels and vendors have committed to incorporating the fix in their VDL Mode 2 units.  However, patches to the existing non-VDL fleets are not being issued.  This data can be readily identified and eliminated from the data set.  This was done during the HFDL NAT trials several years ago.

FMC congestion - It has been observed by crews that a condition exists where by the HFDL radio is connected to the network, but an FMC generated message is placed in the CMU queue and never leaves the aircraft.  AOC messages generated after the FMC ATC message do successfully leave the aircraft.  These messages can be documented by the crew and removed from the data set as well. Honeywell is investigating this issue. In the meantime, crews will begin each flight by performing a CMU reset.

2.3   2010 results to date:

- Recommendations

  - The meeting is invited to note the information in this paper.

  - The April data shows the addressing of issues identified in Paragraph 2.2 show a steadily improving performance of the FOH trial.  The data continues to suggest the viability of the use of HFDL for ATC communications.

  - After CPDLC operations are approved for HFDL, ADS phase of the trial will be added to determine if it conforms with the RCP criteria outlined in DO-306