THE THIRTY-SECOND MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/32)

(Honolulu, Hawaii, USA, 10-14 May 2010)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Tailored Arrivals

(Presented by the Federal Aviation Administration)

SUMMARY

This paper provides an update on the use of Tailored Arrivals in the United States.

- Introduction

  - The FAA has been conducting trials for use of a Tailored Arrival (TA) procedure since November 2006 at San Francisco International Airport (KSFO).  The procedure is initiated by a pilot request to air traffic control (ATC) while operating in oceanic airspace and utilizing Controller Pilot Data Link Communications (CPDLC).  If available, the controller sends the aircraft a clearance that includes a descent profile with speed and altitude restrictions, as applicable.  The aircraft then uses its Flight Management System to fly the most efficient descent to the arrival airport.

- Discussion

  - Since the initial trials began, approximately twelve hundred (1200) full TAs and twenty-three hundred (2300) partial TAs have been flown into KSFO.  During that time, the profile has been refined to increase efficiency while meeting the needs of air traffic control.  Due to the dynamic nature of air traffic, especially in the approach control environment, conditions are such that completion of the full arrival profile is not always possible.  However, significant fuel savings are being gained from both the full and partial TA.

  - When the full TA is flown, the fuel savings realized is between 592 and 1041 kilograms (kg) of fuel, while the partial TA has an associated savings of between 172 and 500 kg.  The KSFO Pacific 1 TA continues to be very successful and the trial is being continued.

  - The FAA Administrator has acknowledged the success of the Tailored Arrival and has placed emphasis on development of similar procedures at other locations.  Los Angeles International Airport (KLAX) and Miami International Airport (KMIA) were the next airports to have Tailored Arrival Trials.  Work has continued to enhance the current procedures.  Work is underway to develop FAA standards for Tailored Arrival development.

  - The Catalina One TA into KLAX is providing a more efficient alternative to the current LEENA Arrival.  It is a product of the collaboration of the FAA, Boeing, NASA and the operators.  Operational trials of the KLAX TA are currently underway and although the current profile requires a segment of level flight by SXC, the operators are reporting good fuel savings.  The Catalina 1 TA and standard oceanic arrivals utilize C1177, a narrow corridor of airspace between military operating areas.  The departing and arriving aircraft conduct their climbs/descents in this corridor. Los Angeles Air Route Traffic Control Center (ARTCC) is working to develop a departure procedure that will offset the arrivals from the departures.  When this departure procedure is completed, it will help provide a more efficient descent profile for the Catalina 1 TA.  Work continues on refinement of the Catalina 1 TA profile.

  - The Florida Eight TA and Florida Nine TA were developed for arrivals into KMIA.  The complexities of the traffic flows into and around KMIA have been obstacles to the development of optimal descent profiles.  Work is continuing, and refinement of procedures that will allow for a more efficient profile is underway.

  - The FAA is committed to the continued development of the TA program.  It has become a significant part for the Next Generation Transportation System (NextGen) Airspace Modeling of Optimal Profile Descents.  Plans are underway for the timely implementation and integration of Initial Tailored Arrivals (ITA) into the National Airspace System by early 2011.  ITA is the designation for the initial implementation of TAs at select coastal city airports in the United States.

- Recommendation

  - The meeting is invited to note the information in this paper.