THE THIRTY-SECOND MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/32)

(Honolulu, Hawaii, 10-14 May 2010)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Status of 30 NM Lateral and 30 NM Longitudinal (30/30)

Separation in Oakland Oceanic Control Area (CTA)

(Presented by the Federal Aviation Administration)

SUMMARY

This paper presents a review of the operational trial of 30 nautical miles (NM) lateral and longitudinal separation in the Oakland Oceanic Control Area (CTA).  It reviews the policies being applied and includes a brief review of the findings of the Oceanic Separation Reduction Working Group (OSRWG).

- Introduction

  - Since June 2007, the Oakland Air Route Traffic Control Center (ARTCC) has applied 30 NM lateral and 30 NM longitudinal (30/30) separation and 50 NM longitudinal separation between appropriately authorized and equipped aircraft throughout the Oakland Oceanic CTA.  The policies for the application of the separation minima are detailed below:

    - 30/30 and 50 NM longitudinal separation is applied to “targets of opportunity” throughout the Oakland Oceanic CTA.   “Targets of Opportunity” are proximate pairs of aircraft that are both eligible for either 30/30 separation or 50 NM longitudinal separation.

    - Published air traffic services (ATS) routes and other tracks (e.g. Pacific Organized Track System) continue to be laterally separated by a minimum of 50 NM.

    - 50 NM remains the minimum Automatic Dependent Surveillance (ADS)-based lateral and longitudinal separation applied between 30/30 eligible aircraft and Required Navigation Performance 10 (RNP 10) aircraft.  Lateral and longitudinal separation standards applied between RNP-10 and non-RNP aircraft also remain unchanged.

    - Oakland ARTCC accommodates operators that are not eligible for 30 NM separation throughout Oakland Oceanic CTA.  Lateral, longitudinal and vertical separation minima for aircraft not eligible for 30 NM separation are unchanged.

  - On 28 October 2008, the trial with Civil Aviation Bureau, Japan (JCAB) to use 50 NM longitudinal separation was extended to the entire Oakland/Fukuoka boundary.

  - In October 2008, Anchorage ARTCC began operational trials of 50 NM longitudinal separation applied to “targets of opportunity”.  Anchorage ARTCCs ability to participate in ADS-C separation permits uniformity across all of the U.S. controlled Pacific Flight Information Region (FIR) boundaries.

- Discussion

  - The FAA Oceanic Separation Reduction Work Group (OSRWG) evaluates the operational and technical issues related to 30/30 operations, including the performance of the satellite data link system.  It includes subject matter experts from air traffic, flight standards, aircraft certification and safety analysis.  The OSRWG was responsible for recommending to FAA managers the readiness to start 30/30 operational trials and the policies to be applied.   The OSRWG meets at regular intervals to assess the status of operations.

  - Current data link performance, evaluated against OSRWG Scrutiny Group criteria, does not justify near-term planning for an implementation of 30/30, which would include the operation of a 30 NM route structure within the Oakland Oceanic CTA.

  - The FAA is currently conducting airspace and benefits analysis in support of 30/30 operational trials within the Anchorage CTA.  The purpose of this analysis is to provide fundamental information about the airspace under consideration for separation reduction and enhanced communication and navigation technologies. This analysis is intended to provide airspace planners, separation analysts and decision makers with a foundation for further analyses and assessments.

- Recommendation

  - The meeting is requested to note the information in this paper.