THE THIRTY-SECOND MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/32)

(Honolulu, Hawaii, USA, 10-14 May, 2010)

Agenda Item 5:  Air Traffic Management (ATM) Issues

User Preferred Route (UPR) between Australia, New Zealand and Japan

(Presented by Civil Aviation Bureau, Japan)

SUMMARY

This information paper provides relax restrictions on UPRs between Australia, New Zealand and Japan.

- Introduction

  - JCAB have been concerning to operator’s benefit, problems on ATS services, system improvement requirement for UPR operation between Australia-RJAA (Narita airport), New Zealand-RJAA.

  - UPR operations between New Zealand and Japan have been operating from September 2007.

  - Paper trial carried out from April to May and from October to November 2008.  UPR trial operation began in May 2009, within the area of 50 NM east of airway B586 in Fukuoka FIR as agreed in IPACG/29.

  - Regular UPR operation between Australia and Japan began in May 2009, with Memorandum of Understanding (MOU).

  - JCAB presented that “other areas are currently being studied for UPR expansion” in IPACG/31.

  - The current regular UPR operation between Australia and Japan, no significant impacts to ATC operations.

  - A simulation said that if the UPRs use the area includes around ATS route A597, reduce 2 minutes and save 400lbs a flight, in other words save 15,000kg fuel and reduce 45,000kg CO2 a year.

- Discussion

  - JCAB planning to relax restrictions and expand the area for use of UPRs.  The UPRs will start or end at the waypoints along latitude 30 North, and planning to expand the UPR applicable area to 50 NM east side of ATS route A597 (exclusive) by July, 2010.

  - JCAB is considering the additional airport in the direction of Australia and New Zealand from Japan step by step.

- Recommendations

  - The meeting is invited to:

    - Some restrictions still required for using UPRs in Pacific airspace.

    - Discuss UPRs and ways to improve their efficiency.