THE THIRTY-SECOND MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/32)

(Honolulu, Hawaii, USA, 10-14 May 2010)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Southern California Special Use Area (SUA) Airspace

(Presented by the Federal Aviation Administration)

SUMMARY

This paper discusses the issues surrounding the make-up of the special use airspace off the coast of Southern California.

- Introduction

  - The special use airspace (SUA) along the Southern California coastline are extensively utilized for a variety of evaluation testing and training with live military munitions.  The SUA spans the entire coastline from Big Sur to Baja California and is made up of eight contiguous areas.  The mauve shaded areas in the graphic below indicate the airspace included in this discussion:

  - Control extensions and corridors created for access to Oakland Oceanic Control Area/Flight Information Region (OCA/FIR) by transoceanic flights, are wholly contained within the SUA, with the exception of the southernmost, Control 1177, which extends westward from Santa Catalina (SXC).  By agreement between the military and the FAA, when the SUAs are simultaneously active,  the military is required to make available at least one of the control extensions north of Control 1177 so as to always have two routes available for ingress/egress.

  - All of the SUAs in this discussion are published as being active intermittently by NOTAM.

- Discussion

  - The FAA recently implemented a new feature in the Special Use Airspace Management System (SAMS) which automatically generates domestic and international Notice to Airmen (NOTAMs) whenever special use airspace (SUA) is scheduled.  The generic NOTAM does not indicate whether particular portions of the SUA might be available, such as control extensions.

  - When dispatch personnel are flight planning, one of the requirements is to take into consideration active SUA along the projected route and plan to avoid overflight.  In many cases, this requires an adjustment in the route, creating a need to plan for increased fuel burn, reduction in payload capacity and lost revenue, when, in fact, the airspace being avoided can be traversed.

  - The FAA is working with the military to subdivide warning areas W532 and W291 into different smaller volumes which would allow for more specific publication of airspace that is being utilized for military operations and potentially allow for ingress/egress corridors to be more readily availably to transoceanic air traffic.

- Recommendation

  - The meeting is invited to comment on the information in this paper.