THE THIRTY SECOND MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/32)

(Honolulu, Hawaii, USA, 10-14 May 2010)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

PAPER TRIAL ON TRACK 1 AS USER PREFERRED ROUTE

(Presented by Civil Aviation Bureau, Japan (JCAB) and the Federal Aviation Administration)

SUMMARY

This Information Paper provides a status of paper trial on TRACK 1 as User Preferred Route (UPR) planning between Anchorage ARTCC, Oakland ARTCC and Air Traffic Management Centre (ATMC).

- Introduction

  - FAA and JCAB have been discussing a UPR paper trial on PACOTS TRACK 1 after the IPACG/29 meeting.  Although some restrictions are required to UPR operations in the North Pacific Oceanic Route (NOPAC); Anchorage ARTCC, Oakland ARTCC and ATMC have agreed to start a UPR paper trial on TRACK 1.

  - JCAB and the FAA have implemented many UPR trials in the Pacific as shown in Table 1.

| Region | Phase | From | To |
| --- | --- | --- | --- |
| Japan - New Zealand, Caledonia | trial | Sep. 2007 | TBD |
| Asia - Hawaii | operational | Aug. 2008 | TBD |
| RJAA - YSSY, YBBN, YBCG, YBCS | trial | May 2009 | TBD |
| TRACK 14, 15 | trial | Sep. 2009 | TBD |
| TRACK H, I | trial | Apr. 2010 | October 2010 |

Table 1: Progress of UPR implementation

- Discussion

  - A Paper Trial of UPRs for aircraft that normally flight plan via PACOTS Track1 is set to begin on June 7, 2010.  The initial paper trial will run for a four week period with participating operators generating UPRs for their flights departing on Mondays to compare against the current PACOTS routes in use that day.  The comparison will look for ATC impacts, operational advantages, time and fuel savings.  A second paper trial will occur begin on November 1, 2010 for a four week period to look at the operational impacts during different seasonal conditions.  The details for the second paper trial will be disseminated at a later date.

  - The details for participating in the Track 1 Paper Trial are detailed in Attachment 1.

  - The format for Track 1 Paper Trial data collection is listed in Attachment 2.  A digital excel file is distributed at the meeting to facilitate the data collection.

- 2.4	The evaluations of the Operational Trial of PACOTS Track 14 and 15 UPRs is ongoing. When the evaluation of the Track 14 and 15 UPRs is completed, JCAB and the FAA will begin evaluating the possibility of PACOTS Track 3 UPRs.

- Recommendations

  - The meeting is invited to participate in the PACOTS Track 1 UPR paper trial, and:

  - Provide data and feedback on the Track 1 UPR Paper Trial

- Attachment 1

A Paper Trial of UPRs for aircraft that normally flight plan via PACOTS Track1 is set to begin on June 7, 2010.  The initial paper trial will run for a four week period with participating operators generating UPRs for their flights departing on Mondays to compare against the current PACOTS routes in use that day.  The comparison will look for ATC impacts, operational advantages, time and fuel savings.

Trial Dates:

Comparison Paper UPRs will be generated for flights departing on the following UTC dates:

June 7, 2010

June 14, 2010

June 21, 2010

June 28, 2010

General UPR Generation Paper Trial Guidelines:

- The Paper Trial UPRs will remain in the Fukuoka, Anchorage and Oakland OCAs.

- The UPR shall utilize a published standard departure routing where applicable.

- The UPR shall be planned to avoid military special use airspace when active.

- In the Fukuoka FIR, operators shall file one of the following Oceanic Transition Routings:

  - PEXEL A590 POWAL

  - DETOX OTR5 ADNIP R591 ADGOR

  - DETOX OTR5 CALMA

  - ONION OTR7 CELIN G344 CALMA

  - ONION OTR7 ETRON

  - ABETS OTR9 CELIN G344 CALMA

  - ABETS OTR9 ETRON

  - ABETS OTR11 DRIVR OTR22 ETRON

  - ABETS OTR11 RIPKI (GARRY)

  - VACKY OTR13 SEALS 150E

  - SMOLT OTR15 MORAY 150E

- When transiting through Anchorage FIR flights shall use one of the following transition routes:

- A590 transition route:

  - PASRO A590 POWAL

- R591 transition route:

  - AKISU R591 ASPIN

- G344 transition route:

  - CUTEE G344 CARTO  or

  - CUTEE 48N170E 49N180E (or point north of 49N180E   or

  - CUTEE 49N170E  or

  - CUTEE 50N170E

- When transiting through Vancouver UTA/FIR flights shall use one of the following transition routes:

  - To CYVR:

  - KATCH:	YZP POWOL KEINN CYVR

  - MANJO:	NAKBI EPTIV POWOL CYVR

  - NUDGE:	PETPA ITGAV POWOL CYVR

  - ORNAI:	SIMLU KEPKO YAZ FOCHE CYVR

  - PRETY:	TAMRU YAZ FOCHE CYVR

  - ROUKE:	VIDKU YAZ FOCHE CYVR

- To KSEA/KPDX:

  - KATCH:			UDMAP TOU JAWBN KSEA

UDMAP TOU KEIKO KPDX

  - MANJO NAKBI EPTIV:	TOU JAWBN KSEA

TOU KEIKO KPDX

  - NUDGE PETPA ITGAV:	TOU JAWBN KSEA

TOU KEIKO KPDX

  - ORNAI SIMLU KEPKO:	TOU JAWBN KSEA

TOU KEIKO KPDX

  - PRETY TAMRU:	 	TOU JAWBN KSEA

TOU KEIKO KPDX

  - ROUKE VIDKU:		DOLFF TOU JAWBN KSEA

DOLFF TOU KEIKO KPDX

  - WIDEN HONSO HQM: 	OLM KSEA

KEIKO KPDX

- Transiting aircraft:

- KATCH YZP or YZT:	UPR to destination

- MANJO NAKBI EPTIV:	UPR to destination

- NUDGE PETPA ITGAV:	UPR to destination

- ORNAI SIMLU KEPKO:	UPR to destination

- PRETY TAMRU SEFIX:	UPR to destination

- ROUKE VIDKU DOLFF TOU:	UPR to destination

- WIDEN HONSO HQM:	UPR to destination

- The UPRs shall remain at least 50nm laterally north of  PACOTS Track 2 or:

The Track 1 UPR may follow PACOTS Track 2 in the CENPAC and then diverge from PACOTS Track 2 to follow a UPR at least 50 nm north of Track 2.

- Track 1 UPRs in the CENPAC shall remain at least 25nm south of Anchorage FIR boundary.

- In an operational trial, UPR aircraft would not have priority for altitude assignment over aircraft on an existing PACOTS Track 1.

Data Collection:

- As a minimum we are looking to collect the following information.  Operators may use the distributed Excel spreadsheet for the data collection.

- Aircraft Call Sign

- Type Aircraft

- Current Structured Routing in use today for the flight including the departure and destination.

- En route time for Current Fixed Routing

- Fuel Burn for the Current Fixed Routing note kilos or pounds.

- Corresponding UPR Route of flight

- En route time for UPR Routing

- Fuel Burn for the UPR Routing in either kilos or pounds.

- Note any other operational advantages if any; e.g “The UPR route allowed us to avoid an area of forecast severe turbulence.”

- After all the data has been compiled for the Trial day please forward it to the following email addresses:

  - imuta-t07j7@cab.mlit.go.jp

  - tom.p.anderson@faa.gov

  - dennis.addison@faa.gov

Attachment 2

| Date | Callsign | Type A/C | Current PACOTS Routing (CPR) | CPR Time | CPR Fuel | UPR Routing | UPR Time | UPR Fuel | Additional Comments | Fuel savings | Time savings |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6/7/10 Sample | UAL 882 | B744 | RJAA GIRAF Y808 ONION/M084F330 OTR7 CELIN G344 COLIC/M084F350 G344 CARTO/N0484F350 G344 CUDDA DCT 60N150W DCT COHIL/N0479F370 NCA11 LEPET/N0481F390 NCA11 YWV J475 VLN DCT FAR J36 GEP/N0435F310 DCT DLL/N0440F290 DCT MSN DCT JVL KORD | 10:49 | 334223 LBS | RJAA GIRAF Y808 ONION/M084F330 OTR7 CELIN G344 COLIC/M084F350 G344 CARTO/N0484F350 G344 CURVS 54N170W 53N160W 52N150W 50N140W ORNAI SIMLU KEPKO DCT DLL DCT MSN DCT JVL KORD | 10:40 | 332223 LBS |  | 2000 | 9 |