THE THIRTY-SECOND MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/32)

(Honolulu, Hawaii, USA, 10-14 May 2010)

Agenda Item x:  	Air Traffic Management (ATM) Issues

Application of 10 min separation minimum without MNT within Fukuoka FIR

(Joint Paper Presented by Civil Aviation Bureau, Japan (JCAB) and Federal Aviation Administration)

SUMMARY

This information paper provides application of 10 minutes longitudinal separation minimum without Mach Number Technique (MNT) within Fukuoka FIR.

- Introduction

  - JCAB and FAA have been working together to improve seamless operations in the Pacific airspace in the past IPACG meetings.  10 minutes longitudinal separation minimum without MNT is one of the methods under the PANS-ATM.  PANS-ATM defined that “if navigation aids permit frequent determination of position and speed” able to provide 10 minutes separation in oceanic airspace.

  - JCAB have reviewed and defined the Air Traffic Services Procedure in the Fukuoka FIR and determined that if ATC obtained position reports at an interval of at least every 40 minutes on the route of flight, ATMC could provide 10 minutes longitudinal separation between the aircrafts without MNT.

  - To apply 10 minutes separation without MNT, required JCAB to implement an improved Flight Plan Conflict Probe (FPCP*) system as was discussed at the last IPACG meeting.  JCAB may begin applying the reduced separation minimum in July 2010 when the FPCP system improvements are implemented.

*Note that FPCP means confliction alert application of oceanic aircrafts controlling devices.

- Discussion

  - In July 2010, JCAB will begin to apply 10 minutes separation minimum without MNT within oceanic airspace within Fukuoka FIR.

  - After a period initial of application of the new standard in the Fukuoka FIR, seamless operation would commence on airway B452 and the ATS routes west of B452 that cross the Fukuoka and Oakland FIR Boundary. Seamless operation of 10 minutes separation without MNT would be expected to begin no later than October, 2010.

  - To expand the application of seamless operations in other part of oceanic airspace, it would be necessary to make modifications to the route structures so that position reports would be received at least every 40 minutes in the Fukuoka FIR (this would mean reports every 5 degrees of longitude) from the concerned aircraft.

- Recommendation

  - The meeting is invited to discuss the information in this paper.