THE TWENTIETH MEETING OF THE IPACG FANS INTEROPERABILITY TEAM

(IPACG FIT/20)

(Naha, Okinawa, Japan, 16 November 2010)

Report of the FIT

1.	Opening of the Meeting

1.1	The Twentieth Meeting of the Informal Pacific Air Traffic Control Coordinating Group (IPACG) FANS Interoperability Team (FIT) was held in Naha, Okinawa, Japan, 16 November 2010, in conjunction with the thirty-third meeting of IPACG, being hosted by the JCAB.

1.2	Mr. Shoichi Kosugi, JCAB Co-chair, and Mr. Reed Sladen, FAA Co-chair, welcomed the group and the participants introduced themselves.

1.3	The meeting considered six working papers (WP) and two information papers (IP’s).  All the papers and these minutes are available in full on the IPACG website.

2. 	Agenda Item 1: Review and approve agenda

2.1	WP-01, the proposed agenda, was accepted.

3.	Agenda Item 2: Working Papers

3.1	WP-02 Reports on the CRA and CRASA activities

3.1.1	Mr. Hayasaka outlined the JCAB CRASA report.  Mr. Miura and Mr. Yamazaki gave the JCAB presentation, in two parts.

3.1.1.1	Systems Performances

3.1.1.1.1	 Both the CPDLC downlink and uplink performance met all standards of the FOM.

3.1.1.1.2	 Both the uplink success rate and the auto-transfer success rates also met the FOM standards.

3.1.1.2	There were four updated PR’s and twenty-five newly received PR’s.  Of those twenty five PR’s, 22 are in various stages of investigation, 1 is closed as lessons learned, and 2 were scheduled for FIT closure

3.1.1.2.1 The following two PR’s were closed by FIT.  PR 10529 was an inability to logon which was caused by the ground system’s erroneous flagging of the flight id as an error.  The ground system’s software was fixed in July 2010.  PR 10530 was a case where an ADS ACK was delayed and actually followed an ADS NAK, which prevented the ground system from uplinking an ADS waypoint request.

3.1.1.4	Mr. Tom Kraft asked if there were any plans to calculate performance according to GOLD standards instead of using the FOM.  Mr. Shoichi Kosugi responded that the methods used would require revisions in order to use the GOLD, and that is being considered.  It is possible that it may be done in next fiscal year, at the earliest.

3.1.2	Mr. Gordon Sandell presented the FAA CRA data.

3.1.2.1	There were four new problem reports and all are proposed for FIT closure.  PR 869 was the same issue as mentioned above in PR 10530.  PR 870 was the same issue as mentioned above in PR 10529.  PR 871 was a failure to connect which was caused by a timing problem in the flight deck, where a logon following a logoff was too quick (11 seconds).  PR 872 was a B777 datalink software lockup issue which is expected to be fixed in the next B777 software release.  All four PR’s were closed by FIT.

3.1.2.2	There were 56 issue reported with FOI operations, and the CRA is working with the equipment providers (ground and flight deck) to further narrow causes and potential remediations.

3.1.2.3	Mr. Sandell reported on miscellaneous reports of old CPDLC clearances displaying on the FMS.  Each manifestation seems to be unique, and while several have been fixed with FMS changes, and one for the B777 (PR 862) is in the next FMS update (BP 16 release), others are still under investigation.

3.1.2.4	Mr. Nagai of ANA asked if PR 872 was just VHF, and asked about specific SATCOM failures the CRA.  Mr. Sandell responded that it is not VHF that is the problem, but a broader fix to the lockup problem is in BP-16.

3.1.2.5	Mr. Shoichi Kosugi asked about the wait time between the logoff and the logon; would a minute be recommended?  Mr. Sandell responded that a minute would probably be fine, but he had no specific recommendation, except to say that 11 seconds was clearly too quick.

3.1.2.5	Mr. Midorikawa asked about old clearances being found in FMS’s of later flights.  Are there any common signs, clues, which would alert the pilot to the fact that the clearance was old?  Mr. Sandell responded that what the pilot saw did not match what the FMS loaded, so there would have been no way to realize the mistake unless the pilot compared the two FMS pages.

3.2	WP-04 GOLD Implementation and Amendments

3.2.1	Mr. Tom Kraft presented an update on the world-wide implementation of, and amendment process for, the Global Operational Data Link Document (GOLD).

3.2.1.2	As of May 2010 all subordinate ATS coordinating groups (formal and informal) in NAT and the Asia-Pacific Regions, and the South Atlantic FIT, have adopted the GOLD.  The GOLD supersedes the FANS 1/A Operations Manual (FOM) and the Guidance Material for ATS Data Link Services in North Atlantic Airspace (NAT Data Link GM).

3.2.1.3	Some examples of implementation considerations that ATSU’s have had to deal with were presented.  These are illustrative of regional differences in datalink use.  Further expansion of the GOLD’s adoption is being pursued in the African-Indian Ocean (AFI) and South American (SAM) Regions (SAT/FIT)and the European (EUR) Region (Maastricht UAC).

3.2.1.4	Examples of proposed amendments to the GOLD, and the amendment process, were also discussed.  A key element is the development of sound configuration management processes that will sustain the “regional document” essence of the GOLD, and ensure the adoption of amendments only when all regions have given their concurrence.

3.2.1.5	The FIT agreed to support a follow-on work program for the GOLD Ad Hoc Working Group, as summarized in section 3 of WP-04.

3.3	WP-05 PARC FANS 1A over Iridium (FOI) and Performance-Based Concept Recommendations

3.3.1	Mr. Kraft presented this paper which discusses:

- FANS 1/A over Iridium (FOI) benefits

- FOI recommendation and rationale

- Data link incentives, mandates, the future

- Performance-based concept (PBC) recommendation and rationale

(Please see the complete paper at the IPACG website for details.)

3.3.2	On behalf of the Performance-Based Operations Aviation Rulemaking Committee (PARC) Mr. Kraft presented two recommendations.

3.3.2.1	Recommendation 1 is for the FAA to expedite the steps necessary to remove restrictions on current FOI operations to realize immediate gains in efficiency and safety benefits.

3.3.2.2	Recommendation 2 is to implement a performance-based framework for communications and surveillance.

3.3.3	Mr. Gene Cameron of United Airlines expressed strong support for expanded use of iridium, partially as a necessity in the trans-polar region.  He is particularly interested in getting data on FOI during adverse weather conditions.  Additionally, Inmarsat does not currently support RNP4 operations in much of the polar region and it is hoped that FOI might.

3.3.4	Mr. Nagai asked if PARC was recommending FOI use in non-polar regions.  Mr. Kraft responded that the data does not indicate regional FOI variations so yes, PARC is recommending more global use of FOI.  The second question was whether or not FOI would require any additional training for PBC use.  Mr. Kraft responded that yes, there is a training aspect to PBC, for both controller and flight crew.

3.3.5	Mr. Kraft indicated that the indicators (designators) are in fact reserved in the Amendment 1, with no assignment to GOLD specifications.  Mr. Midorikawa confirmed that detailed function of the “P1”-“P9” designator in a flight plan would not be implemented by 15 November 2012 with the new ICAO FPL formats.  Mr. Kraft responded that was probably true.

3.3.6	Mr. Kosugi asked about the FOI-related PR’s from the CRA reports and whether or not the PARC recommendations were to be made coincident with fixing the issues in those PR’s.  Mr. Kraft replied that PARC was recommending expedited review and development of expanded FOI use, believing that such review and development would take FOI issues into consideration.  He also stated that in PARC recommendation 2, “implement a performance-based framework for communications and surveillance” they were not focused solely on FOI, since other comm. media also experiences adverse performance issues.

3.4	WP-06 Status of the FANS 1-A over HFDL (FOH) evaluations

3.4.1	Mr. Kraft presented an overview of FOH history, and the PARC data and recommendations for FOH use in the North and Central Pacific.

3.4.1.1	Preliminary results of data collected indicates that FANS 1/A over HFDL is viable for some oceanic data link operations.  However, FOH may not be suitable for some reduced separations, for example, such as 30 NM lateral or longitudinal separation, and 50 NM longitudinal separation.

3.4.1.2	PARC recommends that FAA expedite the steps necessary to remove restrictions on current FOH operations to realize gains in efficiency and safety benefits as soon as possible, acknowledging that HFDL is viable for RCP 400 and ADSC/400 operations.

3.4.2	Mr. Kraft asked JCAB how they and their system handle FOH reports.  Mr. Midorikawa responded that HFDL reports are rejected by the system.

3.4.3	Mr. Kosugi asked the FIT to respond to the request to consider FOH in future planning for North and Central Pacific operations.  Some discussion followed, and Mr. Kraft asked the FIT (FAA and JCAB organizations) to do some evaluation of the HFDL viability for the future.  Mr. Sandell suggested that as a tertiary medium HFDL should be considered for use rather than simply discarded.  Mr. Midorikawa suggested that the ATS System Planning Division would have to become involved, but he did suggest that JCAB should consider HFDL use in the future.  All agreed to discuss HFDL at FIT-21.

3.5	WP-03 Global Satellite Communications (SATCOM) Voice Guidance Material

3.5.1	Mr. Kraft summarized PARC and other group’s effort to begin the process of developing globally applicable standards for use of SATCOM Voice for ATC services.  The effort is toward developing a guideline document similar in form to the GOLD.

4.	Agenda Item 3: Information Papers

4.1	IP-01 Status of Harmonization Process Between APAC and NAT on the AIDC ICD

4.1.1	Mr. Kraft summarized the effort to harmonize the interface control documents (ICD’s) for AIDC use in the Asia-Pacific and North Atlantic regions.  This should be accomplished as quickly as practicable, and the NAT and APAC ICD’s should be frozen in the interim.  Once the NAT and APAC PIRGs have endorsed the resulting ICD, a new round of drafting and review could begin to incorporate any desired new substance, as part of the ongoing inter-regional maintenance of the document.

4.1.2	Mr. Takata thanked the FAA for leadership in this effort and made clear that JCAB strongly supports the effort.  He also asked that Karen Chiodini be thanked for her excellent work.

4.2	IP-04 US FAA Post-Implementation Data Link Monitoring and Analysis Results

4.2.1	Ms. Julia Fuller presented the detailed analysis report of the following:

- GOLD Analysis Overview (starts slide 4)

- FANS 1/A Performance by Media Type (starts slide 9)

- FANS 1/A Performance by Month

  - Over SATCOM (starts slide 19)

  - Over HFDL (starts slide 31)

- FANS 1/A over SATCOM Performance

  - By Operator (starts slide 38)

  - For Operator “A” (starts slide 49)

  - For Operator “A” by Airframe (starts slide 57)

  - By GES (starts slide 62)

(Please see the complete report on the IPACG website for details.)

4.2.2	Mr. Midorikawa reconfirmed that HF does not met the 95% criteria in several instances.

4.2.3	Mr. Nagai asked if any conclusions had been drawn as to the reasons for the differences between the FIR’s.  Ms. Fuller said that to date that had not been analyzed.  Mr. Dennis Addison pointed out that the variables are many in any such analysis, making appropriate comparisons very difficult on the global level.  Mr. Kraft commented that monitoring data does indicate that performance in the New York FIR tends to be worse than in the Oakland FIR, and that such things as VHF frequency coverage and the type aircraft mixture may well contribute to the discrepancy.  So although we do not have answers necessarily, the data does illustrate the value of continued monitoring and analysis.

5.	Agenda Item 4: Other Business

5.1	No other business was brought before the FIT.

6.	Closing of the Meeting

6.1	The Co-chairs thanked the FIT meeting members for their participation, and on behalf of all participants, the Co-chairs expressed their great appreciation for the efforts of the interpreters.

END