THE THIRTY-FOURTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/34)

(Honolulu, Hawaii, USA, 23-27 May 2011)

Agenda Item 5:  Air Traffic Management (ATM) Issues

Simulation Result of UPR for NOPAC Route

(Presented by ENRI)

SUMMARY

UPR implementation on NOPAC Route has been desired. Anchorage ARTCC proposed three models of usage for NOPAC. This paper provides the effect for each models due to PACOTS Track location.

- Introduction

  - IATA proposed the Pacific Project. The project potentially might deliver the greatest environmental benefits by more flexible ATC operation on NOPAC area. However, It may cause additional potential conflict. Previous consideration for daily traffic totally is necessary.

  - ENRI has reported some problems by implementation of UPR without restriction about eastbound UPR. Remaining one route is important to keep economical climb.

  - At IPACG/33, Anchorage ARTCC proposed three models of ATC operation on NOPAC route area. These models are as shown in Table 1.

Table 1  Model of ATC Operation on NOPAC route

| Model | Oceanic Operation |
| --- | --- |
| Model 1: | NOPAC routes are bi-directional |
| Model 2 | NOPAC routes are uni-directional as same as current ATC operation. South from A590 is UPR area using current waypoint. |
| Model 3 | NOPAC area is free UPR area |

  - ENRI evaluated the effect for each models due to PACOTS Track location when UPR is available.

- Discussion

  - UPR location is near PACOTS location due to city-pair. The day is categorized into three types by PACOTS location at Fukuoka-Anchorage, Oakland boundary as shown as Table 2. The ratio of each type in which 2009 Nov. to 2010 Oct. is also as shown in the table. In winter, the type is almost “TYPE W”. In summer, the ratio of “TYPE M” is large.

Table 2 Type of PACOTS TRACK location

| TYPE | Location | ratio |
| --- | --- | --- |
| W | TRK C, E, F are north from TRK1. | 56% |
| M | Eastbound TRK and Westbound TRK are not separated for north and south | 39% |
| E | TRK1,2,3 are north from TRK C | 5% |

  - Model 1 changes flight altitude at R220, R580, A590. Increase of capacity is anticipated when R220 and R580 are available to westbound and A590 is available to eastbound. However, Model 1 causes that a eastbound UPR and westbound UPR might overlap the same NOPAC route. Some flight could not climb below the backward flight. Additionally, flight altitude for same direction decreases. flight altitude decreases on each route. When a lot of westbound concentrate on R220, flight altitude is not enough, even if eastbound traffic is few on R220.

  - Model 2 is almost same as current network. So eastbound and westbound are separated on R220, R580 and A590. Intersection by opposite directional traffic might be few.

  - Model 3 causes a lot of intersection. Especially, eastbound and westbound are concentrated in narrow area at the “TYPE M”. Optimum flight altitude is not guaranteed in peak traffic time.

  - At the “TYPE W”, Model 2 separates almost eastbound UPRs and westbound UPRs. However, the flight route from/to Anchorage should be fixed something NOPAC route.

  - At the “TYPE M”, Model 1, 2 and 3 might causes a lot of intersection. When TRK E and F are south from TRK 3

  - At the “TYPE E”, Model 1 sometimes concentrates eastbound UPR to NOPAC and westbound UPR locates south. However, the flight route from/to Anchorage locates in NOPAC route. Model 2 often separates eastbound UPRs and westbound UPRs.

  - Tendency of Anchorage to/from UPR is different with other UPR. Because effect of jet stream is lower than others due to flight path length.

- Conclusion

  - Tendency of UPR intersection by eastbound and westbound  is summarized as Table 3.

Table 3 UPR intersection by eastbound and westbound

| TYPE | Model 1 | Model 2 | Model 3 |
| --- | --- | --- | --- |
| W | Present | Few | Many |
| M | Many | Many* | Many |
| E | Present | Present | Many |

*Model 2 in TYPE M need additional categorization.

  - Model 1 is difficult to introduce in this evaluation. Concentration of bi-direction traffic might decreases ATC operational efficiency. However, it is potentially introduction by additional restriction for the day “TYPE W” or “TYPE E”. When original directional traffic is few on one NOPAC route, to apply the route for opposite directional flight does not decrease efficiency. Detail analysis and simulation should be necessary.

  - Model 2 is available for the day “TYPE W”. The ratio is 56% for annually. Exclusion of excessive concentration to R220 is necessary. When the flight from Russian route is many, introduction of the model would be possibility. For the day “TYPE M “ and “TYPE E”, when tracks separate for north and south, the model might be introduce. Additional categorization is necessary depend on “TRK1”, “TRK2,3” , “TRK C” and “TRK E,F” group.

  - Model 3 causes a lot of intersection and merging. It has a lot of problem in ATC operation. The flexibility might be re-consider after clear the other models.

- Action by the meeting

  - The meeting is invited to note the information provided .