THE THIRTY-FOURTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/34)

(Honolulu, Hawaii, USA, 23-27 May 2011)

Agenda Item 5:  	Air Traffic Management (ATM) Issue

Update on the status of trial operations on acceptance and provision of

turbulence information by CPDLC

(Presented by JCAB)

SUMMARY

This paper updates the status of trial operations on acceptance and provision of turbulence information to capable aircraft by CPDLC within Fukuoka FIR.

- Introduction

  - ATMC has provided turbulence information to capable aircrafts by CPDLC as trial operation since February 10th 2010.  During this trial operation, JCAB has provided the turbulence information in both of CPDLC and HF voice communication, and has evaluated the efficiency.

  - Current ODP system cannot accept turbulence information automatically from aircraft in particular format such as "20nm SW of CUTEE" into the system. So controllers should convert the location by azimuth and distance into the location by latitude and longitude. JCAB plans to improve ODP system after July of this year. It will be able to accept particular format automatically after system improvement.

- Discussion

  - JCAB intends to continue trial operation in same procedures until complete for ODP system improvement. That means JCAB will continue to provide turbulence information by using both methods CPDLC and HF voice communication.

  - Currently in the Fukuoka FIR, turbulence information is provided from aircraft which fly around location of observation within height difference of ±2,000ft from altitude of observation and will pass within two hours from time of observation. JCAB would like to hear the opinion from aircraft operator regarding adequacy of these provided range. Especially, if we receive high intensity turbulence report from the aircraft, JCAB is considering the need of expanding of range to provide.

  - At the IPACG/33, JCAB and FAA both agreed the necessity and effectiveness on the scheme for providing turbulence information not only Fukuoka FIR but also in the vicinity of FIR boundary. ATMC wishes to share the turbulence information with Anchorage ARTCC and Oakland ARTCC and disseminate it automatically to aircraft in the identified altitude stratum around FIR boundary after the ODP system improvement in July 2011.

  - JCAB had plan to complete trial operation within one year at first, but now intends to extend trial operation until resolve some issues which described in paragraph 2.2 and 2.3. and improvement of ODP system. JCAB also discuss with aircraft operators to enhance delivery procedures for turbulence information adequately from/to aircraft.

- Action by the meeting

  - The meeting is invited to agree to extend trial operation until establish adequate procedures and improve ODP system.