THE THIRTY-FOURTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/34)

(Honolulu, Hawaii, USA, 23-27 May 2011)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Status of the Study for the Implementation of ADS-C Climb/Descend Procedures

(Presented by the Civil Aviation Bureau of Japan)

SUMMARY

The purpose of this working paper is to present the Civil Aviation Bureau of Japan (JCAB) plan for the implementation of an ADS-C Climb/Descend Procedure (CDP) in the Fukuoka Flight Information Region (FIR).

- Introduction

  - The FAA is continuing the work associated with the development of a climb-through and descend-through procedure (ADS-C Climb/Descend Procedure) wherein aircraft pair distances verification is performed by ground flight tracking systems, such as the FAA Ocean21 system, using near simultaneous ADS-C demand contract reports.  ADS-C analysis has determined that this oceanic procedure supports the FAA goals for fuel efficiency, emission reductions and air traffic capacity.

  - The ADS-C procedure is designed to improve FAA service to properly equipped aircraft by allowing an oceanic air traffic controller to have another option for granting an altitude change request when other standard separations, such as ADS-C distance-based 30 NM longitudinal separation minima, do not allow for a climb or descent through the altitude of a blocking aircraft.  It is strictly an air traffic control tool that is controller-initiated, to be applied between RNP4 maneuvering and blocking aircraft pairs.

    - Based on Oakland ARTCC traffic, with a 35% Future Air navigation System (FANS) equipage rate, benefits analysis projected total combined fuel savings up to 44,000 kilogram (kg) or 139,040 kg of carbon dioxide (CO2) per day.

    - A FANS equipage rate nearing 100% increased the potential benefits to nearly 88,000 kg of fuel or 278,080 kg CO2 per day.

  - The FAA started operational trial of ADS-C CDP within the Oakland Oceanic FIR on 15 February 2011.  During the operational trial, Safety Management System monitoring will be performed and benefits metrics will be collected.

- Discussion

  - JCAB has implemented the ADS-C procedure within Fukuoka FIR in 2006.  ADS-C distance-based 50 NM longitudinal separation minima between RNP10 equipped aircraft pairs had been implemented in 2007. ADS-C distance-based 30 NM longitudinal separation minima between RNP4 equipped aircraft pairs had been implemented in 2008.

  - Based on the Fukuoka FIR traffic, 26.9% is equipped with RNP4 as of March 2011.

  - Percentage of RNP4 aircraft within the Fukuoka FIR is still low and JCAB considers that increment of the percentage of RNP4 equipped aircraft is necessary to maximize the benefit of ADS-C distance-based 30 NM longitudinal separation minima.

  - ADS-C CDP will create further advantage for the RNP4 equipped aircraft and this will promise increment of the percentage of RNP4 equipped aircraft.  JCAB will commence necessary study to implement ADS-C CDP within the Fukuoka FIR.  SASP (Separation and Airspace Safety Panel) is continuing the work associated with the amendment of ICAO PANS-ATM Doc 4444 to provide for the new separation standard of an ADS-C CDP.   JCAB is looking forward to the success of the FAA’s operational trial of ADS-C CDP within the Oakland Oceanic FIR, and JCAB plans to implement ADS-C CDP within the Fukuoka FIR as soon as possible after the PANS-ATM Doc 4444 is amended.

  - On the other hand, JCAB is interested in the possibility of the implementation of ADS-C CDP between RNP10 equipped aircraft pair (or between RNP4 equipped aircraft and RNP10 equipped aircraft pair).  ADS-C CDP between RNP10 equipped aircraft pair will enable to allow an oceanic air traffic controller to have another option for granting an altitude change request when other standard separations, such as ADS-C distance-based 50 NM longitudinal separation minima, do not allow for a climb or descent through the altitude of a blocking aircraft.  Based on the Fukuoka FIR traffic, more than 98% is equipped with RNP10 as of March 2011.  Considering the present high percentage of RNP10 equipped aircraft within the Pacific airspace, ADS-C CDP between RNP10 equipped aircraft pair will create benefits for many aircraft operator.

  - Safety assessment (the risk calculations) conducted by SASP shows that the flight count has an inverse relationship to risk, i.e., if the number of CDPs conducted in a given hour is held constant and the flight count increases, then the risk of collision decreases.  The number of CDPs conducted per hour has a direct relationship to risk.  If the flight count is held constant and number of CDPs per hour increase, then the risk increase.  The FAA started operational trial of ADS-C CDP within the Oakland Oceanic FIR on 15 February 2011.  JCAB understands that during the operational trial, the FAA will collect the number of CDPs conducted per hour and the flight count as Safety Management System monitoring.  JCAB recognises that same data collection is necessary to satisfy the TLS when JCAB commence the ADS-C CDP within the Fukuoka FIR.

- Action by the Meeting

  - The meeting is invited to:

- a) Note that JCAB plans to implement ADS-C CDP within the Fukuoka FIR as soon as possible after the PANS-ATM Doc 4444 is amended; and

- b) Discuss the possibility of the implementation of ADS-C CDP between RNP10 equipped aircraft pair (or between RNP4 equipped aircraft and RNP10 equipped aircraft pair); and

- c) Note that JCAB recognises the need of collecting the number of CDPs conducted per hour and the flight count to satisfy TLS when JCAB commences the ADS-C CDP within the Fukuoka FIR.

- ………………………

- -END