THE THIRTY-FOURTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/34)

(Honolulu, Hawaii, USA, 23-27 May, 2011)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Status Update on Implementation of 10 Minute Longitudinal Separation Without

Mach Number Technique within Anchorage Flight Information Region

(Presented by the Federal Aviation Administration)

SUMMARY

This information paper provides an update on implementation of 10 minute longitudinal separation minimum without the mandatory application of Mach Number Technique within the Anchorage Flight Information Region.

- Introduction

  - The operational trial for the use of 10 minute longitudinal separation without Mach Number Technique (10noMNT) within the Anchorage Flight Information Region (FIR) began on 12 April 2007.  The purpose of the operational trial is to obtain data for analysis and to acquaint controllers with the operational aspects of its application.  A similar operational trial in the adjacent Oakland FIR was initiated in September 1998.  Reference 1 contains the analysis conducted to support the operational trial in the Oakland FIR.

- Discussion

  - The gain-loss probability distribution for aircraft pairs throughout the Anchorage FIR was investigated in the data gathered from the trials and the analysis centered on the description of aircraft pair-wise separation and their combined ability to maintain separation once it had been applied.  The premise was based on newer aircraft control mechanisms and sensors being better able to hold speeds, and that the operating characteristics of the fleet of aircraft in general, result in more similar cruise speeds.

  - The data collection, which began in parallel with the operational trial in April 2007, included data on the positions (latitude, longitude (or fix), flight level and associated times of all aircraft in the sector of interest.  These data were obtained through the historical data archives of the Ocean21 system, which include the reported positions from aircraft.  The data were then compared to previous studies’ distribution to see if current practice will support 10-minue longitudinal.

  - Reference 2 provides a description on the data collection process.  Position data from all aircraft operating in the Anchorage Oceanic FIR are obtained from the Ocean21 data reduction archives.

  - Related data analyses for the 10 minute longitudinal separation standard without MNT was reviewed by the ICAO Review of the General Concept of Separation Panel (RGCSP) (subsequently renamed the Separation and Airspace Safety Panel (SASP)).  The report of the Seventeenth Meeting of the RGCSP Working Group A (RGCSP/WG/A/17) at Annapolis, Maryland, United States on 6 to 17 November 2000 states in paragraph 3.1.2:

    - The Meeting noted that the 10 minute longitudinal separation trials for the Pacific Ocean airspace were completed and that application of the separation without MNT could be applied when appropriate measures were established.  It was further noted that the PANS-RAC (Part III, 8.2.1 b) allowed for 10 minute separation to be applied if navigation aids permit frequent determination of position and speed, and under Section 9 (paragraph 9.2b) separation may be reduced.

  - Reference 3 describes a subsequent analysis in which there were 282 eastbound and 596 westbound longitudinally proximate pairs observed in the Ocean21 data for the period March through August 2007.

  - Reference 4 describes an analysis performed after the modification of the five airspace fixes within the Anchorage Oceanic FIR from non-compulsory to compulsory reporting points. The addition of the compulsory reporting points increased the average number of longitudinally proximate pairs found in the data from 146 to 323 pairs per month.

  - Reference 5 describes the preliminary safety assessment conducted for the use of 10 minute longitudinal separation without MNT in the Anchorage FIR.  The result of the study is that the estimate of longitudinal collision risk is 1.13 x 10-12 fatal accidents per flight hour, which meets the target level of safety of 5.0 x 10-9 fatal accidents per flight hour.  This value satisfies the Target Level of Safety (TLS) value applicable to judging the safety of the longitudinal separation standard.

  - The operational trials have been successfully underway since 2007 and the FAA considers that an adequate amount of data has been analyzed.  Anchorage Center continues to use the test procedures until such time as ICAO approval/rejection is concluded.

  - The Proposal for Amendment of the Regional Supplementary Procedures Pacific Region is available for review at Attachment A.

- Conclusion

  - The meeting is invited to note the information in this paper and at the attachment.

- References

- “Status of Operational Trials and Demonstrations for 10 Minutes Longitudinal Separation Without mandatory MNT in the Oakland FIR”, IP/3, Review of the General Concept of Separation Panel (RGCSP), Working Group A Meeting, St. Petersburg, 17-28 May 1999.

- “Status of the Operational Trials and Demonstrations for 10 Minute Longitudinal Separation Without Mandatory Mach Number Technique (MNT) in the Anchorage Flight Information Region (FIR)”, IP/9, IPACG/26, Anchorage, Alaska, 14-18 May 2007.

- “Update on the Status of the Operational Trials and Demonstrations for 10 Minute Longitudinal Separation Without Mandatory Mach Number Technique (MNT) in the Anchorage Flight Information Region (FIR)”, IP/4, IPACG/27, Tokyo, Japan, 5-9 November 2007.

- “Update on the Status of the Operational Trials and Demonstrations for 10 Minute Longitudinal Separation Without Mandatory Mach Number Technique (MNT) in the Anchorage Flight Information Region (FIR)”, IP/8, IPACG/28, Las Vegas, Nevada, 14-16 May 2008.

- “Preliminary Safety Assessment for 10 Minute Longitudinal Separation without Mandatory Mach Number Technique (MNT) in the Anchorage Flight Information Region (FIR)”, IP/01, IPACG/30, Seattle, Washington, USA, 18-22 May 2009.

ATTACHMENT A

PROPOSAL FOR AMENDMENT OF THE

REGIONAL SUPPLEMENTARY PROCEDURES

PACIFIC (PAC) REGION (Doc. 7030/5)

(Serial No.: XXX – PAC 6-2)

a)	Regional Supplementary Procedures:

PAC

b)	Proposed by:

United States

c)	Proposed amendment:

Editorial Note: Amendments are arranged to show deleted text using strikeout (text to be deleted), and added text with grey shading (text to be inserted).

Amend the following in the PAC SUPPS, Chapter 6.

Chapter 6.   AIR TRAFFIC SERVICES

6.2   SEPARATION

6.2.2   Longitudinal

(P-ATM, Chapters 5 and 13)

6.2.2.1	Except as provided for in 6.2.2.2 and 6.2.2.3, the minimum longitudinal separation between turbo-jet aircraft operating within Anchorage Oceanic, Auckland Oceanic, Nadi, Oakland Oceanic and Tahiti FIRs shall be in accordance with the PANS-ATM, 5.4.2.4 or 5.4.2.5.

6.2.2.2	The minimum longitudinal separation shall be 93 km (50 NM) derived by RNAV between aircraft meeting the provision in 4.1.1.1.

Note. – The provision of the PANS-ATM, 5.4.2.6.1 to 5.4.2.6.3, apply in all cases.  Where ADS is not available, the provisions of the PANS-ATM, 5.4.2.6.3 apply.  Where ADS is available, the provisions of the PANS-ATM, 5.4.2.6.4 apply.

6.2.2.3        The minimum longitudinal separation within the Anchorage and Oakland Oceanic FIR shall be 10 minutes, whether in level, climbing or descending flight, provided that the aircraft concerned follow the same track or continuously diverging tracks until some other form of separation is provided.

6.2.2.34      The minimum longitudinal separation shall be 55.5 km (30 NM) between aircraft meeting the provisions in 4.1.2.1.

Note. – ADS is required for the application of this minimum; therefore, the applicable provisions will be those of the PANS-ATM, 5.4.2.6.1 to 5.4.2.6.3, and 5.4.2.6.4.

d)	Date when proposal received:

XXX

e)	Proposers reason for amendment:

1)   The operational trial for the use of 10 minute longitudinal separation without Mach Number Technique (MNT) within the Anchorage FIR began in 2007.  The purpose of the operational trial was to obtain data for analysis and to acquaint controllers with the operational aspects of its application.  The FAA trials were successful and support this proposed amendment.

2)   Previous studies which support the 10 minute longitudinal separation include an operational trial and data collection conducted within the Oakland FIR.  Related data analyses for the 10 minute longitudinal separation standard without MNT was reviewed by the ICAO Review of the General Concept of Separation Panel (RGCSP) (subsequently renamed the Separation and Airspace Safety Panel (SASP)).  The report of the Seventeenth Meeting of the RGCSP Working Group A (RGCSP/WG/A/17) at Annapolis, Maryland, United States on 6 to 17 November 2000 states in paragraph 3.1.2:

The Meeting noted that the 10 minute longitudinal separation trials for the Pacific Ocean airspace were completed and that application of the separation without MNT could be applied when appropriate measures were established.  It was further noted that the PANS-RAC (Part III, 8.2.1 b) allowed for 10 minute separation to be applied if navigation aids permit frequent determination of position and speed, and under Section 9 (paragraph 9.2b) separation may be reduced.

- RGCSP/WG/A/17 further agreed that sufficient provisions were in the PANS-RAC (subsequently revised to the PANS-ATM) to enable authorities to reduce separation 10 minutes without applying MNT.  The meeting noted that authorities would need to determine the regulations and procedures necessary for suitably equipped and approved aircraft to be separated in this way.  Therefore, an amendment to the PANS-RAC was not necessary and the Regional Supplementary Procedures (Doc 7030) could be amended as appropriate.

- The Procedures for Air Navigation Services – Air Traffic Management (PANS-ATM, Doc 4444), Chapter 5, 5.4.2.2.1.1 permits aircraft maintaining the same level to be separated by 10 minutes, if navigation aids permit frequent determination of position and speed.  Further, 5.11.1 supports the reduction in separation provided a safety assessment has shown that an acceptable level of safety will be maintained and users are consulted.

5)   Special electronic, area navigation or other aids enable the aircraft to closely adhere to their current flight plans (referred to in PANS-ATM, 5.11.1.2 a)) can be found in advances in existing aircraft systems capabilities which support airspace development in the form of proven precision of equipment, improved design of procedures and more efficient operations. Modern aircraft systems provide reliable, repeatable, and predictable performance in their capability to precisely navigate, maintain altitude accurately, and to achieve actual time of arrivals that coincide with estimated time of arrivals. This performance is achievable as a result of significant technological advances, the large percentage of operators equipping with modern systems, and the use of global navigation technology that provides primary navigation functions as well as updating capabilities of other primary navigation systems.

6)   This amendment proposes to allow 10 minutes to be applied in Anchorage Oceanic FIR without the mandatory application of the Mach number technique (MNT), utilizing controller discretion in applying methods (including, but not limited to MNT) to ensure minimum separation.

7)   The current longitudinal separation in the Anchorage Oceanic FIR is 15 minutes.  The application of a longitudinal separation minimum of less than 15 minutes requires that a pair of aircraft be speed-controlled using MNT.  This restricts the potential gain or loss of separation between pairs of aircraft.

8)   This amendment does not intend to alter the application of MNT, but allow controllers to have more flexibility in applying 10-minute longitudinal separation thereby allowing flights to operate more efficiently without compromising safety.  This new procedure reduces communication transmissions and cost, maintains the current level of safety and provides an operational benefit to the pilots and controller.

9)   The studies show that assignment of Mach number to aircraft in Anchorage FIR to maintain minimum longitudinal separation is not a common practice in the Pacific Ocean.  With improved flight performance, and in view of aircraft operating more closely in accord with their flight planned speeds, it would be possible to rely on an aircraft’s flight planned speed and pilot estimates in many situations, rather than the mandatory application of MNT to assure longitudinal aircraft spacing at 10 minutes.

10)  The gain-loss probability distribution for aircraft pairs throughout the Anchorage FIR was investigated in the data gathered from the trials and the analysis centered on the description of aircraft pair-wise separation and their combined ability to maintain separation once it had been applied.  The premise was based on newer aircraft control mechanisms and sensors being better able to hold speeds, and that the operating characteristics of the fleet of aircraft in general, result in more similar cruise speeds.

11)  The data collection, which began in parallel with the operational trial in April 2007, included data on the positions (latitude, longitude (or fix), flight level and associated times of all aircraft in the sector of interest.  These data were obtained through the historical data archives of the Ocean21 system which include the reported positions from aircraft.  The data were then compared to previous studies’ distribution to see if current practice will support 10-minue longitudinal.

12)  The operational trials have been successfully underway since 2007 and the FAA considers that an adequate amount of data has been analyzed.  Anchorage Center continues to use the test procedures until such time as ICAO approval/rejection is concluded.

13)  A comparative safety analysis of the airspace where 10 minutes separation is applied was performed by the FAA and it was found that, due to improvements in aircraft performance and air traffic control practices, the risk of collision due to the loss of planned longitudinal separation was no greater than the risk found in the original study which supported 10 minutes with Mach number technique.  The users have been consulted and kept up to date on developments through the Informal Pacific ATS Coordinating Group (IPACG), and its reports are reviewed by the RASMAG and presented to APANPIRG.

f)	Proposed implementation date of the amendment:

Upon approval by the Council.

g)	Action by the Secretary General:

The proposal has been circulated to the following States and international organizations.

XXX

h)	Secretariat’s comments:

TBD after the PfA is submitted to the ICAO PAC Office