THE THIRTY-THIRD MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/34)

(Honolulu, Hawaii, USA, 23-27 May 2011)

Agenda Item 5:  	Air Traffic Management

Outcomes of RASMAG/14

(Presented by ICAO Asia and Pacific Office)

SUMMARY

This paper presents outcomes of the 14th Meeting of the Regional Airspace Safety Monitoring Advisory Group (RASAMG/14, February 2011) for the meeting’s review.

- Introduction

  - This paper presents the discussion at the RASMAG/14 which was held at the Regional Office, Bangkok, Thailand in February 2011.

- Discussion

Reports from Asia/Pacific RMAs and EMAs

2.1	Table 1 below presents the estimates of vertical collision risk for Japanese airspace.  The technical risk was estimated to be 0.31 x 10-9 fatal accidents per flight hour.  The estimate of the overall vertical collision risk was 9.42 x 10-9 fatal accidents per flight hour, which does not satisfy the globally agreed TLS value of 5.0 x 10-9 fatal accidents per flight hour.

| Japanese RVSM Airspace – estimated annual flying hours = 957,711.25 hours (note: estimated hours based on December 2009 traffic sample data) |  |  |  |
| --- | --- | --- | --- |
| Source of Risk | Risk Estimation | TLS | Remarks |
| Technical Risk | 0.31 x 10-9 | 2.5 x 10-9 | Below Technical TLS |
| Operational Risk | 9.11 x 10-9 | - | - |
| Total Risk | 9.42 x 10-9 | 5.0 x 10-9 | Above Overall TLS |

Table 1: Risk Estimates for the RVSM Implementation in the Japanese airspace

  - Table 2 below presents the results of the airspace safety oversight, as of November 2010, in terms of the technical, operational, and total risks for the RVSM operation in the Western Pacific/South China Sea (WPAC/SCS) airspace which is contiguous to the IPACG airspace, i.e. the Oakland and Fukuoka flight information regions (FIRs).

| South China Sea RVSM Airspace – estimated annual flying hours = 905,147 hours (note: estimated hours based on December 2009 traffic sample data) |  |  |  |
| --- | --- | --- | --- |
| Source of Risk | Lower Bound Risk Estimation | TLS | Remarks |
| Technical Risk | 0.64 x 10-9 | 2.5 x 10-9 | Below Technical TLS |
| Operational Risk | 5.07 x 10-9 | - | - |
| Total Risk | 5.71 x 10-9 | 5.0 x 10-9 | Above Overall TLS |

Table 2: Risk Estimates for the RVSM Implementation in WPAC/SCS Airspace

  - Table 3 below presents the estimates of vertical collision risk for Pacific airspace.  The technical risk was estimated to be 0.064 x 10-9 fatal accidents per flight hour.  The operational risk estimate was 4.374 x 10-9 fatal accidents per flight hour.  The estimate of the overall vertical collision risk was 4.438 x 10-9 fatal accidents per flight hour.  This estimate meets the regionally agreed Target Level of Safety (TLS) value of 5.0 x 10-9 fatal accidents per flight hour.  This new estimate is based on the most recent 12 months of Large Height Deviation (LHD) reporting and recently updated collision risk parameters based on the December 2009 TSD collected.

| Source of Risk | Lower Bound Risk Estimation | TLS | Remarks |
| --- | --- | --- | --- |
| Technical Risk | 0.064 x 10-9 | 2.5 x 10-9 | Below Technical TLS |
| Operational Risk | 4.374 x 10-9 | - |  |
| Total Risk | 4.438 x 10-9 | 5.0 x 10-9 | Below Overall TLS |

Table 3:  Vertical Collision Risk Estimates for Pacific Airspace

  - Table 4 below presents the estimates of vertical collision risk for a portion of Northeast Asia airspace, which is contiguous to Fukuoka FIR.  The technical risk was estimated to be 1.786 x 10-9 fatal accidents per flight hour.  The LHD reports received during the most recent 12 calendar months are included in the estimate of vertical operational risk.  Because there were no risk-bearing events reported for Incheon FIR during the time period used for this safety monitoring report, the current operational risk value for November 2010 was 0.00 x 10-9 fatal accidents per flight hour.  The estimate of the overall vertical collision risk was 1.786 x 10-9 fatal accidents per flight hour.  The estimate meets the regionally agreed TLS value of 5.0 x 10-9 fatal accidents per flight hour.  This new estimate was based on the most recent 12 months of LHD reporting and the traffic sample received in December 2009.  The estimated annual flying hours for this portion of Northeast Asia airspace is 133,399 flying hours per year.

| Source of Risk | Lower Bound Risk Estimation | TLS | Remarks |
| --- | --- | --- | --- |
| Technical Risk | 1.786 x 10-9 | 2.5 x 10-9 | Below Technical TLS |
| Operational Risk | 0.00 x 10-9 | - |  |
| Total Risk | 1.786 x 10-9 | 5.0 x 10-9 | Below Overall TLS |

Table 4:  Vertical Collision Risk Estimates for North East Asia Airspace

Airframes Continuously Conducted RVSM Flights with No Registration in RMA’s Approval Data

  - JCAB RMA compared more than 70,000 flight plans of RVSM flights with the global RMA’s latest approval databases.  Some operator-aircraft combinations were continuously detected as non-registered airframes.  JCAB RMA has identified 77 airframes which were flying in the RVSM airspace of Fukuoka FIR with “W” on their flight plans but without the registration in RMAs’ RVSM approval databases between December 2009 and November 2010.  Attachment A to this paper details a list of airframes which conducted RVSM flights with no registration in RMAs’ approval databases.

Preliminary Assessment Work to Support Use of the 30NM Lateral and Longitudinal Separation

  - The United States provided the preliminary assessment work to support 30/30 in NOPAC.  As part of the preliminary work, Table 5 below contains the proportions of aircraft operations and filed RNP status within the Anchorage Oceanic FIR.  These data were obtained through the aircraft filed flight plans.  In some cases, there was no RNP status filed or the filed flight plan was not located in the archived data.  As shown in the table, most of the operations conducted within the Anchorage Oceanic FIR were RNP10 certified.  Roughly, 24 percent of operations conducted within the Anchorage oceanic FIR indicate RNP4 certified.

| RNP Status | Count | Proportion |
| --- | --- | --- |
| RNP 10 | 37543 | 71.95% |
| RNP 4 | 12532 | 24.02% |
| RNP 5 | 1175 | 2.25% |
| RNP 1 | 300 | 0.57% |
| Unknown or no RNP filed | 629 | 1.21% |
| TOTAL | 52179 |  |

Table 5:  Proportion of Operations by Filed RNP Status within the Anchorage Oceanic FIR - 2010

Airspace Safety Monitoring Documentation and Regional Guidance Material

RVSM Long Term Height Monitoring Burden

  - The meeting discussed the extent to which application of the Long-Term Height Monitoring (LTHM) requirement can best be standardised in an operational sense.  The Pacific Aircraft Registry and Monitoring Organisation (PARMO) commented that their intention was to use the most recent date of monitoring projected two years forward, noting it would be more focussed on US registered aircraft, but would possibly follow up with States if monitoring is falling behind. After some further discussion it was agreed that the best means for scheduling monitoring is to set November 2010 as the baseline, and anything monitored prior to 2008 will need to be monitored now and then stagger the remainder according to their last successful monitoring after November 2008.

- Airspace Safety Monitoring Activities/Requirements in the Asia/Pacific Region

- RASMAG List of Competent Airspace Safety Monitoring Organizations

  - RASMAG is required by its terms of reference to recommend and facilitate the implementation of airspace safety monitoring and performance assessment services, and to review and recommend on the competency and compatibility of monitoring organizations.  Accordingly, the meeting reviewed and updated the ‘RASMAG List of Competent Airspace Safety Monitoring Organizations’ shown at Attachment B to this paper for use by States requiring airspace safety monitoring services.

Note: Attachment B is the information at the time of RASMAG/14.  Subsequent update will be made at RASMAG/15 in August 32011.)

Update of HMU Implementation in Japan

  - Japan informed the meeting that ‘Setouchi’ Height Monitoring Unit (HMU) had been already installed.  The flight validation by the flight inspection team of JCAB would start in February 2011. JCAB will start a trial operation of height monitoring in April 2011.  The purpose of the trial will be to collect enough samples of actual height keeping performance data and validate the accuracy of the readings.  It was expected that a trial of at least five months will be needed.  The official start of operations was planned in September 2011.

  - The results for each aircraft monitored will be published on the website (http://www.japan-rma-ema.com/), when measured values are appropriate.  JCAB RMA will provide the value of monitoring result only when the operators request the value for their aircraft.  The data will be updated once or twice a month.

Data Link Performance Monitoring Results

  - The meeting was informed that Informal South Pacific ATS Coordination Group (ISPACG) Central Reporting Agency (CRA) had for some time published a collection of data link monitoring data on its website at http://www.ispacg-cra.com/performance.asp.  De-identified information is presented by aircraft type and by operator, and provides a useful overview of data link performance in the South Pacific.  The data refers to the Auckland Oceanic FIR and is presented on a monthly basis.

  - Continuity is the required probability that an operational communication transaction can be completed within the communication transaction time, either expiration time (ET) or nominal time (TT 95%), given that the service was available at the start of the transaction.  The 95% figure in each case represents the TT within which 95% of all transactions must be completed; the 99.9% figure is the ET, which is the maximum time for the completion of the operational communication transaction after which the initiator is required to revert to an alternative procedure.

  - The meeting noted that while the safety targets for network availability were being achieved at present, it was clear that considerable improvement was necessary if the efficiency target is to be met.  The efficiency target supports operational efficiency and orderly flow of air traffic. The nominal times for ADS-C and CPDLC continuity were being achieved, but some improvement is necessary to reach the target for expiration time.

  - The meeting discussed whether States understood that this type of performance monitoring was an on-going post-implementation requirement. The United States indicated that Appendix D of the Global Operational Data Link Document (GOLD) was based on post-implementation monitoring and corrective action. They noted that the Federal Aviation Administration (FAA) was doing some work to automate the charting of GOLD formatted data and would share that with States on request. Further discussion indicated that there was a need to take some action to encourage ANSPs to provide data link performance data to CRAs. The Secretary advised that he had personally discussed such issues with those States that had not been providing data in an attempt to educate them to the requirements. New Zealand suggested that possibly the FANS Implementation/Interoperability Teams (FITs) should be asked to undertake such an education program.  The meeting agreed to this suggestion and indicated that this may also occur at the South-East Asia ATM Coordination Group (SEACG) meetings. The Secretary was tasked with conveying RASMAG’s concern to relevant coordination groups and FITs.

ADS-C/CPDLC Data Link Performance Monitoring

  - New Zealand advised the meeting while the Guidance Material for End-To-End Safety and Performance Monitoring of Air Traffic Service (ATS) Data Link Systems in the Asia/Pacific Region includes information on the performance data that ANSPs are expected to provide to the CRAs, to date the CRAs have received very little such data.  Consequently, little is known of data link performance in much of the Region, with the inevitable corollary that poor performance may not be detected or corrected.

  - It is therefore most important that all ANSPs, whether state agencies or independent organisations, collect the required data link performance data and pass it to the appropriate CRA for analysis, investigation and initiation of any required corrective action.  The secretariat agreed with New Zealand’s observation and informed the meeting that there had been no data provided from States to FIT for the Bay of Bengal (FIT-BOB).  This lack of data hinders FIT-BOB from examining the data link system performance data to allow for the reduced longitudinal separation to 50 NM, thus pending the implementation.  In the South China Sea area, data are adequately provided to FIT for Southeast Asia (FIT-SEA).

  - The meeting discussed the information presented and recommended appropriate action to encourage ANSPs to provide data link performance data to the CRAs.  Accordingly, the meeting proposed a recommendation.

Clarification of Reporting Requirements by ANSPs for Category D and M Operational Errors

  - As an additional source of information for States, it was proposed that specific examples of incidents/reports for each LHD taxonomy category are provided.  Table 6 below provides suggested wording and examples for some of the categories for consideration.

| Code | LHD Cause |
| --- | --- |
| Operational Errors |  |
| A | Flight crew failing to climb/descend the aircraft as cleared |
|  | Example: Aircraft A was at F300 and assigned F360. A CLAM alert was seen as the aircraft passed F364. The Mode C level reached F365 before descending back to F360. |
| B | Flight crew climbing/descending without ATC Clearance |
|  | Example: At 0648, Aircraft A reported leaving cruise level FL340. The last level clearance was coincident with STAR issue at 0623, when the flight was instructed to maintain FL340. ATC was applying vertical separation between Aircraft A and two other flights. The timing of the descent was such that Aircraft A had become clear of the first conflicting aircraft and there was sufficient time to apply positive separation with the other. |
| C | Incorrect operation or interpretation of airborne equipment (e.g. incorrect operation of fully functional FMS, incorrect transcription of ATC clearance or re-clearance, flight plan followed rather than ATC clearance, original clearance followed instead of re-clearance etc) |
|  | Example: The aircraft was maintaining a flight level below the assigned altitude. The altimeters had not been reset at transition. The FL assigned was 350. The aircraft was maintaining 346 for 4 min 46 s. |
| D | ATC system loop error; (e.g. ATC issues incorrect clearance or flight crew misunderstands clearance message. Includes situations where ATC delivery of operational information, including as the result of hear back and/or read back errors, is absent, delayed, incorrect or incomplete, and may result in a loss of separation.) |
|  | Example: All communications between ATC and aircraft are by HF third party voice relay. Aircraft 1 was maintaining FL360 and requested FL380. A clearance to FL370 was issued, with an expectation for higher levels at a later point. A clearance was then issued to Aircraft 2 to climb to F390, this was correctly read back by the HF operator, but was issued to Aircraft 1. The error was detected when Aircraft 1 reported maintaining F390. |
| E | Coordination errors in the ATC to ATC transfer or control responsibility as a result of human factors issues (e.g. late or non-existent coordination, incorrect time estimate/actual, flight level, ATS route etc not in accordance with agreed parameters) |
|  | Example 1: Sector A coordinated Aircraft 1 to Sector B at FL380. The aircraft was actually at FL400. Example 2: The Sector A controller received coordination on Aircraft 1 for Waypoint X at F370 from Sector B. At 0504 Aircraft 1 was at Waypoint X at F350 requesting F370. |
| F | Coordination errors in the ATC to ATC transfer or control responsibility as a result of equipment outage or technical issues |
| Aircraft Contingency Events |  |
| G | Deviation due to aircraft contingency event leading to sudden inability to maintain assigned flight level (e.g. pressurization failure, engine failure) |
| H | Deviation due to airborne equipment failure leading to unintentional or undetected change of flight level |
| Deviation due to Meteorological Condition |  |
| I | Deviation due to turbulence or other weather related cause |
| Deviation due to TCAS RA |  |
| J | Deviation due to TCAS resolution advisory, flight crew correctly following the resolution advisory |
| K | Deviation due to TCAS resolution advisory, flight crew incorrectly following the resolution advisory |
| Other |  |
| L | An aircraft being provided with RVSM separation is not RVSM approved (e.g. flight plan indicating RVSM approval but aircraft not approved, ATC misinterpretation of flight plan) |
| M | Other – this includes situations where: i) there has been a failure to establish or maintain a separation standard between aircraft; or ii) flights are operating (including climbing/descending) in airspace where flight crews are unable to establish normal air-ground communications with the responsible ATS unit. |

Table 6: Taxonomy with Examples

3.	ACTION BY THE MEETING

  - The meeting is invited to note the information.

……………………………

ATTACHMENT A

Airframes Continuously Conducted RVSM Flights with No Registration in RMAs’ Approval Databases for 12 Months

The list below shows the 77 airframes identified by JCAB RMA, which were flying RVSM airspace of Fukuoka FIR with “W” on their flight plans but without the registration in RMAs’ RVSM approval databases for 12 months in a row between December 2009 and November 2010.

1/3

| Registration Number | Operator name | Aircraft Type | Airline | Country |
| --- | --- | --- | --- | --- |
| CFRAM | ACA | B77W | Air Canada | Canada |
| HL7788 | AAR | A320 | Asiana Airlines | South Korea |
| HL7789 | AAR | A321 |  |  |
| HL7790 | AAR | A321 |  |  |
| HL7792 | AAR | A333 |  |  |
| HL8204 | ESR | B737 | Eastar Jet |  |
| HL7779 | JJA | B738 | Jeju Air |  |
| HL7780 | JJA | B738 |  |  |
| HL7796 | JJA | B738 |  |  |
| HL8206 | JJA | B738 |  |  |
| HL7798 | JNA | B738 | Jin Air |  |
| HL7400 | KAL | B744 | Korean Air |  |
| HL7473 | KAL | B744 |  |  |
| HL7530 | KAL | B772 |  |  |
| HL7531 | KAL | B772 |  |  |
| HL7733 | KAL | B772 |  |  |
| HL7783 | KAL | B77W |  |  |
| HL7784 | KAL | B77W |  |  |
| HL7786 | KAL | B738 |  |  |

2/3

| Registration Number | Operator name | Aircraft Type | Airline | Country |
| --- | --- | --- | --- | --- |
| HSTLD | THA | A345 | Thai Airways International | Thailand |
| RP | CEB | A320 | Cebu Pacific | Philippines |
| RPC3330 | PAL | A333 | Philippine Airlines |  |
| RPC3331 | PAL | A333 |  |  |
| RPC3332 | PAL | A333 |  |  |
| RPC3333 | PAL | A333 |  |  |
| RPC3335 | PAL | A333 |  |  |
| RPC3336 | PAL | A333 |  |  |
| RPC3337 | PAL | A333 |  |  |
| RPC3340 | PAL | A333 |  |  |
| RPC3430 | PAL | A343 |  |  |
| RPC3431 | PAL | A343 |  |  |
| RPC3432 | PAL | A343 |  |  |
| RPC3434 | PAL | A343 |  |  |
| RPC7471 | PAL | B744 |  |  |
| RPC7472 | PAL | B744 |  |  |
| RPC7473 | PAL | B744 |  |  |
| RPC7475 | PAL | B744 |  |  |
| RPC7777 | PAL | B773 |  |  |
| RPC8604 | PAL | A320 |  |  |
| RPC8605 | PAL | A320 |  |  |
| RPC8606 | PAL | A320 |  |  |
| RPC8609 | PAL | A320 |  |  |
| RPC8610 | PAL | A320 |  |  |
| RPC8611 | PAL | A320 |  |  |
| RPC8612 | PAL | A320 |  |  |
| RPC8613 | PAL | A320 |  |  |
| RPC8614 | PAL | A320 |  |  |
| RPC8615 | PAL | A320 |  |  |

3/3

| Registration Number | Operator name | Aircraft Type | Airline | Country |
| --- | --- | --- | --- | --- |
| VN | HVN | A321 | Vietnam Airlines | Vietnam |
| VNA147 | HVN | B772 |  |  |
| VNA151 | HVN | B772 |  |  |
| VNA309 | HVN | A320 |  |  |
| VNA311 | HVN | A320 |  |  |
| VNA350 | HVN | A321 |  |  |
| VNA352 | HVN | A321 |  |  |
| VNA353 | HVN | A321 |  |  |
| VNA354 | HVN | A321 |  |  |
| VNA356 | HVN | A321 |  |  |
| VNA357 | HVN | A321 |  |  |
| VNA359 | HVN | A321 |  |  |
| VNA360 | HVN | A321 |  |  |
| VNA361 | HVN | A321 |  |  |
| VNA362 | HVN | A321 |  |  |
| VNA369 | HVN | A332 |  |  |
| VNA370 | HVN | A332 |  |  |
| VNA371 | HVN | A332 |  |  |
| VNA372 | HVN | A332 |  |  |
| VNA374 | HVN | A332 |  |  |
| VNA375 | HVN | A332 |  |  |
| VPBDQ | RCF | MD11 | Aeroflot-Cargo | Russia |
| VPBDR | RCF | MD11 |  |  |
| RA64045 | VLK | T204 | Vladivostok Air |  |
| VPBFX | VLK | A320 |  |  |
| VPBFY | VLK | A320 |  |  |
| VPBUF | UZB | B763 | Uzbekistan Airways | Uzbekistan |
| HL7799 | HL7 | GLF5 |  |  |
| HS | BCC | B762 |  |  |

- APANPIRG Asia/Pacific Airspace Safety Monitoring

- RASMAG LIST OF COMPETENT AIRSPACE SAFETY MONITORING ORGANIZATIONS

The Regional Airspace Safety Monitoring Advisory Group of APANPIRG (RASMAG) is required by its terms of reference to recommend and facilitate the implementation of airspace safety monitoring and performance assessment services and to review and recommend on the competency and compatibility of airspace monitoring organizations. In order to assist in addressing these requirements, RASMAG updates and distributes the following list of competent airspace safety monitoring organizations for use by States requiring airspace safety monitoring services. In the context of the list, abbreviations have meanings as follows:

- RMA – Regional Monitoring Agency – safety assessment and monitoring in the vertical plane (i.e. RVSM);

- EMA – En-route Monitoring Agency – safety assessment and monitoring in the horizontal plane (i.e. RVSM, RNAV10, RNP4);

- CRA – Central Reporting Agency – technical performance of data link systems (i.e. ADS/CPDLC); and

- FIT – FANS 1/A Interoperability/Implementation Team – parent body to a CRA.

(Last updated 23 February 2011)

| Organisation (including contact officer) | State | Competency | Status | Airspace assessed (FIRs) |
| --- | --- | --- | --- | --- |
| Australian Airspace Monitoring Agency (AAMA) - Airservices Australia http://www.airservicesaustralia.com/organisations/aama/default.asp Mr. Robert Butcher, Operational Analysis Manager, Safety and Assurance Group email: robert.butcher@airservicesaustralia.com or aama@airservicesaustralia.com | Australia | APANPIRG RMA | Current | Brisbane, Honiara, Jakarta, Melbourne, Nauru, Port Moresby and Ujung Pandang (including Timor-Leste) FIRs |
|  |  | EMA | Current | Brisbane, Melbourne FIRs. |
| China RMA - Air Traffic Management Bureau, (ATMB) of Civil Aviation Administration of China (CAAC) http://www.chinarma.cn (secure site) Mr. Tang Jinxiang, Engineer of Safety and Monitoring Technical Group, ATMB email: tangjx@adcc.com.cn | China | APANPIRG RMA | Current | Beijing, Guangzhou, Kunming, Lanzhou, Shanghai, Shenyang, Urumqi Wuhan Sanya and Pyongyang FIR. |
| JCAB RMA - Japan Civil Aviation Bureau Mr. Noritoshi Suzuki, Special Assistant to the Director, Flight Procedures and Airspace Program Office, email: suzuki-n248@mlit.go.jp | Japan | APANPIRG RMA | Current | Fukuoka FIR |
|  |  | EMA | Available fourth quarter – 2011 | Fukuoka FIR |
| Monitoring Agency for the Asia Region (MAAR) – Aeronautical Radio of Thailand LTD http://www.aerothai.co.th/maar Mr. Nuttakajorn Yanpirat, Executive Officer, Systems Engineering,Aeronautical Radio of Thailand Ltd. email: nuttakajorn.ya@aerothai.co.th or maar@aerothail.co.th | Thailand | APANPIRG RMA | Current | Bangkok, Kolkatta, Chennai, Colombo, Delhi, Dhaka, Hanoi, Ho Chi Minh, Hong Kong, Karachi, Kathmandu, Kota Kinabalu, Kuala Lumpur, Lahore, Male, Manila, Mumbai, Phnom Penh, Singapore, Taibei, Ulaan Bataar, Vientiane, Yangon FIRs |
| Pacific Approvals Registry and Monitoring Organization (PARMO) – Federal Aviation Administration (US FAA) http://www.faa.gov/air_traffic/separation_standards/parmo/ Mr. Dale Livingston, Manager, Separation Standards Analysis Team, FAA, email: dale.livingston@faa.gov or aparmo@faa.gov | USA | APANPIRG RMA | Current | Anchorage Oceanic, Auckland Oceanic, Incheon, Nadi, Oakland Oceanic, Tahiti FIRs |
|  |  | EMA | Current | Anchorage Oceanic, Oakland Oceanic |
| South East Asia Safety Monitoring Agency (SEASMA) - Civil Aviation Authority of Singapore (CAAS) Mr. Kuah Kong Beng, Chief Air Traffic Control Officer, email: KUAH_Kong_Beng@caas.gov.sg | Singapore | EMA for South China Sea | Current | Hong Kong, Ho Chi Minh, Kota Kinabalu, Kuala Lumpur, Manila, Sanya and Singapore FIRs |
| FIT - SEA (ICAO Regional Office email icao_apac@bangkok.icao.int & CRA Japan Mr. Mitsuo Hayasaka, Deputy Director, Air Traffic Control Association Japan, email:hayasaka@atcaj.or.jp | ICAO Regional Office & CRA Japan | FIT & CRA | Current | South China Sea FIRs |
| IPACG/FIT Mr. Takahiro Morishima, JCAB Co-Chair email: morishima-t2zg@mlit.go.jp & Mr. Reed Sladen, FAA Co-Chair, email: reed.b.sladen@faa.gov | Japan & USA | FIT & CRA | Current | North & Central Pacific (Oceanic airspace within Fukuoka FIR, and Anchorage & Oakland FIRs) |
| CRA Japan Mr. Mitsuo Hayasaka, Deputy Director, Air Traffic Control Association Japan, email: hayasaka@atcaj.or.jp | Japan | CRA | Current | Fukuoka FIR for IPACG/FIT Ho Chi Minh, Manila, Singapore FIRs for FIT-SEA |
| FIT - BOB ICAO Regional Office email icao_apac@bangkok.icao.int & Mr. Bradley Cornell, Boeing Engineering email: Bradley.D.Cornell@Boeing.Com | ICAO Regional Office & Boeing USA | FIT & CRA | Current | Bay of Bengal FIRs, Ujung Pandang and Jakarta FIRs, provides assistance to the members of the Arabian Sea/Indian Ocean ATS Coordination Group (ASIOACG) |
| ISPACG/FIT Mr. Bradley Cornell, Boeing Engineering email: Bradley.D.Cornell@Boeing.Com | Boeing USA | FIT & CRA | Current | South Pacific FIRs and members of the Informal South Pacific ATS Coordination Group (ISPACG) |