| Action Item | Description | Responsible Office | Status and Action to be taken | Date Due |
| --- | --- | --- | --- | --- |
| IP/21-5 | Implement 30 NM lateral / 30 NM longitudinal (30/30) separation | FAA JCAB | FAA implemented 30/30 in March 2007 within the Oakland FIR. JCAB introduced 30/30 in August 2008 in the Fukuoka FIR. FAA continues with the safety case analysis for the implementation of 30/30 in Anchorage FIR and planning operational trials on targets of opportunity in 2011. FAA and JCAB have implemented a trial for 30/30 cross boundary between Oakland and Fukuoka starting on May 23, 2011. Update to be provided to IPACG/35. | Oct 2011 |
| IP/22-3 | Consider operational testing of in-trail procedure (ITP) using new technologies | FAA | FAA provided a report of their progress on the ADS-C Climb Descent Procedure (CDP) and ADS-B ITP testing. ADS-C CDP Operational Trial started 15 February 2011. Update to be provided to IPACG/35. | Oct 2011 |
| IP/23-1 | Continue inter-agency discussions to conclude the exchange of traffic data for ATFM | JCAB FAA | The FAA and JCAB have developed an Interface Control Document (ICD) version 0.4. FAA and JCAB will continue to investigate options for the establishment of the ATFM data exchange and report status at IPACG/35. | Oct 2011 |
| IP/26-1 | Implement 10 minute longitudinal separation minimum without the mandatory application of Mach Number Technique | FAA JCAB | FAA and JCAB provided information updates on this issue. JCAB in process of coordinating a doc 7030 amendment for Fukuoka FIR with ICAO. FAA is completing the safety case for ZAN and plans to submit doc 7030 amendment to ICAO in 4th Qtr 2011. Update to be provided at IPACG/35. | Oct 2011 |
| IP/27-3 | Implement AIDC Ver. 3 | JCAB FAA | FAA and JCAB agreed to continue AIDC version 3, TCP/IP, and flight plan activities with an implementation after 2012. FAA and JCAB will continue to discuss progress and any problems that may occur. An update to be provided at IPACG/35, as appropriate. | Oct 2011 |
| IP/27-4 | Implement routine operational telcons between JCAB ATMC and FAA ATCSCC | JCAB FAA | The FAA and JCAB have implemented practice international teleconferences between JCAB and FAA service providers. JCAB is working to get ATMC facility managers involved in the telcons. These telcons were discussed as a means to better share information between providers and operators during emergency events. Maturity of these telcons will continue and an update will be provided at IPACG/35. | Oct 2011 |
| IP/29-1 | Speed Variation | JCAB FAA | IPACG will continue to monitor this issue as it still has operational and safety concerns with the proposed revised wording to ICAO Annex 2 paragraph 3.6.2.2 d) and to include the rationale for the wording “first speed entry in flight plan form Item 15”. FAA planning to submit a doc 7030 change related to this issue. FAA to provide an update at IPACG/35. | Oct 2011 |
| IP/29-4 | UPR/PACOTS for NOPAC Routes | FAA JCAB | JCAB and FAA presented studies on the impact of unrestricted UPRs. FAA and JCAB will collaborate on their studies and report progress at IPACG/35. | Oct 2011 |
| IP/30-5 | Develop a Trial to forward significant AIREPS to other affected aircraft. | JCAB | JCAB will continue the operational trial and report status at IPACG/35. | Oct 2011 |
| IP/31-1 | Improve operational contingency procedures for significant events. | FAA JCAB | FAA and JCAB will continue to coordinate procedures to be used in the event of a volcanic eruption or significant event that disrupts efficient ATC operations. Status to be provided at IPACG/35. | Oct 2011 |
| IP/32-2 | UPR for Track 1 | JCAB FAA | Paper trial for Track 1 completed in June 2010. Track 1 paper trial completed in November 2010. Operational trial scheduled for June 30, 2011. Status to be provided at IPACG/35. | Oct 2011 |
| IP/32-3 | PACOTS Track 3 UPRs | JCAB FAA | Track 3 paper trial completed in January 2011. Second trial scheduled in June 2011. Status to be provided at IPACG/35. | Oct 2011 |
| IP/32-4 | Restriction of H/I, J/K and Tracks 14 and 15 | IATA JCAB | A fix over between Naha and Taipei outside of radar coverage. JCAB coordinated with Taipei to move BOGUS further north into radar coverage (Naha ACC) as of August 25, 2011. Status to be provided at IPACG/35. | Oct 2011 |
| IP/32-5 | PAN Regional ICD for (oceanic) AIDC | JCAB FAA | JCAB and the FAA will continue review of the draft PAN Regional ICD document and coordinate comments. Status to be provided at IPACG/35. | Oct 2011 |
| IP/32-8 | West bound UPRs | JCAB FAA | JCAB and FAA to study potential west bound UPRs. H/I operational trials ongoing. E/F paper trials scheduled for July 2011. Updates to be provided at IPACG/35. | Oct 2011 |
| IP/33-1 | Track K UPRs | JCAB FAA | Operational trials started on May 5, 2011 on Track K. | CLOSED |
| IP/33-2 | West bound DARPs initiated within the Oakland FIR | FAA JCAB | Limited operational DARP trials using west bound aircraft (Hawaii to Narita) commenced on April 30, 2011. Updates to be provided at IPACG/35. | Oct 2011 |
| IP/34-1 | 30/30 cross boundary data | FAA JCAB | Coordinate to analyze frequency of 30/30 cross boundary operations. Update to be provided at IPACG/35. | Oct 2011 |