THE THIRTY-FIFTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/35)

(Sapporo, Hokkaido, Japan, 7-11 November 2011)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

RNP4 30 Nautical Mile (NM) Lateral/Longitudinal Distance-based Separation

(Joint Paper Presented by Civil Aviation Bureau, Japan (JCAB) and Federal Aviation Administration )

SUMMARY

This information paper provides an update to the implementation of cross-boundary use of 30/30 distance-based separation.

- Introduction

  - FAA and JCAB have been utilizing the Required Navigation Performance 30 (RNP4) 30 nautical miles (NM) distance-based separation standard internally and are comfortable with its usage.  To provide more efficient operations and help in the reduction of harmful carbon dioxide emissions into the atmosphere, the two organizations entered into a collaborative agreement for an operational trial for expansion to cross-boundary usage, which began in May 2011.

- Discussion

  - To date, the operational trial has been very successful.

  - The procedures being utilized by the two facilities during the operational trial are the same as those used for coordination of the RNP10 50 NM standard.  They are:

    - Oakland Air Route Traffic Control Center (ARTCC) and Fukuoka Air Traffic Management Center (ATMC) may utilize the Automatic Dependent Surveillance (ADS) 30NM longitudinal and lateral separation standard between RNP4 capable aircraft transiting the Fukuoka/Oakland Flight Information Region (FIR) boundary.

    - For aircraft that are separated by ADS 30 NM longitudinal and lateral separation and are transiting the Fukuoka/Oakland FIR boundary, the transferring air traffic control facility shall ensure that address forwarding, which follows the Next Data Authority (NDA) link for both aircraft, has been conducted prior to the initiation of the transfer of control.

    - The receiving controller shall accept the transfer or advise of changes necessary for acceptance of the aircraft.

    - Both Fukuoka ATMC and Oakland ARTCC may apply ADS 30 NM longitudinal separation between two RNP4 capable aircraft on reciprocal courses after the aircraft have passed each other.

    - If a data link failure is recognized on a transferred aircraft utilizing ADS 30 NM separation prior to crossing the common FIR boundary, the transferring controller is responsible for establishing another form of separation if necessary.

      - If the front aircraft has already established CPDLC Data Authority with the receiving facility or crossed the common FIR boundary, the receiving facility shall issue the coordinated revised clearance to that aircraft.

      - If an alternate form of separation cannot be established before entering the receiving facility’s control area, a Mach speed assignment to each aircraft involved shall be available as an appropriate action until another form of separation can be established between the aircraft.

  - Based on the success of the trials, the cross-boundary RNP4 30 NM separation standard causes no adverse coordination conditions; its use will be included in the inter-facility coordination agreement between the two facilities.

  - A 15 day period was reviewed to check for the frequency of 30/30 ADS-C Separation between aircraft crossing the Oakland/Fukuoka FIR boundary.  During the 15 days the standard was used 3 times between aircraft pairs crossing the boundary.  The limited use of the ADS-C 30/30 separation standard between aircraft crossing the boundary is a limitation of the number of RNP-4 equipped aircraft.  As the number of RNP-4 equipped aircraft increases, the use of the ADS-C 30/30 separation standard is expected to increase.

- Conclusion

  - The meeting is invited to note the information provided.