THE THIRTY-FIFTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/35)

(Sapporo, Hokkaido, Japan, 7-11 November 2011)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Westbound User Preferred Route (UPR) Update

Presented by Civil Aviation Bureau, Japan (JCAB) and Federal Aviation Administration (FAA)

SUMMARY

This information paper provides an update on efforts to increase use of UPRs in conjunction with PACOTS Tracks.

- Introduction

  - Westbound UPRs in conjunction with published Pacific Organized Track System (PACOTS) have been growing in popularity over the past several years.  The initial trials, conducted between Hawaii and Japan, began in August 2008 and have since been adopted as a standard operational practice.

  - UPRs in conjunction with PACOTS Tracks H/I have been an ongoing operational trial since April 2010.  Beginning 25 August 2011, UPRs in conjunction with PACOTS Tracks H/I are considered a standard operational practice.

- Discussion

  - FAA and JCAB continue with plans for a westbound PACOTS Tracks E/F UPR operational trial.  The paper trial conducted in July 2011 indicated potential for fuel savings, even though user participation was disappointing. Only one operator provided data for the paper trial.

  - A second paper trial to collect data on winter PACOTS E/F UPRs will begin on 28 November 2011.  The paper trail will be conducted with the following guidelines:

    - The paper trial will run for a four week period with participating operators generating UPRs for their flights departing on Mondays to compare against the current PACOTS routes in use that day.  The comparison will look for ATC impacts, operational advantages, and time and fuel savings.

    - Comparison Paper UPRs will be generated for flights departing on the following UTC dates:

      - November 28, 2011

      - December 4, 2011

      - December 11, 2011

      - December 18, 2011

    - General UPR Generation Paper Trial Guidelines:

      - The Paper Trial UPRs will remain in the Fukuoka, Anchorage and Oakland OCAs.

      - The UPR must enter oceanic airspace over one of the existing waypoints on the Oakland Oceanic FIR boundary along the west coast of North America or follow standard domestic routings to join the NOPAC Routes.

      - Flights must join a NOPAC ATS Route when transiting the Anchorage FIR. Join the route east of the waypoints listed below:

        - R580 OPAKE

        - R591 AKISU

        - G344 CARTO

      - Flights must flight plan so as to remain 50 NM south of PACOTS Track C and D (If a Track D is published).

      - In the Fukuoka FIR, operators shall file one of the following oceanic transition routes (OTR) and connect to appropriate ATS routes when entering domestic airspace:

        - KALNA OTR5 ADNAP;

        - EMRON OTR7 ADNAP OTR5 ONION;

        - EMRON OTR9 AVBET;

        - LEPKI OTR11 AVBET;

        - SEALS OTR13 VACKY;

        - MORAY OTR 15 SMOLT;

        - FERAR OTR17 FORDO; or

        - TONIK G223 ADKAK.

      - The UPR shall be planned to avoid military special use airspace when active.

      - The UPR shall include the appropriate SID/STAR.

- Action by the meeting

  - The meeting is requested to participate the paper trial of UPRs in conjunction with PACOTS Tracks E/F, and.

  - Provide data and feedback on the PACOTS Track E/F UPR paper trial to:

    - dennis.addison@faa.gov

    - imuta-t07j7@cab.mlit.go.jp

    - greg.howard@faa.gov