THE THIRTY-FIFTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/35)

(Sapporo, Hokkaido, Japan, 7-11 November 2011)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Feedback on the advantages of Dynamic Airborne Reroute Procedures (DARP) Trial

(Presented by All Nippon Airways / ANA)

SUMMARY

This information paper introduces the feedback of operational trial between Hawaii and Japan.

- Introduction

  - ANA has participated DARP TRIAL from the period of July 24-31, Sep 19-22 and Sep 24-28 with B777 and B767.

- Discussion

  - DARP introduction effect

- During the trial period, 8 flights of 17 flights implemented DARP, which resulted in the reduction of fuel consumption by 900lbs/5min at maximum and 300lbs/2min at an average. These TRIALs were implemented in summer season with the condition of week rising wind, but if they were implemented in winter season with the condition of stronger rising wind, the fuel consumed is expected to be more reduced.

  - Workload on flight crew

DARP procedures took about 20-25 minutes including 5-10 minutes of response waiting time from ATC.

However a certain workload is expected to be reduced by familiarization.

  - Workload on flight dispatchers

Preparing DARP FLIGHT PLAN took about 20-25 minutes. However a certain workload is expected to be reduced by the improvement of DARP related in-house systems and also by familiarization.
(a part of the in-house system is planned to be renovated in ANA.)

  - Future plan

After several months of DARP evaluating operation, we are planning to start DARP operation on a regular basis on the Honolulu-Japan flights.

We are now evaluating the possibility to implement DARP operation on the Japan to West Coast flights.

2.5　　 Requests

At first we’d like to ask DARP operation available in both FUKUOKA FIR and OAKLAND FIR areas not only Honolulu-Japan flights but for the Japan-Honolulu, Japan-West Coast flights. Secondary we would like to ask JCAB about early start-up of DARP related systems. And lastly, we would like to consider about the coordination with Airline system which enables monitoring the communication on CPDLC. This enables flight dispatchers to understand the progress of DARP. This is very important because flight dispatchers needs to monitor many DARP flights once the DARP procedure became popular in the future.

- Conclusion

  - The meeting is invited to note the information provided.