# THE THIRTY-FIFTH MEETING OF THE
IFORMAL PACIFIC ATC CO-ORDINATING GROUP
(IPACG/35)

# THE TWENTY-SECOND MEETING OF THE IPACG FANS INTEROPERABILITY TEAM
(IPACG FIT/22)

# (Sapporo, Hokkaido, Japan, 8 November 2011)

Agenda Item 4:	Working Papers

CPDLC Route Clearance Messages

(Presented by the FAA)

Summary

The purpose of this paper is to discuss the removal of route clearance messages from the CPDLC message set used for analysis of Required Communication Performance. Analysis is presented to show the effect of removing these messages on the measurement of the actual communication performance (ACP) in New York flight information region (FIR).

# Introduction

## The Global Operational Data Link Document (GOLD) provides the guidance material for the data collection and performance measurement for the application of CPDLC. The prescribed CPDLC analysis is specified in terms of required communication performance (RCP) and is measured using the transit and response times of those CPDLC uplinks that receive a single DM 0 WILCO response, including those message elements related to route clearances:

- UM 79 CLEARED TO [position] VIA [route clearance]

- UM 80 CLEARED [route clearance]

- UM 83 AT [position] CLEARED [route clearance]

## This paper discusses a proposal to remove the CPDLC transactions related to route clearances in the measurement of ACP and to change the GOLD accordingly.

# Discussion

## The following analysis was conducted to determine the impact these route clearance message elements have on Actual Communication Performance (ACP). The GOLD-format CPDLC data for the period from January to March 2011 was used for this analysis.

## During the three-month period from January to March 2011, there were 17,953 observed CPDLC transactions in which a single DM 0 WILCO response was received in the New York FIR. Of these messages 3,680 were related to route clearances (UM79, UM80, UM83).

## During the same time period there were 45,705 observed CPDLC transactions in which a single DM 0 WILCO response was received in the Oakland FIR. Of these messages only 570 were related to route clearances (UM79, UM80, UM83).

## The percentage of CPDLC transactions used to measure ACP that are related to route clearances is approximately 20.5% in New York FIR compared with approximately 1.2% in Oakland FIR. Therefore, the effect of the removal of these messages on ACP will be significantly more noticeable in New York FIR.

## Figure 1 shows the ACP analysis for the New York FIR during the period from January to March 2011 for transactions sent over all media types. This analysis illustrates the effect of filtering out those CPDLC transactions related to route clearances.

## The series labeled “All” represents all observed CPDLC transactions in which a single DM 0 WILCO response was received. The series labeled “With Filter” represents the subset of these messages that are not related to route clearances (UM79, UM80, UM83). The number of CPDLC transactions included in this analysis for each series is shown in the legend of Figure 1. There were 17,953 total transactions (All) and 14,273 of the total transactions that were not related to route clearances (With Filter).

Figure 1.	ACP – New York FIR – All RGS

## Figure 2 shows the same analysis for those transactions sent via satellite media only. Again, the number of CPDLC transactions included in this analysis for each series is shown in the legend of Figure 2. There were 15,856 total transactions (All) sent via satellite and 13,303 that were not related to route clearances (With Filter).

Figure 2.	ACP – New York FIR – Satellite RGS

## The CPDLC transactions related to route clearance messages (UM79, UM80, UM83) have a clear impact on the measured ACP. These messages require more time for the pilot to review and respond, thus affecting the pilot operational response time (PORT), which is one of the two components of the ACP.

## It has been proposed that these route clearance messages do not reflect the conflict resolution capability of the controller and should thus be removed from all future ACP analysis. This change is also proposed for inclusion in the next edition of the GOLD.

## All proceeding ACP analyses presented to this meeting will reflect this proposed change. As illustrated above, this change will significantly affect the ACP analysis for the New York FIR. Due to the fact that these route clearance messages are not commonly used in the Oakland or Anchorage FIR the effect will not be noticeable in the ACP analysis for those regions.

# Action by the meeting

## The Group is invited to:

- Note the information in this paper;

- Provide comment on the proposal to remove the route clearance messages from the RCP analysis; and

- Agree on proposal to amend the GOLD, as appropriate.

-END-