User Manual

Version 1.0

August 2011

US Federal Aviation Administration Technical Center

Atlantic City, New Jersey

- Introduction

  - The GOLD Performance Analysis Tool (G-PAT) is used to monitor performance of the operational data link system in a given airspace, by analyzing operational data and comparing the results to the specifications outlined in the Global Operational Data Link Document (GOLD).

  - The G-PAT creates GOLD-format performance charts for both the CPDLC and ADS-C messages.  The performance for CPDLC messages are measured against the Required Communication Performance (RCP).  The message transit times for ADS-C reports are measured against the Required Surveillance Performance (RSP).  Both the RCP and RSP criteria as described in the GOLD.

  - The G-PAT creates GOLD-format charts and reports with up to 2 levels of filtering, i.e. for a specific aircraft type used by a specific operator. For example, to monitor the fleet of B744 used by Operator ABC, the user would apply a first level filter by selecting Operator ABC from the list of Operators available in the data, then apply a second level filter by selecting Aircraft Type B744 from the list of Aircraft Types available in the data for Operator ABC.

  - This manual provides the format required for the CPDLC and ADS-C input files and details how to use the Tool. Screen shots of the various steps are included along with sample output. GOLD charts and reports are created by the G-PAT by following these steps.

- Loading data into the Tool

- First Level Filtering

- Second Level Filtering

- Selection of Performance Parameter and Criteria

- Generation of the Output Chart and Report

- Modification of the Chart

- Save or Print a Copy of the Chart or Report

- Input Files

  - The input files containing the CPDLC and ADS-C operational data should be in the comma separated values (CSV) format specified in the GOLD. The files should contain all of the fields prescribed in the GOLD as well as an additional field appended to the end of each line specifying the media type.

  - It is essential to include a header row in each input file.  The G-PAT uses this header row to distinguish between ADS-C and CPDLC input data.  The header row also provides the user with an extra means to verify that the fields are in the expected order.

  - Table 1 describes the 20 fields required for the input data files containing the CPDLC data link messages.  The first 19 fields contained in Table 1 are from the GOLD, Appendix D, Table D-1.  The last row in Table 1, is the media type used for the CPDLC transaction, and is necessary to distinguish performance amongst the satellite (SAT), VHF and HF data link messages.

  - Field 20, which contains the data link communication type, will have one of the nine values listed in column 3 of Table 2.  This information is based on the communication types associated with both the indicated MAS RGS and OPS RGS data fields.

  - The header row required for the CPDLC input data is:

ATSP,REGNO,ACTYPE,OPER,DATE,MASRGS,OPRGS,UTIME,MASTIME,MASRTRP,FMSTIME,ATSPTIME,OPRTRIP,DTIME,UMSG,DMSG,ACTP,ACP,PORT,COMTYP

Table 1.  Required CPDLC Input Data

| Ref | Label | GOLD Field Name | Description and/or remarks |
| --- | --- | --- | --- |
| 1 | ATSP | ATSP | The four letter ICAO designator of the FIR, e.g. NZZO. |
| 2 | REGNO | Aircraft registration | The aircraft registration in ICAO Doc 4444 Format (no hyphens, packing dots, etc.), e.g. N104UA. Note.— Extracted from ACARS header or application message. |
| 3 | ACTYPE | Aircraft type designator | The ICAO aircraft type designator, e.g. B744. Note.— Extracted from ATSP database using aircraft registration as key. |
| 4 | OPER | Operator designator | The ICAO designator for the aircraft operating agency, e.g. UAL. Note.— Extracted from ATSP database using aircraft registration as key. |
| 5 | DATE | Date | In YYYYMMDD format, e.g. 20081114. Note.— Extracted from ATSP system data recording time stamp, synchronized to within 1 second of Universal Time Coordinated (UTC). |
| 6 | MASRGS | MAS RGS | Designator of the RGS that MAS downlink was received from, e.g. POR1. Note.— This is a 3 or 4 letter designator extracted from the ACARS header DT line. |
| 7 | OPRGS | OPS RGS | Designator of the RGS that the operational response was received from, e.g. AKL1. Note.— This is a 3 or 4 letter designator extracted from the ACARS header DT line. |
| 8 | UTIME | Uplink time | The timestamp on the uplink CPDLC message sent by the ATSP in HH:MM:SS format, e.g. 03:43:25. Note.— Extracted from ATSP system data recording time stamp, synchronized to within 1 second of UTC. |
| 9 | MASTIME | MAS receipt time | The ATSP timestamp on receipt of the MAS in HH:MM:SS format, e.g. 03:43:55. Note.— Extracted from ATSP system data recording time stamp, synchronized to within 1 second of UTC. |
| 10 | MASRTRP | MAS round trip time | In seconds (#9-#8), e.g. 10. |
| 11 | FMSTIME | Aircraft FMS time stamp | In the operational response messages in HH:MM:SS, e.g. 03:44:15. Note.— Extracted from the ATCmessageHeader timestamp in the decoded operational response message. See RTCA DO-258AEUROCAE ED-100A section 4.6.3.3. |
| 12 | ATSPTIME | ATSP timestamp on the receipt of the operational response | In HH:MM:SS, e.g. 03:44:45. Note.— Extracted from ATSP system data recording time stamp, synchronized to within 1 second of UTC. |
| 13 | OPRTRIP | Operational message round trip time | From sending uplink (#8) to receipt of operational response (#12) in seconds, e.g. 80. |
| 14 | DTIME | Downlink response transit time | In seconds (#12-#11), e.g. 30. |
| 15 | UMSG | Uplink message elements | All uplink message element identifier preceded by U encapsulated between quotation marks with a space between each element, e.g. “U118 U80” Note.— Extracted from the decoded operational uplink that initiated the transaction. |
| 16 | DMSG | Downlink message elements | All downlink message elements encapsulated between quotation marks with a space between each element if required, e.g. “D0” Note.— Extracted from the decoded operational downlink. |
| 17 | ACTP | ACTP | Actual communication technical performance in seconds, e.g. 35. Note.— Truncated to whole seconds. |
| 18 | ACP | ACP | Actual communications performance in seconds measured as the difference between time uplink sent (#8) to operational response received (#12), e.g. 80. |
| 19 | PORT | PORT | Pilot Operational Response Time = ACP (#18) – ACTP(#17), e.g. 45. |
| 20 | COMTYP | COMTYP | Data link communication type, e.g. SAT Note: There are nine possible entries for COMTYP are SAT, VHF, HF, SV, SH, VS, VH, HS, HV. Value is based on the MAS RGS field (#6) and OPS RGS (#7). See Table 2. Satellite (SAT), Very High Frequency (VHF), High Frequency (HF) |

Table 2.  Possible Values for the Data Link Communication Type in the CPDLC Data

| MAS RGS Communication Type | OPS RGS Communication Type | COMTYP |
| --- | --- | --- |
| SAT e.g. MAS RGS = POR1 | SAT e.g. OPS RGS = POR1 | SAT |
| VHF e.g. MAS RGS = ADK | VHF e.g. OPS RGS = ADK | VHF |
| HF e.g. MAS RGS = H02 | HF e.g. OPS RGS = H02 | HF |
| SAT e.g. MAS RGS = POR1 | VHF e.g. OPS RGS = ADK | SV |
| SAT e.g. MAS RGS = POR1 | HF e.g. OPS RGS = H02 | SH |
| VHF e.g. MAS RGS = ADK | SAT e.g. OPS RGS = POR1 | VS |
| VHF e.g. MAS RGS = ADK | HF e.g. OPS RGS = H02 | VH |
| HF e.g. MAS RGS = H02 | VHF e.g. OPS RGS = ADK | HV |
| HF e.g. MAS RGS = H02 | SAT e.g. OPS RGS = POR1 | HS |

  - Table 3 describes the 13 data fields required for the ADS-C input file.  The first 12 fields contained in Table 1 are taken from the GOLD, Appendix D, Table D-2.  The last row in Table 3, field 13, is the media type, and is necessary to distinguish performance amongst the satellite (SAT), VHF and HF data link messages.

Table 3.  Required ADS-C Input Data

| Ref | Label | GOLD Field Name | Description and/or remarks |
| --- | --- | --- | --- |
| 1 | ATSP | ATSP | The four letter ICAO designator for the FIR of the reporting ATSP, e.g. NZZO. |
| 2 | REGNO | Aircraft Registration | The aircraft registration in ICAO Doc 4444 Format (no hyphens, packing dots, etc.), e.g. N104UA. Note.— Extracted from ACARS header or application message. |
| 3 | ACTYPE | Aircraft Type Designator | The ICAO aircraft type designator, e.g. B744. Note.— Extracted from ATSP database using aircraft registration as key. |
| 4 | OPER | Operator Designator | The IATA designator for the aircraft operating agency, e.g. UAL. Note.— Extracted from ATSP database using aircraft registration as key. |
| 5 | DATE | Date | In YYYYMMDD format, e.g. 20081114. Note.— Extracted from ATSP system data recording time stamp, synchronized to within 1 second of UTC. |
| 6 | RGS | RGS | Designator of the RGS that ADS-C downlink was received from, e.g. POR1. Note.— This is a 3 or 4 letter designator extracted from the ACARS header DT line. |
| 7 | TYP | Report Type | The type of ADS-C report extracted from the ADS-C basic group report tag where tag value 7=PER, 9=EMG, 10=LDE, 18=VRE, 19=ARE, 20=WCE. As some aircraft concatenate more than one report in the same downlink extract the ADS-C report tag from each ADS-C basic group and identify them in the REP_TYPE column by using the first letter of the report type as an identifier e.g. for a concatenated report containing two ADS-C basic groups for a periodic report and a waypoint event report the field will contain PW. Where a downlink does not contain a ADS-C basic group the REP_TYPE field will be left blank. |
| 8 | LAT | Latitude | The current latitude decoded from the ADS-C basic group. The format is “+” for North or “-“ for South followed by a decimal number of degrees, e.g. -33.456732. |
| 9 | LONG | Longitude | The current longitude decoded from the ADS-C basic group. The format is “+” for East or “-“ for West followed by a decimal number of degrees, e.g. +173.276554. |
| 10 | STIME | Aircraft Time | The time the ADS-C message was sent from the aircraft in HH:MM:SS, e.g. 03:44:15. Note.— Decoded from the ADS-C basic group timestamp extracted as seconds since the most recent hour. See RTCA DO-258A/EUROCAE ED-100A, section 4.5.1.4. |
| 11 | RTIME | Received Time | The ATSP timestamp on the receipt of the ADS-C message in HH:MM:SS, e.g. 03:44:45. Note.— Extracted from ATSP system data recording time stamp, synchronized to within 1 second of UTC. |
| 12 | DEL | Transit Time | The transit time of the ADS-C downlink in seconds calculated as the difference between #10 Aircraft Time and #11 Received Time, e.g. 30. |
| 13 | COMTYP | COMTYP | Data link communication type, e.g. SAT Note: There are three possible entries for COMTYP are SAT, VHF, or HF. Value is based on the RGS field (#6). Satellite (SAT), Very High Frequency (VHF), High Frequency (HF) |

  - The header row required for the ADS-C input data is:

ATSP,REGNO,ACTYPE,OPER,DATE,RGS,TYP,LAT,LONG,STIME,RTIME,DEL,COMTYP

# Loading Data Into the Tool

  - The first step in using the Tool is to load the CPDLC or ADS-C input data. This is accomplished by selecting “Open” from the File Pull-down Menu of the Toolbar.  Alternately, the user can click the “Open” button below the Toolbar as illustrated in Figure 1. From here, the user selects the data of interest to be loaded into the Tool.  The input data must be in a comma separated format following the data field structure described in Table 1 for CPDLC data or Table 3 for ADS-C data. It is important that the first row of the input file contain the necessary header row (see section 2 for details).

  - Once the data has been loaded, the G-PAT automatically generates a generic GOLD chart for the unfiltered data.  The header row is used to determine the input data, ADS-C or CPDLC.

Figure 1

- First-Level Filtering

  - The first level filtering option is accessed by selecting the “Filters” option on the Toolbar.  The following types of first-level filters are available:

- FIR - Flight Information Region

- Media Type – Satellite (SAT), VHF, HF , etc…

- GES - Ground Earth Stations

- Operator

- Aircraft Type

  - In the “Filters” pull-down Menu, the number of options within the selected data set that are associated with each filter type is shown next to the name of the filter type. The example shown in Figure 2 lists only one option for FIR, eight options for Media Type, and so on.  The number of available options will vary depending on the data loaded into the program.

  - Once a filter is selected from the “Filters” pull-down Menu, a new window is presented that contains further information about the selected filter.  Figure 3 shows a sample of a pop-up window for Filter by Operator.  This window lists all of the possible options for the selected filter (Operator) and how many data points exist in the data set for each of the options.  In Figure 3, there are 287 data link messages for AAL, 483 data points for AAR, and so on. Initially all options are selected.

  - By default, the filter pop-up window displays the options sorted by name in alphabetical order.  By pressing the “Sort by Count” button (located on the bottom right side of the window), the options can be sorted in descending order of the number of data points associated with each option.  The “Select All” and “Deselect All” buttons provide the named functionality which is available to aid in the selection of few or many of the available options.  To return to the default, press the “Sort by Name” button.

Figure 2

Figure 3

# Second-Level Filtering

# Once the first-level filter has been applied, a second-level filter may be chosen.  Second-level filtering is available under “Type of Analysis to Perform,” which can be found in the “Options” Pull-down Menu.  Figure 5 illustrates the pop-up window produced by selecting “Type of Analysis to Perform.”

# The default setting for this filter is the “All data” option, which applies no second-level filtering.  If second-level filtering is required, the following are the available options:

# Month

# FIR

# Media Type

# GES

# Operator

# Aircraft Type

# Airframe

Figure 1

Figure 5

  - If one of the second-level filtering options listed in the “Type of Analysis to Perform” window is selected, the options corresponding to that selection with the first-level filter applied are displayed.  For the example shown in Figure 6, the first-level filter selected was Operator ABC. When the “Aircraft Type” analysis was selected, only the available aircraft types for Operator ABC were displayed, i.e. A332 and A388.

  - In Figure 6, the numbers within parentheses to the right of the aircraft type provide the number of messages to be included in the chart based on the selected first level filter.  The number on the right represents the total number of data points for this category in all of the input data.  The number of the left represents the number of data points available for this category after the first level filter has been applied.  Thus, in Figure 6, it is shown that there are a total of 901 data points corresponding to an A332 aircraft in the input data and 133 of these belong to the Operator ABC.

  - The aircraft types contained in the input data file that are not available for Operator ABC are also shown in the window in Figure 6, but these options are shadowed as they are not available for this selection based on the first-level filter.

  - Only one option may be selected for the “Type of Analysis.” However, once the option for the “Type of Analysis” has been selected, multiple options may be selected within the second-level filter (such as multiple aircraft types).

Figure 6

# Selection of Performance Parameter and Criteria

# For CPDLC analysis, the G-PAT can create the ACP, ACTP and PORT charts.  The options for the type of chart to be created can be found by selecting “Performance Parameter” from the “Options” pull-down menu of the Toolbar shown in Figure 7.  The pop-up window generated by selecting “Performance Parameter” is shown in Figure 8.

Figure 2

Figure 8

  - There are no “Performance Parameter” options to select for the ADS-C data.  The only chart prescribed for ADS-C analysis in the GOLD is the ADS-C Downlink Latency chart so this is the only chart generated by G-PAT for ADS-C data.

  - The options for performance criteria, i.e. the 95 and 99.9 percent performance goals to be used in the chart, can be found by selecting “Performance Criteria” from the “Options” pull-down menu of the Toolbar shown in Figure 9.  The pop-up window generated by selecting “Performance Criteria” is shown in Figure 10. As shown in Figure 10, the default performance criteria are initially checked based on the selected “Performance Parameter.”

Figure 3

Figure 10

# Generation of the Output Chart and Report

Figure 4

  - Once the input data has been loaded, the first and second-level filters have been applied, and the performance parameter and criteria have been selected, the G-PAT is ready to produce the GOLD chart and report. The chart and report corresponding to the current selections are visible by selecting the appropriate tab at the bottom of the G-PAT window (circled in red in Figure 11).

  - To create the GOLD chart and report for the  existing selections in a new Tab, click the “Create Chart” button.  To create a new chart in the current Tab, overwriting the currently-displayed chart and report, click the “Refresh Chart” button.

  - Figure 12 shows a sample chart and report for the observed performance of CPDLC message transactions from the A380 and B744 aircraft types for Operator QFA using the default performance criteria (RCP240/RSP180).

Figure 5

# Modification of the Chart

  - Once a GOLD chart has been generated, the properties of the chart can be changed to modify the view.

  - The default view is a scale from 90 to 100 percent on the vertical range (y) axis, with increments of 1 percent.  To change the view to a scale from 0 to 100 percent on the vertical range axis, right-click and select “Auto Range.” To return to the default setting, press the “Refresh Chart” button.

  - To zoom on a particular portion of the chart, click and drag the mouse to draw a box enclosing the desired area.  This area will then be enlarged to fit the screen.

  - Several other chart properties are accessible by right-clicking on the chart and selecting “Properties.”  This action will open the “Chart Properties” window, which contains the three tabs shown in Figure 13: “Title”, “Plot”, and “Other.”

Figure 13

  - The “Title” tab, shown in Figure 13, allows properties of the chart title to be modified. The chart title can be hidden by deselecting the check box displayed to the right of the chart title text box. The text of the title can be changed in addition to the font, size, and color of the title.

Figure 14

  - The “Plot” tab allows properties of the horizontal domain (x) axis and of the vertical range (y) axis to be independently modified.  As shown in Figure 14, the label as well as its font, size, and color can be changed.  Additional properties, such as the showing or hiding of the tick labels and tick marks for the axes, can be modified.

  - The range displayed on the chart can be modified by selecting the “Range” tab, shown in Figure 15.  To adjust the range, deselect the checkbox for “Auto-adjust range” and then enter the desired minimum and maximum range values.

Figure 15

# Figure 16

  - The remaining tab, “Other”, shown in Figure 16, allows the line colors in the chart to be modified.

# Save or Print a Copy of the Chart or Report

# The generated chart or report can be saved using the “File” pull-down menu of the Toolbar.  Alternatively, “Save Chart” and “Save Report” buttons are available below the Toolbar.  Note that when saving the report, a valid file extension must be added to the end of the file name.

# When saving the chart using the File pull-down menu or the “Save Chart” button, the chart will be saved as a JPG file.  To save the chart as a PNG file, simply right-click the chart and select “Save as.”

Figure 6

References

1.  International Civil Aviation Organization (ICAO) Global Operational Data Link Document  (GOLD), First edition, June 2010.