THE THIRTY-FIFTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/35)

(Sapporo, Hokkaido, Japan, 7-11 November 2011)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Tailored Arrivals Update

(Presented by the Federal Aviation Administration )

SUMMARY

This information paper provides an update on the use of Tailored Arrivals in North America.

- Introduction

  - Operational use of the Tailored Arrival (TA) into KSFO began in November 2006.  This was followed by KLAX and KMIA where unique traffic conditions have created the need for restrictions in the descent profile that somewhat reduces the overall efficiency of the procedure.  However, even with the restrictions, studies indicate that meaningful fuel savings are being realized.  Work continues to enhance the current procedures at all locations.  In addition, work is underway at the FAA headquarters level to develop official FAA standards for TA development.

- Discussion

  - In a perfect environment, the TA would allow for an unrestricted descent from the aircraft’s last-assigned cruising flight level to touch down at the airport.  It has been shown that such a scenario can be accomplished about 33% of the time on the Pacific 2 Tailored Arrival.  Initially, the low number of fully completed TAs was viewed as a disappointment.  However, at least with the tailored arrival into KSFO, it was determined that an impressive fuel savings/emissions reduction was also being realized when the profile was only partially flown.

  - Analysis indicates that each fully flown TA into KSFO garners a savings of 300 to 536 kilograms of fuel (a reduction of 948 to 1693 kilograms of carbon dioxide, (CO2) while a partially flown profile produces a savings of approximately 131 to 236 kilograms of fuel (a reduction of 413 to 745 kilograms of CO2).

  - At KLAX the Catalina 1 TA provided a significant efficiency improvement over the LENNA STAR that was used for arriving traffic over SXC.  Initially, after its development, an average of 132 aircraft a month was enjoying the benefits of the Catalina 1 TA.

  - In March of 2011, the FAA published the BUFIE 1 RNAV STAR.  The BUFIE 1 STAR utilizes the same crossing restrictions as the CATALINA 1 TA and provides many of the same efficiencies.  The use of the BUFIE 1 STAR has led to a decrease in the numbers of aircraft utilizing the Catalina 1 TA.  Currently an average of 38 aircraft a month utilizes the CATALINA 1 TA.

  - A new ZILLI 1 RNAV departure procedure is due to be published on 9 February 2012.  The new ZILLI 1 will offset KLAX departures north of the CATALINA 1 TA arrivals.  It is hopeful that this will allow a more efficient descent profile for the CATALINA 1 TA.  The altitude crossing restrictions for the CATALINA 1 TA will be reviewed after the implementation of the ZILLI 1.

  - The success of the TA has caused this concept to become a significant part for the Next Generation Air Transportation System (NextGen) Airspace Modelling of Optimal Profile Descents.  Plans are underway for the timely implementation and integration of Initial Tailored Arrivals (ITA) into the National Airspace System.

- Conclusion

  - The meeting is invited to note the information in this paper.