# THE THIRTY-FIFTH MEETING OF THE
IFORMAL PACIFIC ATC CO-ORDINATING GROUP
(IPACG/35)

# THE TWENTY-SECOND MEETING OF THE IPACG FANS INTEROPERABILITY TEAM
(IPACG FIT/22)

# (Sapporo, Hokkaido, Japan, 8 November 2011)

Agenda Item 5:	Working Papers

GOLD Performance Analysis Tool (G-PAT) Status

(Presented by the FAA)

Summary

This paper provides the status for the GOLD – Performance Analysis Tool (G-PAT). The purpose of the G-PAT is to provide air navigation service providers (ANSPs) with a software tool to aid in the creation of the GOLD performance charts.  This paper also provides the draft user manual for G-PAT.

# Introduction

## This paper provides the current status of the GOLD – Performance Analysis Tool (G-PAT).  G-PAT is software which can be used to create GOLD performance charts from input CPDLC and ADS-C data.  The G-PAT has a graphical user interface (GUI) which can be run on a MS Windows-based personal computer.

## G-PAT will import pre-filtered data in the GOLD format, create the requisite cumulative distributions and generate the reports and charts selected by the user.  Currently, G-PAT is in beta test with a few selected users.  The FAA is awaiting responses from these users and will eventually make G-PAT available to all ANSPs.

## This paper contains a description of the important requirements related to the input data for use by the G-PAT.  In addition, this paper contains the current draft version of the G-PAT user manual, and samples of input data files.

# DISCUSSION

## The prescribed data for the CPDLC and ADS-C data found in Appendix D of the GOLD are needed to successfully run the software.  The input file must contain data of only one type; CPDLC or ADS-C.  A file containing the CPDLC data and another file containing the ADS-C data are needed to create the Required Communication Performance (RCP) and Required Surveillance Performance (RSP) charts.  .  G-PAT automatically determines the transaction type from the input data.

## The current draft version of the G-PAT user manual is attached as Attachment A to this paper.  Detailed instructions with screen shots are provided.

## The input data must be in comma separated format.  There are a few notes related to the input files which are important.

### The first item is the need for a header row in each input file.  For the CPDLC transaction data, this header row should appear as follows:

ATSP,REGNO,ACTYPE,OPER,DATE,MASRGS,OPRGS,UTIME,MASTIME,MASRTRP,FMSTIME,ATSPTIME,OPRTRIP,DTIME,UMSG,DMSG,ACTP,ACP,PORT,COMTYP

Although the above text wraps, it is to be the first row in the CPDLC input data files.  A small sample of input CPDLC transaction data is shown in Attachment B.

The header row for ADS-C transaction data should appear as follows:

ATSP,REGNO,ACTYPE,OPER,DATE,RGS,TYP,LAT,LONG,STIME,RTIME,DEL,COMTYP

A small sample of input ADS-C transaction data is shown in Attachment C.

### The second item that is needed in the input data is an additional field which specifies the media used to transmit the message.  This information is needed to observe data by media; for example, it might be necessary to observe performance for HF data link messages only.  The additional field is the last field in both input files; COMTYP.  Table 1 contains the possible values for COMTYP for the CPDLC transaction data.

### There are nine possible entries for the COMTYP field for the CPDLC transaction data.  The left column of Table 1 specifies the media used in the uplink CPDLC message.  The middle column in Table 1 specifies the media used in the downlink CPDLC response message.  The resulting value for COMTYP is contained in the last column of Table 1.  For example, if both the uplink CPDLC transaction and downlink CPDLC response were sent using satellite (SAT) data link, the value for COMTYP is SAT.  If the uplink CPDLC transaction was sent via SAT and the downlink CPDLC response was sent via VHF, the resulting value of COMTYP is SV.

Table 1.	Input Data - COMTYP field

| MAS RGS Communication Type | OPS RGS Communication Type | COMTYP |
| --- | --- | --- |
| SAT e.g. MAS RGS = POR1 | SAT e.g. OPS RGS = POR1 | SAT |
| VHF e.g. MAS RGS = ADK | VHF e.g. OPS RGS = ADK | VHF |
| HF e.g. MAS RGS = H02 | HF e.g. OPS RGS = H02 | HF |
| SAT e.g. MAS RGS = POR1 | VHF e.g. OPS RGS = ADK | SV |
| SAT e.g. MAS RGS = POR1 | HF e.g. OPS RGS = H02 | SH |
| VHF e.g. MAS RGS = ADK | SAT e.g. OPS RGS = POR1 | VS |
| VHF e.g. MAS RGS = ADK | HF e.g. OPS RGS = H02 | VH |
| HF e.g. MAS RGS = H02 | VHF e.g. OPS RGS = ADK | HV |
| HF e.g. MAS RGS = H02 | SAT e.g. OPS RGS = POR1 | HS |

### For the ADS-C transaction data, there are three possible entries for the COMTYP field.  The ADS-C transaction was either sent via SAT, VHF, or HF.

## It is necessary to have both the header row and valid entries in the COMTYP field for the G-PAT software to run successfully.

## The FAA recognizes that currently the G-PAT is not compliant with the GOLD formats. The FAA will consider changes to G-PAT to comply with GOLD formats or propose amendments to the GOLD, as appropriate.  Specifically:

- The header row is required in the file;

- COMTYPE is a new field, and it was suggested that it does not need to be part of the data collection from ANSPs and can/should be derived by the G-PAT tool from the xxxRGS fields.

- UMSG and DMSG field entries are in quotes in the GOLD, but not in the G-PAT format;

- ACTP, ACP and PORT have resolution to the nearest second, but in the G-PAT format to the tenth of second; and

## For Items a) and b) above, the FAA anticipates that it will propose amendments to the GOLD to require the header row and valid entries in the COMTYP field.  For items c) and d) above, the FAA is planning to revise G-PAT to comply with GOLD formats.

# Action by the Meeting

## The meeting is invited to:

- Note the information in the paper;

- Review the G-PAT manual and provide further comments to tom.kraft@faa.gov.

Attachment A.	G-PAT User Manual

[Provided as a separate file]

Attachment B.	Sample CPDLC Transaction Data for use in G-PAT

ATSP,REGNO,ACTYPE,OPER,DATE,MASRGS,OPRGS,UTIME,MASTIME,MASRTRP,FMSTIME,ATSPTIME,OPRTRIP,DTIME,UMSG,DMSG,ACTP,ACP,PORT,COMTYP

KZAK,9VSGB,A345,SIA,20101201,POR1,POR1,08:07:05,08:07:09,4,08:07:17,08:07:23,18,6,U30,DM0,18.0,8.0,10.0,SAT

KZAK,9VSGB,A345,SIA,20101201,POR1,POR1,09:47:38,09:47:42,4,09:47:57,09:48:04,26,7,U20,DM0,26.0,9.0,17.0,SAT

KZAK,9VSGB,A345,SIA,20101201,POR1,POR1,10:57:23,10:57:29,6,10:57:32,10:57:47,24,15,U82,DM0,24.0,18.0,6.0,SAT

KZAK,9VSGB,A345,SIA,20101201,POR1,POR1,12:55:12,12:55:18,6,12:55:28,12:55:51,39,23,U20,DM0,39.0,26.0,13.0,SAT

KZAK,B16412,B744,EVA,20101201,POR1,POR1,21:14:33,21:14:39,6,21:14:51,21:15:00,27,9,U31,DM0,27.0,12.0,15.0,SAT

KZAK,B16412,B744,EVA,20101201,POR1,POR1,21:41:55,21:42:02,7,21:42:10,21:42:20,25,10,U20,DM0,25.0,13.5,11.5,SAT

KZAK,B16483,B744,EVA,20101201,POR1,POR1,09:54:36,09:54:43,7,09:54:49,09:54:59,23,10,U20,DM0,23.0,13.5,9.5,SAT

KZAK,B16483,B744,EVA,20101201,POR1,POR1,12:53:38,12:53:45,7,12:53:53,12:54:24,46,31,U20,DM0,46.0,34.5,11.5,SAT

KZAK,B16701,B77W,EVA,20101201,POR1,POR1,08:44:19,08:44:29,10,08:44:32,08:44:48,29,16,U20,DM0,29.0,21.0,8.0,SAT

KZAK,B16701,B77W,EVA,20101201,POR1,POR1,09:19:53,09:20:04,11,09:20:08,09:20:24,31,16,U20,DM0,31.0,21.5,9.5,SAT

KZAK,B16701,B77W,EVA,20101201,POR1,POR1,09:52:55,09:53:05,10,09:53:15,09:53:28,33,13,U26,DM0,33.0,18.0,15.0,SAT

KZAK,B16707,B77W,EVA,20101201,POR1,POR1,21:51:06,21:51:16,10,21:51:38,21:51:52,46,14,U23,DM0,46.0,19.0,27.0,SAT

KZAK,B16707,B77W,EVA,20101201,POR1,POR1,22:03:49,22:03:59,10,22:04:00,22:04:17,28,17,U23,DM0,28.0,22.0,6.0,SAT

Attachment C.	Sample ADS-C Transaction Data for use in G-PAT

ATSP,REGNO,ACTYPE,OPER,DATE,RGS,TYP,LAT,LONG,STIME,RTIME,DEL,COMTYP

KZAK,N851FD,B77L,FDX,20101130,POR1,ADS,+51.9845581054688,-149.733180999756,23:59:48,00:00:01,13,SAT

KZAK,N69063,B764,COA,20101201,XXC,ADS,+34.5065116882324,-133.064002990723,00:00:41,00:00:59,18,SAT

KZAK,N220UA,B772,UAL,20101201,XXC,ADS,+25.242805480957,-171.414184570312,00:01:25,00:01:34,10,SAT

KZAK,N220UA,B772,UAL,20101201,XXC,ADS,+25.2455520629883,-171.426200866699,00:01:30,00:01:43,13,SAT

KZAK,B16713,B77W,EVA,20101201,POR1,ADS,+44.0457344055176,-144.599475860596,00:01:31,00:01:48,17,SAT

KZAK,JA603J,B763,JAZ,20101201,HNL,ADS,+21.0994148254395,-158.212738037109,00:01:52,00:01:56,4,VHF

KZAK,N213UA,B772,UAL,20101201,KOA,ADS,+21.4827346801758,-155.467529296875,00:02:10,00:02:14,5,VHF

KZAK,JA743J,B77W,JAL,20101201,POR1,ADS,+45.9980392456055,-150.003547668457,00:02:09,00:02:16,7,SAT

KZAK,BHUG,B744,CPA,20101201,XXC,ADS,+47.0384788513184,-179.028568267822,00:02:12,00:02:32,20,SAT

KZAK,N76065,B764,CMI,20101201,XXC,ADS,+15.1077461242676,+170.891647338867,00:02:16,00:02:35,19,SAT

KZAK,9VSPO,B744,SIA,20101201,POR1,ADS,+48.2315254211426,-130.203266143799,00:02:14,00:02:41,27,SAT

KZAK,BKPQ,B77W,CPA,20101201,XXC,ADS,+46.0011291503906,-159.993209838867,00:02:48,00:03:05,17,SAT

KZAK,N599FE,MD11,FDX,20101201,MKK,ADS,+24.048900604248,-156.320686340332,00:03:10,00:03:16,6,VHF

KZAK,JA615A,B763,ANA,20101201,POR1,ADS,+27.0016479492188,+179.997940063477,00:03:22,00:03:37,15,SAT

-END-

There is one additional field needed by G-PAT – see Table 1 in this paper