# Working Draft

# Satellite Voice Guidance Material (SVGM)

Version 0.8

Working Draft – 23 Sep 2011

International Civil Aviation Organization

Inter-Regional SATCOM Voice Task Force
(IR-SVTF)

Revision History

| Date | Description of changes | Version |
| --- | --- | --- |
| 13-Jul-10 | Initial working draft |  |
| 26-Jan-11 | Added Joint working relationship with ICAO SATCOM Voice TF, added version control. | 0.1 |
| 27-Jan-11 | Added material from IRSVTF/1 meeting. First TF baseline. no track changes. | 0.2 |
| 31-Mar-11 | Added inputs since IRSVTF/1 meeting (See comment matrix for specific changes) | 0.3 |
| 1-Jun-11 | Added inputs from review on v0.3 (See comment matrix for specific changes) | 0.4 |
| 22-Jul-11 | Added inputs from IRSVTF Web1 review and actions on v0.4 (See comment matrix for specific changes) | 0.5 |
| 23-Aug-11 | Added inputs from IRSVTF Web2 review and actions on v0.5 (See comment matrix for specific changes) | 0.6 |
| 4-Sep-11 | Added inputs from IRSVTF Web3 review and actions on v0.6 (See comment matrix for specific changes) | 0.7 |
| 23Sep-11 | Added inputs from IRSVTF/2 review and actions on v0.7 (See comment matrix for specific changes) | 0.8 |

- Editor’s notes may appear throughout the document, however, refer to the master comment matrix, which includes open comments that may not have been resolved in this “working draft” version.  Comments and contributory material should be submitted to "Elkhan NAHMADOV" enahmadov@paris.icao.int, "Catherine DALY" cdaly@paris.icao.int, and “Tom KRAFT” tom.kraft@faa.gov.

Table of contents

Page

FOREWORD	.	vi

Chapter 1.	Definitions	1-1

Chapter 2.	Overview of satellite voice communications	2-1

2.1	General	2-1

2.2	Aeronautical Satellite Communication Systems Overview	2-1

2.3	Iridium	2-2

2.4	Inmarsat	2-3

2.5	Japan Multi-Function Transport Satellite (MTSAT)	2-3

2.6	Public Switched Telephone Networks and Dialing Systems Used	2-3

Chapter 3.	Administrative provisions related to satellite voice operations	3-1

3.1	ATSP and aeronautical station service provision	3-1

3.1.2	ATSP system validation and operational readiness	3-1

3.1.3	Aeronautical information, notifications, and interfacility agreements	3-2

3.1.4	Considerations for the aeronautical station	3-2

3.1.5	Considerations for the communication service provider (CSP)	3-3

3.1.6	Considerations for the satellite service provider (SSP)	3-4

3.2	Operator eligibility	3-6

3.2.1	Operational authorization to use satellite voice communications	3-6

3.2.2	Radio equipment to be carried on board	3-7

3.2.3	Criteria for aircraft operators	3-8

3.2.4	Aircraft equipage	3-9

3.2.5	Maintenance and in-service difficulties	3-9

3.3	Flight planning	3-9

Chapter 4.	Controller and radio operator procedures	4-1

4.1	General	4-1

4.2	Controller procedures	4-1

4.3	Radio operator procedures	4-1

4.3.1	Outgoing SATVOICE calls – Radio operator initiated (ground-to-air)	4-1

4.3.2	Incoming SATVOICE calls – radio operator receives calls (air-to-ground)	4-3

4.4	Communication failures and emergencies	4-9

4.4.1	Loss of SATVOICE Connection	4-9

4.4.2	Airspace emergencies – loss of controller or radio facility capabilities	4-9

Chapter 5.	Flight crew procedures	5-1

5.1	General	5-1

5.2	Flight crew initiated	5-1

5.3	Flight crew receives call	5-2

5.4	Oceanic clearances	5-2

5.5	Contingencies	5-2

5.5.1	SATCOM busy signal or no answer	5-2

5.5.2	SATCOM Failure	5-2

List of figures

Figure 2-1.	Aeronautical satellite communication system overview	2-2

Figure 3-1.	Radio operator contracted by organizations	3-4

Figure 4-1.	Ground to air (radio operator to flight crew) SATVOICE flowchart	4-4

Figure 4-2.	Air-ground (flight crew to radio operator) SATVOICE flowchart	4-7

List of tables

Table 3-1.	Priority levels for SATCOM voice calls	3-5

Table 4-1.	Ground to air (radio operator to flight crew) SATVOICE flow chart descriptions	4-5

Table 4-2.	Air-ground (flight crew to radio operator) SATVOICE flowchart descriptions	4-8

Appendices

Appendix A	– RCP 400 specification – allocations for satellite voice communications	1

A.1	Terms and definitions	1

A.2	RCP 240 specification	6

A.3	RCP 400 specification	7

A.3.1	RCP 400/D allocations	7

A.3.2	RCP 400/V allocations	8

A.3.2.1	Air traffic service provider (ATSP)	8

A.3.2.2	Communication service provider (CSP)	10

A.3.2.3	Aircraft system	11

A.3.2.4	Aircraft operator	13

Appendix B	Post-implementation monitoring	1

B.1	ATSP or aeronautical station data collection and analysis	1

B.2	Problem reporting and resolution	1

B.3	Regional/global performance monitoring	1

Appendix C	Regional/State-specific information	1

C.1	Current uses of SATCOM voice	1

C.2	Region/State contact information	1

C.3	Planning for SATCOM voice implementation	1

Appendix D	Planning for desirable SATCOM voice features	1

D.1	2-channel SATCOM voice system	1

D.2	Caller identification display	1

D.3	Conference calling	1

- .

1.	Historical background

1.1	The Satellite Communications (SATCOM) Voice Guidance Material (SVGM) is the result of a task force established at the request from the ICAO Air Navigation Commission (ANC) made during its 4th Meeting of the 183rd Session held on 21 January 2010..

1.2	Over the oceanic and remote continental areas, aeronautical communications have historically been conducted with high frequency (HF) radios due to the advantage of being able to transmit and receive air/ground communications for thousands of miles.  Most competent authorities hence required two independent HF sets on-board.

1.3	In the early 1980s, civil aviation recognized the increasing limitations of the present communications, navigation, and surveillance (CNS) systems for air traffic management (ATM) and the need to make improvements to overcome them and meet the future needs. Thus the Council of ICAO established the Special Committee on future air navigation systems (FANS) to study new concepts and new technologies and to recommend a system that would overcome the present and foreseen problems. The Committee made an extensive study of existing systems and the applications of new technologies. It concluded that the limitations of the existing systems are intrinsic to the systems themselves and were so restrictive that the problems could not be overcome on a global scale except by the exploitation of satellite technology. Thus a new concept of air navigation based on satellite technology was developed and consequently endorsed by the Tenth Air Navigation Conference in September 1991

1.4	In 1995, the initial future air navigation system (FANS 1/A) provided an integrated airborne CNS package.  In addition to required navigation performance (RNP) and global navigation satellite system (GNSS) capabilities, FANS 1/A includes controller pilot data link communications (CPDLC) and automatic dependent surveillance – contract (ADS-C) capabilities using SATCOM, VHF, and HF data links.  CPDLC and ADS-C were seen as the normal or preferred means of ATS communications and surveillance over the oceanic and remote continental areas.  However, voice communications would continue to be required in situations where data link was not suitable. At the same time, these aircraft became equipped with SATCOM voice capability.

1.5	 In June 2001, the 37th Meeting of the NAT SPG (12-14 June 2001) agreed that a study would be initiated to assess the viability of using satellite voice communications for waypoint position reporting as an initial step. The study was accompanied by the NAT trials that had been successful and demonstrated that while there were costs associated with implementation and use, SATCOM voice could be an effective and reliable long range communication system to support ATS voice communications.

1.6	In 2003, the 39th NAT SPG Meeting (17-19 June 2003) agreed that the NAT SUPPs needed to be amended to clearly state the conditions under which SATCOM voice could be used. In 2008, the 44th Meeting of the NAT SPG (17-20 June 2008) agreed that the authorization to use SATCOM voice for all ATS communications would permit reduction in risk of communications failure, improve safety of operations, and alleviate HF congestion.  However, guidance material would be needed to address a number of issues related to call setup times, security and system performance and capacity.  It was further concluded that any decision regarding MEL relief of one HF radio was subject to approval by the appropriate authority.

1.7	Some State authorities have granted some operators time-limited MEL dispatch relief of one HF radio whereby the aircraft may be dispatched for a limited period of time (e.g., 5 or 10 days) with only a single operational HF radio system and a single operational SATCOM voice system.  Operators are now seeking MEL dispatch relief of one HF radio with no time limits by demonstrating that either the Iridium or Inmarsat SATCOM voice system meets the long range communication system (LRCS) requirements.

1.8	In 2010, the ICAO ANC having reviewed the progress of the NAT SPG SATCOM voice studies, requested that an ICAO inter-regional task force would be established to develop a globally applicable guidance material Satellite Voice Guidance Material (SVGM) in support of the global implementation of aeronautical mobile satellite (route) communications systems (AMS(R)S).

1.9	This edition of the Satellite Voice Guidance Material (SVGM) provides for a comprehensive update of various regional and State guidance material to use SATCOM voice for ATS communications.  This includes the incorporation of performance-based specifications and associated guidance on data collection, monitoring, and analysis.  This guidance material may facilitate the appropriate authority in establishing its policies on MEL for some dispatch relief.  However, it assumes that sufficient HF voice infrastructure must remain in service and that the aircraft must be equipped with at least one operational HF voice system.

2.	Scope and purpose

2.1	The SVGM provides guidance and information concerning SATCOM voice communications for aeronautical use and is intended to facilitate the uniform application of Standards and Recommended Practices contained in Annex 2 — Rules of the Air and in Annex 11 — Air Traffic Services, the provisions in the Procedures for Air Navigation Services — Air Traffic Management (PANS-ATM, Doc 4444) and, when necessary, the Regional Supplementary Procedures (Doc 7030).

2.2	This guidance material is intended to maximize operational benefits in SATCOM voice operations by promoting seamless and interoperable SATCOM voice operations throughout the world. This edition provides guidance material for current and near term operations in oceanic environments.  It also includes an appendix, which provides SATCOM voice features not currently available, to serve as a basis for development of future SATCOM voice systems.  Future editions are expected to incorporate guidance for expansion of SATCOM voice capability in the high-density continental areas.

2.3	The following principles were adhered to in the development of this guidance material:

- build on the ICAO required communication performance (RCP) framework to provide States with flexibility to apply different standards for different uses, without implication to seamless operations, interoperability and safety;

- provide a basis for States in determining acceptability of any implementation within an ATS facility, a radio facility or aircraft equipage, taking into account routine and emergency use, the provision of ATS using SATCOM voice communications, procedures for the radio operator, controller and flight crew, performance specifications and qualification;

- note that in-flight use of portable SATCOM phones for ATS communications is not allowed, according to many existing State operating regulations;

- do not specifically address MEL matters, but serve to facilitate State authorities or Regional Safety Oversight Organizations (RSOOs) in establishing policies in such matters; and

- do not address the use of SATCOM voice in isolation (i.e., HF voice capability remains available on board and in the ground infrastructure).

2.4	While directed primarily at air traffic services personnel and flight crews, the following personnel should be familiar with various aspects of its contents: regulators, airspace planners, aircraft operators, dispatchers, communication service providers and radio operators, training organizations, central monitoring and reporting agencies, automation specialists at centers and radio facilities, and aircraft manufacturers and equipment suppliers.

2.5	The guidance material will support the following activities:

- the States’ roles and responsibilities in relation to the following:

  - safety regulatory oversight of air navigation services;

  - operational eligibility, flight crew training and qualification;

  - airworthiness certification of aircraft SATCOM voice systems.

- the development of agreements and/or contractual arrangements between air traffic service providers and aircraft operators and their respective communication service providers;

- development of operational procedures; and

- operational monitoring, analysis, and exchange of operational data among regions, States, RSOOs and communication service providers.

2.6	Guidance material and information concerning SATCOM data communications is not within the scope of this guidance material and can be found in the Global Operational Data Link Document (GOLD).

3.	Status

3.1	This guidance document may contain material that may eventually become Standards and Recommended Practices (SARPs), or PANS provisions when it has reached the maturity and stability necessary for adoption or approval.  It may also comprise material prepared as an amplification of the basic provisions in the corresponding SARPs to assist the user in the application of the SARPs and PANS.

4. Implementation

4.1	The implementation of procedures is the responsibility of Contracting States; they are applied in actual operations only after, and in so far as, States have enforced them. However, with a view to facilitating their processing towards implementation by States, this guidance material has been prepared in language which will permit direct use by air traffic services personnel and others associated with the provision of air traffic services to international air navigation.

5.	Promulgation of information

5.1	Information relating to the establishment and withdrawal of and changes to facilities, services and procedures affecting aircraft operations should be notified and take effect in accordance with Annex 15 — Aeronautical Information Services.

6.	References

6.1	The following references are cited in this document:

- ICAO Annex 1 — Personnel Licensing;

- ICAO Annex 2 — Rules of the Air;

- ICAO Annex 4 — Aeronautical Charts;

- ICAO Annex 6 — Operation of Aircraft – Part I — International Commercial Air Transport — Aeroplanes;

- ICAO Annex 10 — Aeronautical Telecommunications – Volume II — Communication Procedures including those with PANS status;

- ICAO Annex 10 — Aeronautical Telecommunications – Volume III — Communication Systems;

- ICAO Annex 11 — Air Traffic Services;

- ICAO Annex 15 — Aeronautical Information Services;

- Procedures for Air Navigation Services — Air Traffic Management (PANS-ATM, ICAO Doc 4444);

- Regional Supplementary Procedures (Regional SUPPs, ICAO Doc 7030);

- Procedures for Air Navigation Services — ICAO Abbreviations and Codes (PANS-ABC, ICAO Doc 8400);

- Designators for Aircraft Operating Agencies, Aeronautical Authorities and Services (ICAO Doc 8585);

- Aircraft Type Designators (ICAO Doc 8643);

- Manual of Radiotelephony (ICAO Doc 9432);

- Manual on Airspace Planning Methodology for the Determination of Separation Minima (ICAO Doc 9689);

- Performance-based Navigation Manual (PBN) (ICAO Doc 9613);

- Manual on Required Communication Performance (RCP) (ICAO Doc 9869);

- Manual on the Aeronautical Mobile Satellite (Route) Service (ICAO Doc 9925).

7. 	Changes to the document

This document is maintained as a regional document in coordination with ICAO planning and implementation regional groups (PIRGs) providing SATCOM voice services within their region.  Each participating PIRG establishes a mechanism for submitting and administering change proposals.

Change proposals (CPs) can be submitted by any stakeholder participating in SATCOM voice operations.  The stakeholder should submit a Change Proposal to their ICAO regional office.  The ICAO regional office will coordinate the change proposal within its own region, other regions, and ICAO HQ, to determine the acceptability of the change proposal.  Once the ICAO regional office has completed coordination and the participating PIRGs accept the change proposal, the change is concluded by each of the PIRGs.

8.	Amendments to the SVGM

| Amendment | Source(s) | Subject(s) | Approved applicable |
| --- | --- | --- | --- |
| 1st Edition (2012) | Asia/Pacific Air Navigation Planning and Implementation Regional Group (APANPIRG/23 – 2012) North Atlantic Systems Planning Group (NAT SPG/48 – 2012) | Satellite Voice Guidance Material (SVGM) | Applicable within participating Regions on 1 October 2012. |

# Definitions

- To be administratively updated just prior to completion.  Terms generally will only be included if used in the document.

When the following terms are used in the present document they have the following meanings.  Where the term has “(ICAO)” annotated, the term has already been defined as such in SARPs and/or PANS.

Term

ACC. The symbol used to designate area control centre. (ICAO)

ACP. The symbol used to designate actual communication performance.

ACTP. The symbol used to designate actual communication technical performance.

Aeronautical fixed telecommunication network (AFTN). A worldwide system of aeronautical fixed circuits provided, as part of the aeronautical fixed service, for the exchange of messages and/or digital data between aeronautical fixed stations having the same or compatible communications characteristics. (ICAO)

Aeronautical Information Publication (AIP). A publication issued by or with the authority of a State and containing aeronautical information of a lasting character essential to air navigation. (ICAO)

Aeronautical mobile satellite (route) service (AMS(R)S). Includes both voice and data, The use of AMS(R)S for voice communications is commonly referred to as SATCOM voice. This convention is maintained throughout this Document.

Aeronautical mobile service (RR S1.32). A mobile service between aeronautical stations and aircraft stations, or between aircraft stations, in which survival craft stations may participate; emergency position-indicating radio beacon stations may also participate in this service on designated distress and emergency frequencies. (ICAO)

Aeronautical operational control (AOC). Communication required for the exercise of authority over the initiation, continuation, diversion or termination of flight for safety, regularity and efficiency reasons. (ICAO)

Aeronautical station (RR S1.81). A land station in the aeronautical mobile service. In certain instances, an aeronautical station may be located, for example, on board ship or on a platform at sea. (ICAO) Note.— Aeronautical station is commonly referred to as a radio facility.

AFTN. The symbol used to designate aeronautical fixed telecommunication network. (ICAO)

AIDC. The symbol used to designate ATS interfacility data communications. (ICAO)

AIP. The symbol used to designate Aeronautical Information Publication. (ICAO)

Air traffic services provider (ATSP). An organization responsible for the provision of air traffic services.

Air traffic control (ATC) service. A service provided for the purpose of: - preventing collisions: - between aircraft, and - on the manoeuvring area between aircraft and obstructions; and - expediting and maintaining an orderly flow of air traffic. (ICAO)

Air traffic management (ATM). The dynamic, integrated management of air traffic and airspace including air traffic services, airspace management and air traffic flow management — safely, economically and efficiently — through the provision of facilities and seamless services in collaboration with all parties and involving airborne and ground-based functions. (ICAO)

Air traffic service (ATS). A generic term meaning variously, flight information service, alerting service, air traffic advisory service, air traffic control service (area control service, approach control service or aerodrome control service). (ICAO)

Air traffic services unit (ATSU). A generic term meaning variously, air traffic control unit, flight information centre or air traffic services reporting office. (ICAO)

Aircraft active flight plan. (See flight plan).

Aircraft address. A unique combination of 24 bits available for assignment to an aircraft for the purpose of air-ground communications, navigation and surveillance. (ICAO)

Aircraft identification. A group of letters, figures or a combination thereof which is either identical to, or the coded equivalent of, the aircraft call sign to be used in air-ground communications, and which is used to identify the aircraft in ground-ground air traffic services communications. (ICAO) Note 1.— The aircraft identification does not exceed 7 characters and is either the aircraft registration or the ICAO designator for the aircraft operating agency followed by the flight identification. Note 2. ICAO designators for aircraft operating agencies are contained in ICAO Doc 8585.

Aircraft registration. A group of letters, figures or a combination thereof which is assigned by the State of Registry to identify the aircraft. Note. Also referred to as registration marking.

Aircraft system availability (AAIR). The required probability of available capability on an aircraft. Note.— The actual aircraft system availability is computed assuming that the service is available in the relevant airspace.

AIREP. The symbol used to designate an air-report. (ICAO)

Air-report. A report from an aircraft in flight prepared in conformity with requirements for position, and operational and/or meteorological reporting. (ICAO)

AMS(R)S. The symbol used to designate aeronautical mobile satellite (route) service. (ICAO)

AOC. The symbol used to designate aeronautical operational control. (ICAO)

Appropriate ATS authority. The relevant authority designated by the State responsible for providing air traffic services in the airspace concerned. (ICAO)

Appropriate authority. - Regarding flight over the high seas: The relevant authority of the State of Registry. - Regarding flight other than over the high seas: The relevant authority of the State having sovereignty over the territory being overflown. (ICAO)

Area control centre (ACC). A unit established to provide air traffic control service to controlled flights in control areas under its jurisdiction. (ICAO)

ATC. The symbol used to designate air traffic control. (ICAO)

ATC waypoint. A waypoint contained in Item 15 of the ICAO flight plan, or as amended by ATC. Note.— A waypoint inserted by the flight crew for purposes of conducting flight operations such as points of no return are not ATC waypoints.

ATM. The symbol used to designate air traffic management. (ICAO)

ATS interfacility data communication (AIDC). Automated data exchange between air traffic services units, particularly in regard to co-ordination and transfer of flights. (ICAO)

ATSP. The symbol used to designate air traffic service provider.

ATS. The symbol used to designate air traffic service. (ICAO)

ATSU. The symbol used to designate ATS unit. (ICAO, sort of)

C for RCTP. The proportion of intervention messages and responses that can be delivered within the specified RCTP time for intervention. Note.— For voice communications, continuity would take into consideration any dropped calls.

C for RCTPAS. The proportion of intervention messages and responses that can be delivered within the specified RCTPAS time for intervention.

C for RCTPAS/AIR. The proportion of intervention messages and responses that can be delivered within the specified RCTPAS/AIR time for intervention. Note.— For voice communications, continuity would take into consideration any dropped calls.

C for RCTPATSU. The proportion of intervention messages and responses that can be delivered within the specified RCTPATSU time for intervention.

C for RCTPCSP. The proportion of intervention messages and responses that can be delivered within the specified RCTPCSP time for intervention.

C for operational performance. The proportion of intervention messages and responses that can be delivered within the specified operational performance time for intervention. Note.— For voice communications, continuity would take into consideration any dropped calls.

Call performance. The operational portion of the transaction time to prepare the operational response, and includes the recognition of the instruction, and message composition during an interactive call between the Radio Operator and Flight Crew, e.g. flight crew/HMI for intervention transactions. Note.— For voice communications, the call begins when the aircraft indicates an incoming call to the flight crew and ends when the callers operationally completes the call. The call includes any tasks for the radio operator to send the message to the ATSU.

Call sign. The designator used in air-ground communications to identify the aircraft and is equivalent to the encoded aircraft identification.

Caller line identification. A display of the identification of a caller to the recipient prior to answering the call. Note.— For the purposes of ATS communications, caller line identification to the flight crew is a display of facility name or the facility designator for the aeronautical station or ATS unit. For the controller it is a display of the aircraft identification.

CLI. The symbol used to designate caller line identification.

CNS. The symbol used to designate communications, navigation and surveillance. (ICAO)

CNS/ATM. The symbol used to designate communications, navigation and surveillance/air traffic management. (ICAO)

COM. The symbol used to designate communications, (ICAO)

Communication service provider (CSP). Any public or private entity providing communication services for general air traffic.

Communication services. Aeronautical fixed and mobile services to enable ground-to-ground and/or air-to-ground communications for safety and regularity of flight.

Compulsory reporting point. An ATC waypoint for which a position report is required by the aircraft.

Control area (CTA). A controlled airspace extending upwards from a specified limit above the earth. (ICAO)

CSP. The symbol used to designate communication service provider.

CTA. The symbol used to designate control area. (ICAO)

Current flight plan. (See flight plan).

EMERG. The symbol used to designate emergency. (ICAO)

ETD. The symbol used to designate estimated time of departure or estimating departure. (ICAO)

FANS 1/A. The symbol used to designate the initial future air navigation system, as defined by RTCA DO-258A/EUROCAE ED-100A, or previous standards that defined the FANS 1/A capability. Note.— FANS 1/A generally means that the data link system on an aircraft, the ATSU ground system, and communication service provision comply with the standard. In certain cases, specific reference is made to a particular type of FANS 1/A aircraft as follows: a) FANS 1/A+ means that the aircraft completely complies with Revision A of the standard, which includes message latency timer; and b) FANS 1/A ADS-C means that the aircraft complies with AFN and ADS-C applications, but does not include the CPDLC application.

FANS. The symbol used to designate future air navigation system.

FDPS. The symbol used to designate flight data processing system. (ICAO)

FIR. The symbol used to designate flight information region. (ICAO)

Filed flight plan. (See flight plan).

Flight identification. A group of numbers, which is usually associated with an ICAO designator for an aircraft operating agency, to identify the aircraft in Item 7 of the flight plan.

Flight information region (FIR). An airspace of defined dimensions within which flight information service and alerting service are provided. (ICAO)

Flight level (FL). A surface of constant atmospheric pressure which is related to a specific pressure datum, 1 013.2 hectopascals (hPa), and is separated from other such surfaces by specific pressure intervals. (ICAO) Note 1.— A pressure type altimeter calibrated in accordance with the Standard Atmosphere: a) when set to a QNH altimeter setting, will indicate altitude; b) when set to QFE altimeter setting, will indicate height above the QFE reference datum; c) when set to a pressure of 1 013.2 hPa, may be used to indicate flight levels. Note 2.— The terms “height” and “altitude”, used in Note 1 above, indicate altimetric rather than geometric heights and altitudes.

Flight plan. Specified information provided to air traffic services units, relative to an intended flight or portion of a flight of an aircraft. (ICAO) A flight plan can take several forms, such as: Current flight plan (CPL). The flight plan, including changes, if any, brought about by subsequent clearances. (ICAO) Note 1.— When the word “message” is used as a suffix to this term, it denotes the content and format of the current flight plan data sent from one unit to another. Filed flight plan (FPL). The flight plan as filed with an ATS unit by the pilot or a designated representative, without any subsequent changes. (ICAO) Note 2.— When the word “message” is used as a suffix to this term, it denotes the content and format of the filed flight plan data as transmitted. Aircraft active flight plan. The flight plan used by the flight crew. The sequence of legs and associated constraints that define the expected 3D or 4D trajectory of the aircraft from takeoff to landing. (RTCA/EUROCAE)

GEO. The symbol used to designate geosynchronous earth orbit.

HF. The symbol used to designate high frequency (3-30 Mhz). (ICAO)

ICD. The symbol used to designate interface control document.

LEO. The symbol used to designate low earth orbit.

Long-range communication system (LRCS). A system that uses satellite relay, data link, high frequency, or another approved communication system which extends beyond line of sight.

LRCS. A symbol used to designate long-range communication system.

Master minimum equipment list (MMEL). A master list appropriate to an aircraft type which determines those instruments, items of equipment or functions installed on board that, while maintaining the intended level of safety, may temporarily be inoperative at commencement of the flight.

Maximum accumulated unplanned outage time (min/yr). A criterion applied to a given operational airspace of FIR that defines the maximum time allowed for the total sum of the unplanned outages that exceed the unplanned outage duration limit in any twelve month period Note.— The criterion does not apply to unplanned outages that are less than the unplanned outage duration limit or planned outages. Unplanned outages that are less than the unplanned outage duration limit are considered against the criterion for continuity.

Maximum number of unplanned outages (per year). A criterion applied to a given operational airspace or FIR that defines the maximum number allowed for unplanned outages in any twelve month period.

MEL. The symbol used to designate minimum equipment list. (ICAO)

MEO. The symbol used to designate medium earth orbit.

MET. The symbol used to designate meteorological or meteorology. (ICAO)

Minimum equipment list (MEL). A list which provides for the operation of aircraft, subject to specified conditions, with particular equipment inoperative, prepared by an operator in conformity with, or more restrictive than, the MMEL established for the aircraft type. (ICAO)

Monitored performance. The monitored combination of the time it takes to organize, prepare, dial and perform an interactive call plus the technical performance of the ground and air equipment. Note.— For voice communications, monitored performance comprises RCTPAS/AIR, queue/connect performance and call performance.

NOTAM. A notice distributed by means of telecommunication containing information concerning the establishment, condition or change in any aeronautical facility, service, procedure or hazard, the timely knowledge of which is essential to personnel concerned with flight operations. (ICAO)

Operational communication transaction. The process a human uses to initiate the transmission of an instruction, clearance, flight information, and/or request, and is completed when that human is confident that the transaction is complete. (GOLD)

Operational performance. The portion of the transaction time (used for intervention) that does not include the times for message composition or recognition of the operational response. Note.— For voice communications, operational performance comprises RCTP, queue/connect performance and call performance.

PANS-ATM. The symbol used to designate Procedures for Air Navigation Services — Air Traffic Management (ICAO Doc 4444). (ICAO)

PIN. The symbol used to designate personal identification number.

Personal identification number. A secret numeric password shared between a user and a system that can be used to authenticate the user to the system. Note.— For the purposes of ATS communications, all PIN numbers are issued for the same purpose, as there is no PIN that grants higher priority or access than another. The priority of the call is determined by the dialing string and GtA calling service used. Calling Line Identification (caller ID) is just a substitute for the operator not having to dial the PIN number for GtA calls. When CLI is implemented for the customer, then all calls made from the phone numbers provided to the GES provider will not be prompted for a PIN when the call is placed to the aircraft. If the switch does not recognize the pre-defined CLI list provided to the GES, then the caller will be prompted for the PIN code.

Preemption. A higher priority call will interrupt communication resources being used by a lower-priority communication to establish a connection without any indication or delay. Note.— If the intervening call is the same or lower, the current call will not be preempted and the intervening caller will get an indication that the line is not available. The effects of preemption can be minimized by multiple channels and conference calling, but not completely eliminated.

Priority level. An indication of call precedence for ground-to-air or air-to-ground calls. Priority level may be used to establish preemption.

Queue/connect performance. The operational portion of the transaction time to organize and place the call either via a manual or automated dialing sequence depending on equipment at the RO facility. Note.— For voice communications, queue/connect performance begins when the message from the ATSU via the network is sent to the queue and ends when the last digit of the dialing sequence is finished.

Radio facility. A term commonly used to refer to an aeronautical station.

Required communication performance (RCP). A statement of the performance requirements for operational communication in support of specific ATM functions.

Required communication technical performance (RCTP). The portion of the (intervention) transaction time that does not include the human times for message composition, operational response, and recognition of the operational response. Note.— For voice communications, RCTP comprises RTCPATSU, RCTPAS, RCTPAS/AIR and RCTPCSP.

RCP. The symbol used to designate required communication performance.

RCP availability (A). The required probability that an operational communication transaction can be initiated when needed. Note.— For voice communications, this translates to any failure prohibiting the call to be initiated to include congestion (much like the analogy of a terrestrial mobile phone network). However this definition does not apply to a busy condition whereby the entity being called is already on the phone and does not have a way to put the existing call on hold or if able to, rejects the additional incoming call.

RCP continuity (C). The required probability that an operational communication transaction can be completed within the communication transaction time, either ET or TT 95%, given that the service was available at the start of the transaction. Note.— For voice communications, this translates into 5 out of 100 calls not being able to conclude their voice transactions within the allotted time or the call could be disconnected for any reason, including aircraft maneuvers, switching satellites or any loss of service while on the call.

RCP expiration time (ET). The maximum time for the completion of the operational communication transaction after which the initiator is required to revert to an alternative procedure.

RCP integrity (I). The required probability that an operational communication transaction is completed with no undetected errors. Note 1.— Whilst RCP integrity is defined in terms of the “goodness” of the communication capability, it is specified in terms of the likelihood of occurrence of malfunction on a per flight hour basis, e.g. 10-5, consistent with RNAV/RNP specifications. Note 2.— For voice communications, this translates to the ability to conduct the voice transaction with enough intelligibility to understand the verbal request. Additionally, this could also manifest in the crew executing an incorrect instruction because it was misunderstood. (It may not be clear at the time of this submittal, what type of error rate would reflect this condition, or if the read-back would mitigate or compound this).

RCP nominal time (TT 95%). The maximum nominal time within which 95% of operational communication transactions are required to be completed.

RCP specification. A specification (e.g. RCP 240) that provides the values assigned to RCP parameters for communication transaction time, continuity, availability and integrity, and allocations to ATSP, aircraft, CSP and operator.

RCTP. The symbol used to designate required communication technical performance.

RCTPAS. The summed critical transit times for an ATC intervention message and a response message allocated to the aeronautical station. Note.— For voice communications, RCTPAS includes two concurrent processes: a) the aircraft and aeronautical station technically disconnect the call; which is assumed. Operationally, the call is disconnected when the flight crew and radio operator complete the call; and b) the aeronautical station sends the response to the to the ATSU via the ground-ground network; the performance is denoted by RCTPAS.

RCTPAS/AIR. The technical time for the air-ground network and associated components to initiate a call. Note.— For voice communications, RCTPAS/AIR begins when the last digit of the dialing sequence is finished and ends when the aircraft indicates an incoming call to the flight crew.

RCTPATSU. The summed critical transit times for an ATC intervention message and a response message, allocated to the ATSU system.

RCTPCSP. The summed critical transit times for an ATC intervention message and a response message, allocated to the CSP system.

Required communication performance (RCP). A statement of the performance requirements for operational communication in support of specific ATM functions. (ICAO)

Required communication technical performance (RCTP). The portion of the (intervention) transaction time that does not include the human times for message composition, operational response, and recognition of the operational response.

Required navigation performance (RNP). A statement of the navigation performance necessary for operation within a defined airspace. (ICAO) Note.— Navigation performance and requirements are defined for a particular RNP type and/or application.

Responder performance criteria. The operational portion of the transaction time to prepare the operational response, and includes the recognition of the instruction, and message composition, e.g. flight crew/HMI for intervention transactions.

RGS. The symbol used to designate radio ground station.

RNAV. The symbol used to designate area navigation.

RNP. The symbol used to designate required navigation performance.

RTF. The symbol used to designate radiotelephone (ICAO).

SARPs. The symbol used to designate Standards and Recommended Practices. (ICAO).

Satellite communication service provider. Typically provides the inter-working unit of the terrestrial sub-system which connects the satellite ground earth station, or Gateway, and the terrestrial network in support of AMS(R)S. Commonly referred to as SSP (see SSP definition).

Satellite network operations provider. Typically provides the satellite sub-system which includes the satellite(s) and may or may not include the ground earth stations or Gateway. Commonly referred to as SSP (see SSP definition)

Satellite service provider (SSP). An entity or group of entities that provide, via satellite, aeronautical fixed services and/or aeronautical mobile services at least from the signal in space to/from aircraft, to the attachment point of the ground earth station (GES) to the ground communication services network.

SATCOM. The symbol used to designate satellite communication. (ICAO)

SELCAL. The symbol used to designate selective calling system. (ICAO)

Service availability (ACSP). The required probability that the communication service is available to all users in a specific airspace when desired.

SSP. The symbol used to designate satellite service provider.

Terrestrial Network Service Provider. Typically provides the aviation centric terrestrial sub-system which provides connectivity to the end-users, such as ATS providers, airlines and flight departments. Commonly referred to as CSP ( see CSP Definition).

Unplanned outage duration (minutes). The time from when an unplanned outage begins to when the ATSU receives notification that the service has been restored.

Unplanned outage duration limit (minutes). A criterion applied to a given operational airspace or FIR that defines the maximum time for the duration of an unplanned outage at which time there is an operational impact.

Unplanned outage notification delay (min). Notification to the ATSU of an unplanned outage. Measured from when the unplanned outage begins to when the ATSU receives notification.

VHF. The symbol used to designate very high frequency (30-300 Mhz). (ICAO)

# Overview of satellite voice communications

## General

    - The guidance material provided in this document is intended for use of SATCOM voice equipment to provide additional ATS communications capability, in accordance with airspace requirements in respective AIP or regional SUPPS and under the following conditions:

- the aircraft equipment is approved by the State of the Operator or the State of Registry;

- the flight crew communicates with the appropriate aeronautical station or ATS unit depending on the type of communication and as allowed by airspace requirements; and

- the flight crew operates SELCAL or maintains a listening watch on the assigned HF frequency.

    - SATCOM voice communication initiated due to HF propagation difficulties does not constitute urgency. Dedicated SATCOM voice telephone numbers (short codes) for air-ground radio facilities and air traffic control facilities are published in national AIPs where approved.

    - SATCOM voice is not a replacement for ADS-C, CPDLC or HF voice communications, but rather a means of reducing the risk of communications failure, improving the safety of operations and alleviating HF congestion. SATCOM voice provides an additional discrete communications medium and potential MEL relief as States, approving reduced carriage requirements for HF radio, may allow aircraft to operate with only one serviceable HF radio.

    - It is further noted that while States may allow MEL relief approving reduced carriage requirements for HF radio, airspace requirements will take precedence over any relief.  This guidance material may be used to facilitate alignment of airspace requirements with State MEL policies.

## Aeronautical Satellite Communication Systems Overview

    - Satellite communication systems are defined by three different altitude orbits; low earth orbit (LEO), medium earth orbit (MEO) and geosynchronous earth orbit (GEO).  The altitude of the orbit determines the area illuminated by the satellite.  The higher the orbit the weaker the signal is from the satellite, but it has a much larger footprint. Propagation loss is overcome by increased complexity of the antenna systems along with higher transmitter power.  Conversely, a LEO satellite’s footprint is much smaller requiring a higher number of satellites to provide coverage, but the antennas used are much simpler along with reduced RF power requirement on the subscriber end. Also, the lifetime of a LEO satellite is less due to drag caused by the close proximity of earth.

    - Today there are three satellite systems servicing the aeronautical market. Inmarsat and Japan operate GEO satellite systems, and Iridium operates a LEO satellite system. These satellite systems use AMS(R)S L-band frequencies reserved for aeronautical safety services.

    - Figure 2-1 provides an overview of the aeronautical satellite communication system and shows the various components that make up the complete system, including the ATSP(s), ATS unit(s), aeronautical station(s), CSP(s), SSP(s), PSTN, aircraft and the operator(s).

Figure 2-1.	Aeronautical satellite communication system overview

.

## Iridium

    - Iridium uses a constellation of 66 satellites at an altitude of 780 km (450 miles) in six orbital planes, with eleven satellites in each orbital plane, providing global coverage.  Additionally there are a number of spare satellites to replace any in-orbit failures.  At that altitude each satellite covers a circular area of 4700 km (2900 miles) and is in-view to a location on the ground for approximately 9 minutes.  Using a small amount of overlap in coverage between each satellite, the Iridium network hands-off the call to the next satellite coming into view to the ground location.  This is similar to a GSM cellular telephone system where the subscriber moves from one cell site to another except that the satellite is the moving vehicle.  The speed of the aircraft appears almost stationary compared to the speed the satellite moves.

    - Iridium uses a combination of both frequency and time multiplexing along with 48 spot beams developed by the phased array antenna from each satellite. The frequency band used by subscribers are within L-band AMS(R)S 1616 MHz to 1626.5 MHz block.  Iridium transmits and receives on this single block by multiplexing time slots to control the direction of the signal.  This L-band connection from the satellite to the mobile subscriber is cross-linked via inter-satellite connectivity using Ka-band frequencies. Each satellite connects to the satellite in front, behind and to each side on Ka-band. This inter-satellite Ka-band connection is routed within the satellite constellation to the ground station gateway in Tempe Arizona.  Connectivity in Tempe is also on the Ka-band.  Iridium’s reliability is affected by rain fade on Ka-band to the Tempe gateway.  A second ground earth station is being considered.

    - Iridium satellites current system was launched in the late 1990’s and became operational in early 2000.  The original life expectancy of Iridium’s satellites was calculated to be seven plus years.  Iridium is planning to design, build and launch their next generation satellite system call “Iridium Next” by the end of 2017.  A recent study of their current system indicates that the original satellite system is expected to last through 2017.

## Inmarsat

    - The Inmarsat network of satellites is in geostationary orbit directly above the earth equator at an altitude of 35,786 km (22,236 miles).  At that altitude above earth, each satellite’s spot beam covers approximately 120 degrees of the earth at the equator and to approximately 82 degrees North and 82 degrees South latitude.  The orbital period of each satellite is exactly the same as the rotation period of earth so each satellite appears to remain in the same position.  Inmarsat is currently replacing their older generation I-3 satellites with new technology I-4 satellites providing advanced services. That project is nearly complete. The end of life for Inmarsat I-3’s is 2018.

    - There are three new I-4 and four older I-3 satellites providing aviation services, including PSTN voice.  L-band frequencies allocated for aviation AMS(R)S are split between a transmit block and a receive block with a guard band in between..  This allows the subscriber unit using a frequency duplexer to receive and transmit simultaneously.   Inmarsat’s primary transmit frequency allocation is adjacent to Iridium’s allocation used for both transmit and receive; this can cause interference to the secondary Iridium receive allocation when the aircraft operator desires both satellite services.

    - Swift Broadband users of Inmarsat I-4 satellites have available Voice over Internet Protocol (VoIP) capability.  The ground user network interface doesn’t exist as yet but is expected to evolve in time.  Iridium’s Next satellite system has indicated the possible availability of these VoIP services.

## Japan Multi-Function Transport Satellite (MTSAT)

    - MTSAT is a GEO satellite system. The specification is equivalent to that of I-3 except for the footprint which is limited to the Asia and the Pacific Ocean. MTSAT is interoperable with I-3 so that the subscriber unit can seamlessly carry out the handover between MTSAT and I-3 (and legacy communication module of I-4).

    - Japan expects end of life of MTSAT-1R to occur during 2014.  In 2010, Japan conducted a comprehensive study for the next generation satellite system.  Based on the results, Japan decided not to replace MTSAT-1R, but to continue to provide AMSS through MTSAT-2 after the termination of MTSAT-1R AMS(R)S payload. The calculation of remaining fuel showed that MTSAT-2 has an outlook of four year expansion of its end of life from 2015. Japan believes that MTSAT system by single satellite will still meet the requirements of communication service mainly due to the high redundancy of ground system.

## Public Switched Telephone Networks and Dialing Systems Used

    - Current offerings by Inmarsat, MTSAT and Iridium for voice communication services use the Public Switched Telephone Network (PSTN) or leased telephone lines from the appropriate gateway to the ground party.  The telephone number country codes for both satellite systems are provided by the ITU and are designated as Global Mobile Satellite Systems (GMSS).  Inmarsat uses a Single Network Access Code (SNAC) and switches the call automatically to the correct Inmarsat earth station where the aircraft is logged into.  Iridium employs telephone number ITU sub-blocks and optionally a US-based telephone number.

# Administrative provisions related to satellite voice operations

## ATSP and aeronautical station service provision

    - When providing SATCOM voice services, ATSPs and aeronautical stations should provide these services consistent with voice communication procedures, in accordance with Doc 4444, Doc 9432 and Volume II of Annex 10.

### ATSP system validation and operational readiness

#### The ATSP should ensure a validation process that confirms their equipment and procedures and/or its aeronautical stations are reliable and adequate for the intended use.  This process should include:

- A system safety assessment which demonstrates that the service provision meets the safety objectives.  The ATSP should conduct a system safety assessment through a functional hazard analysis or a documented system safety case for initial implementation as well as for future enhancements.

- Integration test results confirming interoperability for operational use of the aircraft and ground systems; and

- Confirmation that the ATS operation manuals are compatible with those of adjacent providers.

#### Following the safety assessment, the ATSP should institute measures including its aeronautical stations, CSPs and SSPs, to ensure acceptable mitigation of the identified failure conditions.

#### The ATSP should ensure that it provides communication services that meet the performance specifications provided at Appendix A, and that its aeronautical stations, the CSP and SSP meet their performance allocations under expected capacity and loading conditions.

#### The ATSP should ensure appropriate procedures or other means either by the ATSU/controller or its aeronautical stations/radio operators to:

- Maintain access numbers so controllers and/or radio operators can contact flights with SATCOM voice capability;

- Ensure appropriate priority level when initiating calls, in accordance with Table 3-1; and controller/radio operator procedures contained in Chapter 4.

- Respond to SATCOM voice calls, where SATCOM voice services are provided;

- Notify adjacent ATSUs of system failures, software upgrades (or downgrades) or other changes, which may impact them.  Such notification procedures will normally be detailed in letters of agreement between adjacent units; and

- Notify airspace users of SATCOM voice service outages, performance degradation, and restoration.

#### The ATSP should ensure that the controllers and radio operators receive appropriate training in accordance with ICAO Annex 1 and obtain any necessary approval from the State.

#### The ATSP should ensure that the SATCOM voice service provision meets applicable security requirements, considering its ATSUs/controllers and/or its aeronautical stations/radio operators.

#### The ATSP should ensure that its ATSUs, aeronautical stations and CSP(s) retain records for at least 30 days to allow for accident/incident investigation purposes.  The ATSP and CSPs should make these records available for air safety investigative purposes on demand.  These recordings should allow replaying of the situation and identifying the SATCOM voice communications between the radio operator/controller and the flight crew.

### Aeronautical information, notifications, and interfacility agreements

#### The ATSP should notify operators of SATCOM voice services using the AIP or NOTAM, which includes:

- Procedures for publishing contact information, that is associated with current airspace boundaries, e.g. specific SATCOM voice numbers for applicable facilities; and

- Requirements for use, e.g., criteria for when to contact the ATS unit or aeronautical station; and

- Flight plan form and submission requirements.

### Considerations for the aeronautical station

#### To provide SATCOM voice radio services at a full operational level, the radio facility should be able to accept or place a SATCOM voice call given the necessary infrastructure to handle the expected SATCOM voice traffic demand and in accordance with prescribed performance specifications.  Additionally, radio facility providers must ensure that adequate resources are available in the event that SATCOM voice services are disrupted.

#### When supporting satellite voice communications, radio facilities should establish procedures that:

- Enable operators to register SATCOM voice capabilities and means to contact the aircraft.

- Ensure ATSPs and radio operators receive the relevant information needed to establish two-way communications with the aircraft.

- Maintain, on a 28-day update cycle, INMARSAT’s and Iridium’s master aircraft phone number list as new SATCOM radio facilities become operational.

- Ensure users are notified when service conditions change, i.e., service outages, degraded performance, restoration of service.

- Establish procedures with operators to verify correct SATCOM voice number in the event of a SATCOM voice number change to a specific airframe with less than 24 hour notice.

#### When supporting satellite voice communications, radio facilities should provide automation support that allow the radio operator to provide SATCOM voice services for the intended operations in accordance with performance specifications provided in Appendix A.  The automation support should include capabilities that allow the radio operator to:

- Place and receive SATCOM voice calls to either the INMARSAT or Iridium access points.

- Obtain the correct authorization Personal Identification Number (PIN) for both INMARSAT and Iridium networks to place authorized ATC calls.

- Answer and route incoming SATCOM voice calls to the appropriate SATCOM/radio operator.

- Auto-dial capability to enable faster call setup times.

- To exchange air traffic control information with ATC.

- Provide for the ability to prioritize, preempt and establish precedence on outgoing calls in accordance with Table 3-1.

### Considerations for the communication service provider (CSP)

      - Requirements for CSP

##### The CSP should ensure that the SATCOM voice service meets the performance criteria as prescribed by the aeronautical station or ATSP in accordance with RCP specifications provided in Appendix A.

##### The CSP should provide updated aircraft SATCOM telephone lists to aeronautical stations and ATSP.

##### The CSP should provide notification of SATCOM voice service outages, performance degradation, and restoration to aeronautical stations, ATS units, and aircraft operators that use the service.

##### For those situations when a CSP cannot continue to provide SATCOM voice communications, it should inform the involved ATSPs and operators and/or, if appropriate, the providers of Aeronautical Information Services, in accordance with coordination procedures established in writing.

##### The CSP should be under safety oversight by an air traffic service provider (ATSP).

##### In the case of provision of additional SATCOM services, e.g. as those which may be required by NextGen or SESAR, for which a safety objective more severe than 10E-3 is postulated, an authorized service provider should demonstrate to the competent authority the compliance with the applicable requirements.

      - Contracted COM services

##### The principle of certification and subsequent continued surveillance (or “oversight”) of aviation operators is established in two Annexes to the Chicago Convention: Annex 6 Part I  for Commercial Air Transport operators and 14 for aerodrome operators.

##### Annex 11 (Air Traffic Services) does not necessarily require certification of the air traffic service provider (ATSP) but requires  States to establish a State Safety Programme (SSP) further described in Attachment D therein, which clarifies that States shall establish effective mechanisms to inspect and audit ATSPs in order to ensure effective safety oversight. In other words the requirement for States to oversee safety exists even in the absence of certification or approval of a certain type of aviation operators or service providers.

##### Chapter 2 (i.e. Administrative provisions for the international telecommunication service) of Volume II of Annex 10 also contains standards for the “supervision” of the COM service by a responsible authority designated by the State.  Note 2 to par. 2.27.4 of Annex 11 clarifies that, when COM services are directly provided by an ATSP, their oversight is through the SSP and the Safety Management System (SMS) established by the ATSP. Same Note clarifies that if COM services are contracted to an entity other than the ATSP, the SMS requirement still applies, but only to those services with direct operational implications (e.g. communications used for ATS purposes).

##### In any case therefore, an aviation organization (commercial air transport operator, aerodrome operator, ATSP) shall be, according to specific ICAO standards, under safety oversight by the competent aviation authority. Organizations under safety oversight should take responsibility for the safe, regular and efficient conduct of operations, including for the services provided by any contractors. For instance, this latter ICAO provision is transposed in the EU by so called “EU-OPS” in respect of commercial air transport operators and by so called “common requirements ” for ATSPs.

##### In this possible business model (e.g. an ATSP or air operator contracting COM services from a different organisation), depicted in Figure 3-2 below, the contracting organization does not only take responsibility to demonstrate to the competent authority that proper mechanisms exist to oversee the contracted CSP, but also inevitably attracts on itself some liability also for the contracted COM service.

Figure 3-2.	Radio operator contracted by organizations

### Considerations for the satellite service provider (SSP)

#### The following criteria apply to the satellite service provider (SSP):

- The ground earth station should be capable of preventing unauthorized calls to aircraft;

- The SATCOM voice system should provide Caller ID with PIN security information for display to the receiving party.

Note.— This guidance material supports both Iridium and INMARSAT communications.  INMARSAT systems typically support several levels of SATCOM voice communications.  Iridium systems are typically single channel or dual channel voice systems dedicated for flight deck communications.  Iridium based systems will require additional infrastructure in the Ground Earth Station to implement the necessary priority services needed for ATC voice communications.

#### The priority level column of Table 3-1 shows the order of precedence in setting up and receiving a satellite voice call.  Preemption is the immediate and automatic seizure of resources allocated to a lower-priority call.  Trade-offs of flight safety requirements versus passenger satisfaction should not be a consideration.

- Technological limits of PSTNs and AMS(R)S may require CSPs and SSPs to use indications of priority levels different than those indicated in figure 3-3.  When a SATCOM voice call is transmitted to and from an aircraft, the priority indicated to flight crews will conform with Annex 10, Volume II  and  the levels indicated in  Figure 3-3.

- Satellite voice calls should be prioritized consistent with figure 3-3.  If the equipment differentiates between levels the priority should be Level 1 / EMG, Level 2 / HGH, and Level 3 / LOW, and Level 4 PUB. The satellite voice equipment should configure the cockpit default priority to level 2 / HGH.   Level 4 / PUB calls should not be routed to the flight deck.

- The flight crew must have the capability to set the priority level for all calls.  The satellite voice equipment must provide the flight crew the means to preempt any call at any time.  The equipment must provide the means for automatic preemption of all cabin communications.

- If a satellite voice channel is in use and the ground earth station wants to send a higher-priority call, the satellite voice equipment should clear the lower-priority channel.  If all available channels are in use, the equipment should preempt the channel supporting the lowest priority channel in favor of the higher-priority call.

Table 3-1.	Priority levels for SATCOM voice calls

| Priority level | Application category | SATCOM voice call examples |
| --- | --- | --- |
| 1 / EMG Emergency (highest) Safety of Flight | Distress and Urgency | Rapid Descent, Urgent Sidestep for Weather |
| 2 / HGH Operational High (second highest) Safety of Flight | Flight Safety | Altitude Request |
| 3 / LOW Operational Low (third highest) Safety of Flight | Regularity of Flight, Meteorological, Administrative | Air Traffic Information Service, Redispatch, Maintenance |
| 4 / PUB Nonoperational (lowest) Nonsafety | Public Correspondence | Public Phone Calls |

## Operator eligibility

### Operational authorization to use satellite voice communications

#### Pilots holding an instrument rating (and where necessary a type rating) have the privilege to fly an aircraft under Instrument Flight Rules (IFR): e.g. en-route following a series of VOR (VHF Omni-Range) stations in the domain of navigation or using on-board radio equipment to liaise with Air Traffic Control in the domain of communication. Granting to pilots privileges linked to possess of a valid licence and proper ratings, is the normal way used by States to “authorize” aviation operations, without requiring additional administrative processes. However the meaning of “authorization”, although this word is widely used in Annex 6, is not defined.

#### Attachment E to Annex 6 Part I clarifies that an “approval” is an active response by the competent aviation authority established by State(s) to a matter submitted for its review, constituting a finding or determination of compliance with the applicable standards. An approval will be evidenced by the signature by the approving official, the issuance of a document or certificate, or some other formal action (e.g. a letter). Same Attachment explains that an “acceptance” does not necessarily require an active response by the authority to a matter submitted for its review. In other words this is a form of “silent approval”, unless the authority specifically rejects all or a portion of the matter under review, usually after some defined period of time after submission.

#### Hence in this guidance material the word “authorization” refers to a privilege granted by the applicable rules to persons holding the required licences or certificates and enjoying the related privileges (no additional administrative process). The word “approval” indicates an administrative procedure based on an application and an explicit written reply by the competent authority. The word “acceptance” means that a certified organisation (e.g. a commercial air operator) submits a document (e.g. a change to the operations manual) to the competent authority and, if the latter does not object within a given time, the change is in fact accepted.

#### Authorizations and approvals may be applied also to general aviation. On the contrary, since in most ICAO Contracting States an Air Operator Certificate (AOC) is not required for general aviation, the “acceptance” process does not apply to this segment of aviation.

#### From the point of view of air operators, the authorization (i.e. no additional administrative procedures) is normally preferred in comparison to the approval (i.e. written application followed by written reply by the authority).

#### The general principle of granting to properly rated pilots the authorization to use on board radio equipment, as part of their privileges, is however implicitly based on several underlying assumptions, such as:

- the aircraft, including its avionics, has an airworthiness approval covering the type of envisaged IFR operations (e.g. long range) and a radio licence;

- the complexity of using radio equipment, including SATCOM, does not present particular challenges;

- the concept and systems upon which the operation will be carried out are mature enough (= not “new”), which is the case of SATCOM voice;

- the risk associated with improper operation (including for third parties in the air or on the ground) is tolerable, which is the case for SATCOM voice since the transmission, if unclear can be repeated, and for which, a totally independent long rang communication system (i.e. HF) exists;

- availability and continuity of SATCOM voice is ensured, under responsibility of a Service Provider as explained in previous paragraph 3.1.4;

- appropriate standards for quality and management are established;

- accuracy and integrity of the address data base is ensured;

- appropriate training and checking standards and procedures for using SATCOM equipment exist and are implemented mainly for pilots; and

- provision of information (e.g. MMEL and training requirements) from holders of Type Certificates (TC) to air operators, throughout the life cycle of the aircraft is ensured.

#### Should one or more of the requirements listed above not be substantiated, then the competent authority at national or regional level, should assess whether rules and procedures for an explicit approval are necessary. Historically this has been the case in several instances in the navigation domain, but it has almost never been considered necessary in the communication domain.

### Radio equipment to be carried on board

#### Competent authorities also establish the minimum number of long range radio equipment to be carried on board. For instance, in the European Union (EU) the competent regional authority (i.e. EASA) has proposed that, at the level of legally binding rules (Opinion 04/2011 of 01 June 2011) for aircraft operators, aeroplanes shall be equipped with the radio communication equipment required by the applicable airspace requirements. Radio communication equipment shall include at least two independent radio communication systems necessary under normal operating conditions to communicate with an appropriate ground station from any point on the route, including diversions. This means that in principle one set of SATCOM and one set HF could be approved in regions where both services are available for routine communications.

#### The possible acceptance of one set of SATCOM and one set of HF on long range routes, is further clarified by proposed EASA Acceptable Means of Compliance (AMC4-CAT.IDE.A.345) expected to be promulgated by EASA in 2012 immediately after the adoption of the above mentioned rules by the European Commission and clarifying that:

- An HF - system is considered to be long range communication equipment;

- Other two-way communication systems may be used if allowed by the relevant airspace procedures; and

- Other” (e.g. SATCOM vocie) two-way communication systems may be used “if allowed by the relevant airspace procedures”. Therefore aircraft operators established in the European Union, may ask to have installed on board one set of HF and one SATCOM if, from applicable ICAO Regional Supplementary Procedures (Doc 7030) or published AIPs, it emerges that both services are available for routine use along the intended routes (including diversions).

#### The proposed EASA rules mentioned above, would hence allow national authorities in the EU Member States to accept, as normal requirement for minimum number and type of communication equipment installed on aircraft intended to be used on long range routes, one set of SATCOM voice and only one set of HF radio, providing that said services are available for routine communications.

#### In principle installed equipment has to be operational when commencing a flight. However experience has demonstrated that temporary unserviceability may be in some cases be tolerated. Several ICAO Contracting States hence require aircraft manufacturers to provide a Master MEL, The MMEL contains a list of which equipment can be tolerated as unserviceable at commencement of flight and for how long. The MMEL is approved by the authority designated by the State of Design (e.g. FAA in the USA and EASA in the EU).

#### Aircraft operators are mandated (e.g. by rule OPS 1.030 in the EU) to establish a Minimum Equipment List (MEL), based upon, but no less restrictive than the relevant MMEL. The MEL is approved by the competent authority established by the State of the Operator or State of Registry.

#### If changes to the Minimum Equipment List (MEL) are desired to allow dispatch with one satellite voice communication system and only one HF radio system, the operator should obtain operational approval or acceptance from the State of the Operator or State of Registry.

#### On 28 June 2011 EASA has published NPA 2011-11 aiming at establishing common EU specifications for MMEL (and replacing JAA TGL 26). Therein it is clarified that since not all ATC facilities are yet adequately equipped to handle SATCOM data or voice as the primary means of communication, the relief for dispatch with one HF and a backup SATCOM is restricted to 3 calendar days, to ensure that reliance on SATCOM is limited. In other words in areas requiring two operational Long Range Communication Systems at least one must be HF-voice.

### Criteria for aircraft operators

#### Aircraft operators should meet the following criteria:

- Distribute advisory information within the flight operations department to ensure that all personnel concerned are aware of SATCOM concepts and procedures associated with the use of SATCOM for routine ATC communications (i.e. phraseology).

- Assess operational requirements, establish policy and procedures, and incorporate them in appropriate documents, including:

  - Procedures to ensure that each participating aircraft is registered with their communication service provider;

  - Flight crew responsibilities for establishing and maintaining HF/VHF voice communications (SELCAL) with the appropriate OCA/FIR’s;

  - Procedures on use of SATCOM voice in accordance with ATC communication procedures and guidance material provided in Chapter 5, including its use in relationship to other means of communication on the specific aircraft, e.g., CPDLC/ADS-C, FMS WPR, Oceanic clearance;

  - Contact information for the aeronautical station/ATS unit;

  - Procedures when SATCOM voice fails;

  - Procedures for reporting problems associated with SATCOM voice to appropriate monitoring agencies.

### Aircraft equipage

#### The installations should be approved by the State of Registry or State of the Operator in accordance with FAA AC 20-150A (or equivalent), and verified to comply with the following:

- RTCA DO-210D, Minimum Operational Performance Standards for Geosynchronous Orbit Aeronautical Mobile Satellite Services (AMSS)

- RTCA DO-262A, Minimum Operational Performance Standards for Avionics Supporting Next Generation Satellite Systems (NGSS)

- ARINC 741, Aviation Satellite Communication System.

- ARNIC 761(Iridium & Inmarsat-2G)

- ARINC 781 (Inmarsat-3G)

Note.—  The above criteria apply only to the SATCOM voice installation when used for ATC communications.  The communication equipment requirements as prescribed for the airspace operated in or by the State of Registry or State of the Operator are beyond the scope of this document.

### Maintenance and in-service difficulties

#### The operator should establish procedures to report any problems its flight crews and dispatchers have with SATCOM voice operations in accordance with normal reporting practices.

#### The operator should establish procedures to ensure its flight crews and dispatchers are notified of significant degradation of SATCOM service, e.g., outage.

## Flight planning

    - The operator should ensure that the proper information is included in the ICAO flight plan. Until Amendment 1 to the PANS/ATM becomes valid in November 2012 the following procedures should be followed.

- Insert in item 10, Equipment, the letter “Z” to denote “other equipment”.

- Insert in item 18, Other information, the text “COM/” followed by the word SATVOICE, followed by further identification of the type of equipment such as INMARSAT or IRIDIUM.

Example:

(FPL-ACA101-IS
-B773/H-SHXWZ/SD
-EGLL1400
-N0450F310 L9 UL9 STU285036/M082F310 UL9 LIMRI
 52N020W 52N030W 50N040W 49N050W
-CYQX0455 CYYR
-EET/EISNN0026 EGGX0111 020W0136 CYQX0228 040W0330
 050W0415 REG/CFIUV SEL/FQHS COM/SATVOICE INMARSAT
 CODE/C0173E)

    - After Amendment 1 becomes valid, the appropriate indication(s) should be inserted in item 10, namely “M1” for an INMARSAT RTF capability, “M2” for an MTSAT RTF capability and/or “M3” for an Iridium RTF capability.

Example:

(FPL-ACA101-IS
-B773/H-SHXWM1M3/S
-EGLL1400
-N0450F310 L9 UL9 STU285036/M082F310 UL9 LIMRI
 52N020W 52N030W 50N040W 49N050W
-CYQX0455 CYYR
-EET/EISNN0026 EGGX0111 020W0136 CYQX0228 040W0330
 050W0415 REG/CFIUV SEL/FQHS CODE/C0173E)

    - When the necessary information for establishing contact with the aircraft can be derived from the ICAO 24-bit address, that information should be included in item 18 using the “CODE/” indicator as shown in the above examples.

# Controller and radio operator procedures

## General

    - The underlying SATCOM voice technology (duplex mode/open mic) lends itself to a conversational mode of communications.  Therefore, such use can create misunderstanding and confusion.  When using SATCOM voice, normal RTF conventions must be followed in accordance with standard ICAO phraseology, as defined in Annex 10, Volume II, Chapter 5, Doc 4444, Chapter 12, Doc 9432 and Doc. 8400.  Those communications procedures and examples are listed here to further clarify radio operator and controller procedures.

    - When establishing a SATCOM voice contact, the radio operator or controller should ensure positive identification of the aircraft. With each communication, the radio operator or controller will address the aircraft by its flight plan-filed Flight ID/Callsign for flight safety reasons. The aircraft, in turn, will re-state the caller’s identification, e.g., Gander Radio, as well as repeat the aircraft’s Flight ID.

Note.— Caller ID is not currently available in the aircraft avionics; however, manufacturers will consider this human factors element for ease of use in future design.

    - If unable to contact the aircraft via SATCOM voice for communication other than Level 1 / EMG, then reversion to any other means of communication medium should be followed, including HF, VHF, and data link, to establish positive communications with the flight.

## Controller procedures

- To be updated after the next version of Radio Operator procedures is finalized. We will either create a complementary procedure for Controllers or integrate it into the Radio Operator procedures.

## Radio operator procedures

### Outgoing SATVOICE calls – Radio operator initiated (ground-to-air)

#### Radio operators may initiate SATVOICE calls to aircraft in order to transmit ATC messages on behalf of the ATC controller. This traffic generally consists of clearances, requests, or advisories. Normally, this traffic will be sent at the priority reserved for ATC traffic, designated as “HGH”, “Level 2” or “Q12”. Given the high priority of ATC traffic, radio operators will use SATVOICE or the most expeditious means possible to deliver this traffic in order to meet ATC performance criteria. The conversations will also maintain a standard of radio telephony procedure in order to ensure accuracy and clarity.

#### The method of establishing ground initiated calls will be dependent on the technical/operational implementation at each one of the radio stations.  However, some steps should be common to each station regardless of the technical/operational methodology employed. These are:

- Identify the 8-digit ICAO code (octal) of the aircraft from radio operator database;

- Initiate the dialing sequence ensuring CLI/PIN and security measures to access the Ground Earth Station are in place;

- Use priority levels defined in Table 3 1, as available from avionics manufacturer and satellite provider;

- Wait for the flight crew to answer the call;

- Confirm the aircraft Flight ID/Callsign prior to delivering the clearance or message;

- Initiate the conversation; and

- Terminate the call after the dialog is finished.

Example:

| Radio operator | <Initiates call and line rings in flight deck> |
| --- | --- |
| Flight crew | Air France 465. |
| Radio operator | Air France 465, Gander Radio, <message> |
| Flight crew | Gander Radio, Air France 465, <read back message> |
| Radio operator | Air France 465, Gander Radio, readback correct, out |

#### If a radio facility or ATS unit recognizes that an aircraft is in imminent danger, an ATC message is urgent or delivery time is critical, the RO or controller should use the most expeditious means of communications.  If SATCOM voice is used as the first attempt, the RO or controller should call the aircraft at the highest priority Level 1 / EMG if possible, and state the threat or delivery the ATC message to the aircraft as part of the initial communication.  If unable to contact the aircraft via SATCOM, then the RO or controller should revert to any other means of communication, including HF, VHF, or data link, to establish positive communications for that flight and state the threat to the aircraft as part of the initial communication.

Example:

| Radio operator | <Initiates call and line rings in flight deck> |
| --- | --- |
| Flight crew | Air France 465. |
| Radio operator | Air France 465, Gander Radio, For collision avoidance , ATC clears <message> |
| Flight crew | Gander Radio, Air France 465, <read back message> |
| Radio operator | Air France 465, Gander Radio readback correct, out. |

#### Figure 4-3 provides a flow chart for SATCOM voice calls initiated by the radio operator to the flight crew.  Table 4-2 provides descriptions associated with each number flowchart item.

### Incoming SATVOICE calls – radio operator receives calls (air-to-ground)

#### Similarly, the radio operator may receive a SATVOICE call from the flight crew.  The radio operator should follow radio telephony practices in responding to the call. The radio operator serves to relay messages to the controller. Since the flight crew called the radio operator, the call will generally be ATC priority (HGH, Level 2, Q12), but it may be an emergency call (EMG, Level 1, Q15), depending upon flight status. The pilot may also ask the radio operator to facilitate a ground conference to ATC as well as their dispatch and/or maintenance departments.

#### For SATVOICE calls made to a radio station, the radio operator should:

- confirm the identification of the calling flight;

- acknowledge message; read back the message or selected contents, as required; and;

- if not already completed, provide primary and secondary HF frequencies and ensure flight establishes HF/VHF and SELCAL check, where required by the regulatory authority

Example:

|  | <line rings at aero radio> |
| --- | --- |
| Flight crew | <Initiates call and line rings at aero radio> |
| Radio operator | Flight calling Shanwick Radio |
| Flight crew | Shanwick Radio, Speedbird 255, <message> |
| Radio operator | Speedbird 255, Shanwick Radio <read back message> |
| Flight crew | Shanwick Radio, Speedbird 255, ROGER |
| Radio operator | Shanwick Radio OUT |

#### If the initial call from the flight crew to a radio station is made on SATCOM, the radio operator should:

- receive and read-back the message, if required; and

- allocate the primary and secondary HF frequencies and perform a SELCAL check on HF.

#### Figure 4-4 provides a flow chart for SATVOICE calls received by the radio operator from the flight crew.  Table 4-3 provides descriptions associated with each number flowchart item.

Figure 4-3.	Ground to air (radio operator to flight crew) SATVOICE flowchart

Table 4-2.	Ground to air (radio operator to flight crew) SATVOICE flow chart descriptions

| Ref | Ground to Air Communication Process | What is done |
| --- | --- | --- |
| 1 | ATC sends message to Radio Operator | ATSP/ATC (controller) inputs a message via ground network to radio operators within ATC agency or to their CSP. |
| 2 | Attempt contact through SATVOICE | Radio operator begins SATVOICE attempt. |
| 3 | Is SATVOICE number/octal code listed in RO database? | Decision Point – Is the SATVOICE number/octal code already associated with the Flight ID in the RO’s database and readily available for use? |
| 4 | Radio Operator locates octal code via tools and aircraft registration or SELCAL code. | If SATVOICE number is not readily available, other tools may be queried to cross reference Aircraft registration number and/or SELCAL code. |
| 5 | Initiate SATVOICE call to flight with appropriate priority level. | Once the SATVOICE number is available, the radio operator will initiate a call to the aircraft with the appropriate priority level, normally for ATC traffic this is Level 2, HGH, or Q12. |
| 6 | Is SATVOICE call successful? | Decision point – are the SATVOICE call attempts to the aircraft successful? |
| 7 | Yes: Pilot Answers SATVOICE call stating Flight ID. | This branch indicates that the SATVOICE call was successfully answered by the pilot, who identifies the aircraft by stating the Flight ID. |
| 8 | No: Try other means. Successful? | Decision Point – Radio operator tries another means of communication (such as HF, VHF, or other means). Is it successful now? |
| 9 | No: Contact ATC for further guidance, if all comm unsuccessful | If all means of communication are unsuccessful, then the radio operator will contact ATC to report failed communication attempt and obtain further ATC guidance. |
| 10 | ATC advises to cancel message or continue to attempt contact with flight. | Given previous status report to ATC by the radio operator, ATC will provide additional guidance on message delivery – either to cancel the message or to continue attempted delivery. |
| 11 | Cancel message? | Decision Point - Did ATC advise the radio operator to cancel any further message attempts? Continue through Yes (End Message) or No (Reattempt Delivery) options. |
| 12 | Yes: Deliver Message: ATC Clearance ATC Request ATC Advisory | With the successful contact either via SATVOICE or other means, the radio operator will deliver one of the 3 types of messages – clearance, request or advisory. |
| 13 | ATC – Is message a clearance? | Decision Point – Is the current message a clearance message? |
| 14 | Yes: Pilot reads back ATC clearance | If the current message is a clearance, then the pilot will readback the clearance verbatim to the radio operator. The radio operator is closely monitoring to ensure pilot readback is correct. |
| 15 | No: ATC request or advisory | Since the current message is not a clearance, then it is determined to be a request or advisory. |
| 16 | Pilot responds to ATC request or advisory | Since the current message is a request or advisory, it is not necessary for the pilot to read it back; therefore, he simply responds to it. |
| 17 | Acknowledge clearance readback or request/advisory pilot response. | The radio operator will acknowledge the message as appropriate. If it is a Clearance, the RO acknowledges the pilot’s readback. However, if it is a request or advisory, the RO will just acknowledge the pilot’s response for documentation back to ATC. |
| 18 | Send message back to ATC w/ ATC clearance Pilot readback or request/advisory Pilot Comments. | Since this message activity started with an ATC Clearance, request or advisory, the radio operator is completing the communication back to ATC with the pilot’s response – either documenting the pilot’s readback to the clearance or providing any comments from the pilot to the request or advisory. |
| 19 | ATC receives and processes message. | The ATC controller receives the message sent from the radio operator with the pilot’s response. ATC processes the message/information as appropriate. |
| 20 | End | End communication either due to successful delivery or cancellation direction by ATC. |

Figure 4-4.	Air-ground (flight crew to radio operator) SATVOICE flowchart

Table 4-3.	Air-ground (flight crew to radio operator) SATVOICE flowchart descriptions

| Ref | Ground to Air Communication Process | What is done |
| --- | --- | --- |
| 1 | Pilot calls RO via SATVOICE, provides Flight ID | Pilot from an oceanic flight calls the radio operator via SATVOICE. Pilot provides Flight ID. |
| 2 | Answer SATVOICE line; acknowledge Flight | Radio operator answers the SATVOICE line and acknowledges the flight’s call. |
| 3 | Pilot states intentions or Request | Pilot states his/her intentions for placing the SATVOICE call, which could be an Initial Check-in, a Guard Change, a pilot request, or provides a position report |
| 4 | Acknowledge Position Report or Request | Radio operator acknowledges the pilot’s position report or his/her request |
| 5 | RO process message per local procedures | Radio operator relays position report or request to ATC per local procedures. |
| 6 | Request Flight perform Radio Check and SELCAL on HF, if not previously done | After obtaining flight information for a new flight contact, the radio operator will request an HF radio check and SELCAL with the aircraft, which they will perform as soon as possible. Depending upon the configuration of communications assets within the aeronautical station, the SELCAL could be with the same radio operator who is on the SATVOICE call, or the pilot could be transferred to another radio operator, handling those HF frequencies and radio assets. |
| 7 | Final Position Report? | Decision Point – Is this the pilot’s final position report? This may determine activity with an adjacent FIR. |
| 8 | Yes: Assign Adjacent FIR Frequencies and advise Flight to contact next center at FIR Boundary. | When this is the pilot’s final position report in the radio operator’s FIR responsibility, then the radio operator provides the adjacent FIR frequencies to the pilot and advises the aircraft to contact the next center at the FIR boundary. |
| 9 | Pilot acknowledges Frequency Assignment. | Pilot acknowledges the HF or VHF Frequency Assignment for the next FIR boundary from the radio operator. |
| 10 | Send Message to ATC and Adjacent FIR | Radio operator sends a message to ATC and the adjacent FIR about flight status and/or pending entry into next FIR. |
| 11 | ATC Receives message and/or phone call, handled by local procedures | The ATC controller will receive the position report or the pilot request from the radio operator. The radio operator may also call the ATC controller depending upon local procedures. |
| 12 | Adjacent FIR Receives Message | The Adjacent FIR will receive the Position Report and flight status, in preparation of handling the flight as it enters their FIR. |
| 13 | End | End communication flow due to completion of communications activities, such as delivery of pilot Position Report or Request, and handoff to adjacent FIR, if appropriate, and/or updating of Radio Operator’s flight information database. |

## Communication failures and emergencies

### Loss of SATVOICE Connection

#### If the SATVOICE connection is lost during a communication, the radio operator should:

- attempt to contact the aircraft using any means at their discretion, e.g., SATVOICE retry, HF (SELCAL), data link, VHF or relay through another aircraft, to establish positive voice communications for that flight; and

- deliver the clearance or receive the message as appropriate.

### Airspace emergencies – loss of controller or radio facility capabilities

#### In situations where the controller or radio facility loses capabilities, then the radio operator should comply with procedures related to emergencies, communication failure and contingencies provided (Doc 4444, Chapter 15) and use whatever means are available to provide information on the emergency situation and any directives, for example:

- HF broadcast capability – Radio operators will transmit a voice broadcast on HF radio of emergency situation and any directives.

- Contact alternate FIR communications facilities for backup communications support;

- Aircraft-aircraft communications;

- Volmet broadcasts, containing emergency information, where available; and

- SATVOICE Group Call Broadcast function for future consideration per ICAO Annex 10, Vol III, Sec 8.5

# Flight crew procedures

## General

    - Operators with data link equipped aircraft (CPDLC, ADS-C, and FMC WPR) operating in airspace where data link services are provided should use data link as their normal means of communications.  Some normal ATC communications and most non normal communications will require use of voice communications.  The flight crew should use either SATCOM or HF voice at their discretion.  Urgency, type of message being communicated, current atmospheric conditions, and company standard operating procedures are all factors in determining which voice system to use.

    - Although the underlying technology lends itself to a conversational mode of communications, such use can create misunderstanding and confusion.  Therefore, when using SATCOM voice, normal RTF conventions must be followed identical to HF communications in accordance with standard ICAO phraseology, as defined in Annex 10, Volume II, Chapter 5 and Doc 4444 chapter 12 and Doc. 8400.

    - On initial contact with a radio station, the flight crew should provide flight identification and request frequency assignment and perform a SELCAL check on HF. After a successful SELCAL check, all subsequent communications with that radio station may be performed via SATCOM Voice or HF voice based on regulatory requirements and company policies.

    - The flight crew should normally make calls to the radio station facility serving the airspace in which the aircraft is flying.  If oceanic airspace has not been entered, the radio station serving the first oceanic centre should be contacted.  If communications are lost with the current aeronautical station, the flight crew should attempt contact with any other aeronautical station to relay.

    - If a HF SELCAL check is required before or after entering a FIR, the flight crew should contact the radio operator and complete a HF SELCAL check.  A check of the SATCOM voice system similar to a HF SELCAL is not required because the system will alert the crew if the system is not working properly.

## Flight crew initiated

    - SATCOM short codes are published in State AIPs and some charts.  Short codes may be stored in SATCOM avionics for easy access by the flight crew.

    - When contacting ATC crews should utilize the appropriate safety priority.

Example:

|  | <line rings at aeronautical station> |
| --- | --- |
| Flight crew | Arctic Radio, Continental 99, position report |
| Radio operator | Continental 99 Arctic Radio |
| Flight crew | Arctic Radio, Continental 99, <message> |
| Radio operator | Continental 99, Arctic Radio, <read back message> |
| Flight crew | Arctic Radio, Continental 99 out |

## Flight crew receives call

    - The flight crew should visually confirm the priority of the incoming call and verify that it is an ATC priority call.  Reply to calls utilizing  standard phraseology ( see paragraph 5.1.2)

    - The flight crew should not act on ATC instructions from SATCOM calls with other than ATC priority calls, and if in doubt terminate the call and initiate a new call for confirmation.

## Oceanic clearances

    - Coordination of oceanic clearances should be in accordance with state AIPs.

## Contingencies

### SATCOM busy signal or no answer

#### Normally, when initiating a SATCOM voice call to a radio facility that supports SATCOM voice services, the flight crew should receive an answer.  When a SATCOM voice call returns a busy signal or there is no answer, the flight crew should use other means of communications.

### SATCOM Failure

#### If the aircraft SATCOM voice equipment has malfunctioned or for any other reason the SATCOM voice system is unavailable the flight crew should:

- revert to an other means of communication; and

- notify the radio station of the SATCOM voice failure.

- – RCP 400 specification – allocations for satellite voice communications

This appendix provides a supplement to the RCP 400 specification provided in the First Edition of the Global Operational Data Link Document (GOLD).  The supplement includes the RCP allocations for satellite voice communications, consistent with RCP 400 “top sheet.”

  - Terms and definitions

Refer to GOLD, Appendix B, paragraph B.1 for general terms and definitions applicable to RCP specifications.  This section provides additional terms and definitions to describe the RCP allocations for radio operator satellite voice communications.

Figure A- .	Typical voice communication transaction allocation – controller initiated via a radio operator using satellite voice communication

| RCP specification |  |
| --- | --- |
| Term | Description |
| Operational communication transaction | The process a human uses to initiate the transmission of an instruction, clearance, flight information, and/or request, and is completed when that human is confident that the transaction is complete. |
| Required communication performance | A statement of the performance requirements for operational communication in support of specific ATM functions. |
| RCP specification | A specification (e.g. RCP 400) that represents the values assigned to RCP parameters for communication transaction time, continuity, availability and integrity, and allocations to ATSP, aircraft, CSP and operator. |
| RCP expiration time (ET) | The maximum time for the completion of the operational communication transaction after which the initiator is required to revert to an alternative procedure. |
| RCP nominal time (TT 95%) | The maximum nominal time within which 95% of operational communication transactions are required to be completed. |
| RCP continuity (C) | The required probability that an operational communication transaction can be completed within the communication transaction time, either ET or TT 95%, given that the service was available at the start of the transaction. Voice communications: This translates into 5 out of 100 calls not being able to conclude their voice transactions within the allotted time or the call could be disconnected for any reason, including aircraft maneuvers, switching satellites or any loss of service while on the call. |
| RCP availability (A) | The required probability that an operational communication transaction can be initiated when needed. Voice communications: This translates to any failure prohibiting the call to be initiated to include congestion (much like the analogy of a terrestrial mobile phone network). However this definition does not apply to a busy condition whereby the entity being called is already on the phone and does not have a way to put the existing call on hold or if able to, rejects the additional incoming call. |
| RCP integrity (I) | The required probability that an operational communication transaction is completed with no undetected errors. Note.— Whilst RCP integrity is defined in terms of the “goodness” of the communication capability, it is specified in terms of the likelihood of occurrence of malfunction on a per flight hour basis, e.g. 10-5, consistent with RNAV/RNP specifications. Voice communications: This translates to the ability to conduct the voice transaction with enough intelligibility to understand the verbal request. Additionally, this could also manifest in the crew executing an incorrect instruction because it was misunderstood. (It may not be clear at the time of this submittal, what type of error rate would reflect this condition, or if the read-back would mitigate or compound this). |

| /V transaction time |  |
| --- | --- |
| Term | Description |
| Operational Performance | The portion of the transaction time (used for intervention) that does not include the times for message composition or recognition of the operational response. Voice communications: Operational Performance (C to N) = RCTP+ Queue/Connect Performance (F to G) + Call Performance (H to J) |
| Required Communication Technical Performance (RCTP) | The portion of the (intervention) transaction time that does not include the human times for message composition, operational response, and recognition of the operational response. Voice communications: RCTP = RTCPATSU + RCTPAS + RCTPAS/AIR + RCTPCSP |
| Monitored Performance | The monitored combination of the time it takes to organize, prepare, dial and perform an interactive call plus the technical performance of the ground and air equipment. Voice communications: Monitored Performance (F to J) = RCTPAS/AIR + Queue/Connect Performance (F to G) + Call Performance (H to J) |
| Call Performance | The operational portion of the transaction time to prepare the operational response, and includes the recognition of the instruction, and message composition during an interactive call between the Radio Operator and Flight Crew, e.g. flight crew/HMI for intervention transactions. Voice communications: Call Performance = (H to J), where “H” denotes when the aircraft indicates an incoming call to the flight crew and “J” denotes when the callers operationally completes the call. (H to J) includes any tasks for the radio operator to send the message to the ATSU. |
| Queue/Connect Performance | The operational portion of the transaction time to organize and place the call either via a manual or automated dialing sequence depending on equipment at the RO facility. Voice communications: Queue/Connect Performance = (F to G), where “F” denotes when the message from the ATSU via the network is sent to the queue, and “G” denotes when the last digit of the dialing sequence is finished. |
| RCTPATSU | The summed critical transit times for an ATC intervention message and a response message, allocated to the ATSU system. Voice communications: RCTPATSU = (C to D) + (M to N) |
| RCTPCSP | The summed critical transit times for an ATC intervention message and a response message, allocated to the CSP system. Voice communications: RCTPCSP = (D to E) + (K to M) |
| RCTPAS/AIR | The technical time for the air-ground network and associated components to initiate a call. Voice communications: RCTPAS/AIR = (G to H), where “G” denotes when the last digit of the dialing sequence is finished and “H” denotes denotes when the aircraft indicates an incoming call to the flight crew. |
| RCTPAS | The summed critical transit times for an ATC intervention message and a response message allocated to the aeronautical station. Voice communications: RCTPAS = (E to F) + (J to K), where (J to K) includes two concurrent processes: a) the aircraft and aeronautical station technically disconnect the call; which is assumed. Operationally, the call is disconnected when the flight crew and radio operator complete the call; and b) the aeronautical station sends the response to the ATSU via the ground-ground network; the performance is denoted by RCTPAS. |

| Continuity |  |
| --- | --- |
| Term | Description |
| C for operational performance | The proportion of intervention messages and responses that can be delivered within the specified operational performance time for intervention. Voice communications: Continuity would take into consideration any dropped calls. |
| C for RCTP | The proportion of intervention messages and responses that can be delivered within the specified RCTP for intervention. Voice communications: Continuity would take into consideration any dropped calls. |
| C for RCTPATSU | The proportion of intervention messages and responses that can be delivered within the specified RCTPATSU for Intervention. |
| C for RCTPCSP | The proportion of intervention messages and responses that can be delivered within the specified RCTPCSP for Intervention. |
| C for RCTPAS/AIR | The proportion of intervention messages and responses that can be delivered within the specified RCTPAS/AIR for Intervention. Voice communications: Continuity would take into consideration any dropped calls. |
| C for RTCPAS | The proportion of intervention messages and responses that can be delivered within the specified RCTPAS for Intervention. |

| Availability |  |
| --- | --- |
| Term | Description |
| Service availability (ACSP) | The required probability that the communication service is available to all users in a specific airspace when desired. |
| Unplanned outage duration (minutes) | The time from when an unplanned outage begins to when the ATSU receives notification that the service has been restored. |
| Unplanned outage duration limit (minutes) | A criterion applied to a given operational airspace or FIR that defines the maximum time for the duration of an unplanned outage at which time there is an operational impact. |
| Maximum number of unplanned outages (per year) | A criterion applied to a given operational airspace or FIR that defines the maximum number allowed for unplanned outages in any twelve month period. |
| Maximum accumulated unplanned outage time (min/yr) | A criterion applied to a given operational airspace of FIR that defines the maximum time allowed for the total sum of the unplanned outages that exceed the unplanned outage duration limit in any twelve month period Note.— The criterion does not apply to unplanned outages that are less than the unplanned outage duration limit or planned outages. Unplanned outages that are less than the unplanned outage duration limit are considered against the criterion for continuity. |
| Unplanned outage notification delay (min) | Notification to the ATSU of an unplanned outage. Measured from when the unplanned outage begins to when the ATSU receives notification. |
| Aircraft system availability (AAIR) | The required probability of available capability on an aircraft. Note.— The actual aircraft system availability is computed assuming that the service is available in the relevant airspace. |

  - RCP 240 specification

Refer to GOLD, Appendix B, paragraph B.2 for RCP 240 specification.

  - RCP 400 specification

| RCP Specification |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| RCP specification |  |  |  |  | RCP 400 |  |
| Airspace specific considerations |  |  |  |  |  |  |
| Interoperability |  | Specify interoperability criteria, e.g. FANS 1/A, satellite voice communications. |  |  |  |  |
| ATS Function |  | Specify ATS function(s), e.g. applicable separation standard |  |  |  |  |
| Application |  | Specify controller-pilot ATC communication intervention capability, e.g. CPDLC application per ICAO Doc 4444, and RTCA DO-306/EUROCAE ED-122, Annex A, satellite voice communications per SVGM. |  |  |  |  |
| RCP parameter values |  |  |  |  |  |  |
| Transaction time (sec) |  |  | Continuity (C) | Availability (A) |  | Integrity (I) |
| ET = 400 |  |  | C(ET) = 0.999 | 0.999 |  | Malfunction = 10-5 per flight hour |
| TT 95% = 350 |  |  | C(TT 95%) = 0.95 |  |  |  |
| RCP monitoring and alerting criteria |  |  |  |  |  |  |
| Ref: | Criteria |  |  |  |  |  |
| MA-1 | The system shall be capable of detecting failures and configuration changes that would cause the communication service to no longer meet the RCP specification for the intended function. |  |  |  |  |  |
| MA-2 | When the communication service can no longer meet the RCP specification for the intended function, the flight crew and/or the controller shall take appropriate action. |  |  |  |  |  |
| Notes |  |  |  |  |  |  |
| Note 1.— Rationale for the criteria provided in this specification can be found in ICAO Annex 11, ICAO Doc 4444, ICAO Doc 9689, and RTCA DO-306/ED-122. Note 2.— The values for transaction times are to be applied to transactions that are representative of communication capability for the controller to intervene with a specific operator, aircraft type, and aircraft identification. Note 3.— If changes are made to the system capacity limits, as specified by the airspace requirements, and the changes cause the system to perform below the RCP specification, this would be considered a change in system configuration. |  |  |  |  |  |  |

    - RCP 400/D allocations

Refer to GOLD, Appendix B, paragraph B.3.1 for RCP 240/D allocations applicable to CPDLC.

    - RCP 400/V allocations

The RCP 400/V allocations are applicable controller-initiated communications via a to radio operator using satellite voice communications.

      - Air traffic service provider (ATSP)

| RCP communication transaction time and continuity criteria |  |  |  |
| --- | --- | --- | --- |
| Specification: RCP 400/V | Application: Controller intervention (GTA), Satellite voice |  | Component: ATSP |
| Transaction Time Parameter | ET (sec) C = 99.9% | TT (sec) C = 95% | Compliance Means |
| Transaction Time (A to Z) | 400 | 275 | Analysis, CSP contract/service agreement. See also paragraph A.3.2.2. |
| RCP Time Allocations |  |  |  |
| Initiator (A to C) + (N to Z) | 30 | 20 | Analysis, simulations, safety and human factors assessments |
| Operational Performance (C to N) | 370 | 255 | Monitored, CSP contract/service agreement. See also paragraph A.3.2.2. |
| Operational Performance Time Allocations |  |  |  |
| Monitored Performance (F to J) | 352 | 245 | Initially, by analysis, simulations, safety human factors assessments Post-implementation monitoring |
| RCTP (RCTPATSU + RCTPCSP + RCTPAS + RCTPAS/AIR) | 25 | 15 | Monitored, estimated, CSP contract/service agreement. See also paragraph A.3.2.2 |
| RCTP Time Allocation |  |  |  |
| RCTPATSU (C to D) + (M to N) | 4 | 2 | Pre-implementation demonstration |

| RCP availability criteria |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specification: RCP 400/V | Application: Controller intervention (GTA), Satellite voice |  |  | Component: ATSP |
| Availability parameter | Efficiency | Safety | Compliance means |  |
| Service availability (ACSP) | N/A | 0.999 | Contract/service agreement terms. Note.— For guidelines to aid in the development of the contract/service agreement with the CSP, see paragraph A.3.2.2, RCP 400/D allocation to CSP for RCP availability criteria. |  |

| RCP integrity criteria |  |  |  |
| --- | --- | --- | --- |
| Specification: RCP 400/V |  | Application: Controller intervention (GTA), Satellite voice | Component: ATSP |
| Integrity parameter | Integrity value |  | Compliance means |
| Integrity (I) | Note.— RCP integrity criteria related to RCP 400/D are the same as those related to RCP 240/D. See GOLD, paragraph B.2.1.1. |  |  |

| RCP monitoring and alerting criteria |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specification: RCP 400/V |  | Application: Controller intervention (GTA), Satellite voice | Component: ATSP |  |
| Ref: | Criteria |  |  | Compliance means |
| All | Note.— RCP monitoring and alerting criteria related to RCP 400/D are the same as those related to RCP 240/D. See GOLD, paragraph B.2.1.1. |  |  |  |

| RCP related safety requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specification: RCP 400/V |  |  | Application: Controller intervention (GTA), Satellite voice | Component: ATSP |
| Ref | Related RCP Parameter | Safety requirement |  |  |
| All | A, C, I | Note.— Safety requirements related to RCP 400/D are the same as those related to RCP 240/D. See GOLD, paragraph B.2.1.1. |  |  |

      - Communication service provider (CSP)

| RCP communication transaction time and continuity criteria |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specification: RCP 400/V | Application: Controller intervention (GTA), Satellite voice |  |  | Component: CSP |
| Transaction Time Parameter |  | ET (sec) C = 99.9% | TT (sec) C = 95% | Compliance means |
| RCTP Time Allocation |  |  |  |  |
| RCTPCSP (D to E) + (K to M) |  | 10 | 6 | Contract/service agreement terms |
| RCTPAS (E to F) + (J to K) |  | 4 | 2 | Contract/service agreement terms |
| RCTPAS/AIR (G to H) |  | 7 | 5 | Contract/service agreement terms Note: Criteria are shared between aircraft system, ground system and air-ground network. |
| Queue/connect performance (E to G) |  | 75 | 60 | Monitored, estimated, CSP contract/service agreement. |
| Call performance (H to J) |  | 270 | 180 | Initially, by analysis, simulations, safety human factors assessments Post-implementation, monitored, estimated |

| RCP availability criteria |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specification: RCP 400/V | Application: Controller intervention (GTA), Satellite voice |  | Component: CSP |  |
| Availability parameter |  | Efficiency | Safety | Compliance means |
| Service availability (ACSP) |  | N/A | 0.999 | Contract/service agreement terms |
| Unplanned outage duration limit (min) |  | N/A | 20 | Contract/service agreement terms |
| Maximum number of unplanned outages |  | N/A | 24 | Contract/service agreement terms |
| Maximum accumulated unplanned outage time (min/yr) |  | N/A | 520 | Contract/service agreement terms |
| Unplanned outage notification delay (min) |  | N/A | 10 | Contract/service agreement terms |

| RCP integrity criteria |  |  |  |
| --- | --- | --- | --- |
| Specification: RCP 400/V |  | Application: Controller intervention (GTA), Satellite voice | Component: CSP |
| Integrity parameter | Integrity value |  | Compliance means |
| Integrity (I) | Note.— RCP integrity criteria related to RCP 400/D are the same as those related to RCP 240/D. See GOLD, paragraph B.2.1.2. |  |  |

      - Aircraft system

| RCP communication transaction time and continuity criteria |  |  |  |
| --- | --- | --- | --- |
| Specification: RCP 400/V | Application: Controller intervention (GTA), Satellite voice |  | Component: Aircraft system |
| Transaction Time Parameter | ET (sec) C = 99.9% | TT (sec) C = 95% | Compliance Means |
| RCP Time Allocation |  |  |  |
| Operational Performance Time Allocation |  |  |  |
| Call performance (H to J) | 270 | 180 | Human-machine interface capability, pre-implementation demonstration |
| RCTP Time Allocation |  |  |  |
| RCTPAS/AIR (G to H) | 7 | 5 | Pre-implementation demonstration Note: Criteria are shared between aircraft system, ground system and air-ground network. |

| RCP availability criteria |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specification: RCP 400/V | Application: Controller intervention (GTA), Satellite voice |  |  | Component: Aircraft system |
| Availability parameter |  | Efficiency | Safety | Compliance means |
| AAIR |  | N/A | 0.999 | Analysis, architecture, design, pre-implementation demonstration |

| RCP integrity criteria |  |  |  |
| --- | --- | --- | --- |
| Specification: RCP 400/V |  | Application: Controller intervention (GTA), Satellite voice | Component: Aircraft system |
| Integrity parameter | Integrity value |  | Compliance means |
| Integrity (I) | Note.— RCP integrity criteria related to RCP 400/D are the same as those related to RCP 240/D. See GOLD, paragraph B.2.1.3. |  |  |

| RCP monitoring and alerting criteria |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specification: RCP 400/V |  | Application: Controller intervention (GTA), Satellite voice | Component: Aircraft system |  |
| Ref: | Criteria |  |  | Compliance means |
| All | Note.— RCP monitoring and alerting criteria related to RCP specification 400/D are the same as those related to RCP 240/D. See GOLD, paragraph B.2.1.3. |  |  |  |

| RCP related safety requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specification: RCP 400/V |  |  | Application: Controller intervention (GTA), Satellite voice | Component: Aircraft system |
| Ref | Related RCP Parameter | Safety requirement |  |  |
| All | A, C, I | Note.— Safety requirements related to RCP 400/D are the same as those related to RCP 240/D. See GOLD, paragraph B.2.1.3. |  |  |

      - Aircraft operator

| RCP communication transaction time and continuity criteria |  |  |  |
| --- | --- | --- | --- |
| Specification: RCP 400/V | Application: Controller intervention (GTA), Satellite voice |  | Component: Aircraft operator |
| Transaction Time Parameter | ET (sec) C = 99.9% | TT (sec) C = 95% | Compliance Means |
| RCP Time Allocations |  |  |  |
| Operational Performance Time Allocations |  |  |  |
| Call performance (H to J) | 270 | 180 | Procedural capability, flight crew training and qualification in accordance with safety requirements. |
| RCTP Time Allocation |  |  |  |
| RCTPAS/AIR (G to H) | 7 | 5 | CSP contract/service agreement. Aircraft type design approval, maintenance. |

| RCP availability criteria |  |  |  |
| --- | --- | --- | --- |
| Specification: RCP 400/V | Application: Controller intervention (GTA), Satellite voice |  | Component: Aircraft operator |
| Availability parameter | Efficiency | Safety | Compliance means |
| AAIR | N/A | 0.999 | Aircraft type design approval, maintenance, properly configured user-modifiable software, e.g. ORT |
| Service availability (ACSP) | N/A | 0.999 | Contract/service agreement terms. Note.— For guidelines to aid in the development of the contract/service agreement with the CSP, see paragraph A.3.2.2, RCP 400/D allocation to CSP for RCP availability criteria. |

| RCP integrity criteria |  |  |  |
| --- | --- | --- | --- |
| Specification: RCP 400/V |  | Application: Controller intervention (GTA), Satellite voice | Component: Aircraft operator |
| Integrity parameter | Integrity value |  | Compliance means |
| Integrity (I) | Note.— RCP integrity criteria related to RCP 400/D are the same as those related to RCP 240/D. See GOLD, paragraph B.2.1.4. |  |  |

| RCP monitoring and alerting criteria |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specification: RCP 400/V |  | Application: Controller intervention (GTA), Satellite voice | Component: Aircraft operator |  |
| Ref: | Criteria |  |  | Compliance means |
| All | Note.— RCP monitoring and alerting criteria related to RCP 400/D are the same as those related to RCP 240/D. See GOLD, paragraph B.2.1.4. |  |  |  |

| RCP related safety requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Specification: RCP 400/V |  |  | Application: Controller intervention (GTA), Satellite voice | Component: Aircraft operator |
| Ref | Related RCP Parameter | Safety requirement |  |  |
| All | C, I | Note.— Safety requirements related to RCP 400/D are the same as those related to RCP 240/D. See GOLD, paragraph B.2.1.4. |  |  |

- Post-implementation monitoring

The ICAO Global Plan calls for the implementation of a performance based system and ICAO Annex 11 requires that communication system performance is monitored to verify that an acceptable level of safety continues to be met.  Annex 11 at paragraph 2.2.7.5 states:

“Any significant safety-related change to the ATC system, including the implementation of a reduced separation minimum or a new procedure, shall only be effected after a safety assessment has demonstrated that an acceptable level of safety will be met and users have been consulted.  When appropriate, the responsible authority shall ensure that adequate provision is made for post-implementation monitoring to verify that the defined level of safety continues to be met.”

Oversight of the compliance to the Annex 11 requirements is a matter for the States. However, States participate in planning and implementation regional groups (PIRGs), and most use a regional monitoring agency to facilitate monitoring activities within their respective region. The individual states/ATSPs will need to provide the data and information and analysis that will portray regional performance measures. The ATSPs, operators, CSPs, airframe manufacturers, and equipment suppliers all need to participate in reporting and  resolving problems associated among the ATSPs and with aircraft.

Monitoring of SATCOM voice communications in terms of RCP is an important part of the performance based system described in the ICAO global plan. To successfully achieve this performance monitoring on a global scale will require the use of common practices and data. It is only through this common performance monitoring that RCP data can be aggregated from an ATSP level through to a regional monitoring agency level and then to global level. This aggregation of performance data is in accordance with the guidelines provided in ICAO Doc 9883 Manual on Global Performance of the Air Navigation System.

This appendix contains the following guidance material:

- ATSP or aeronautical station data collection and analysis - This section defines a common data reporting format.  Guidance material is included on how to obtain the required data points and measure actual SATCOM voice call performance.

- Problem reporting and resolution – This section provides guidance on the problem identification and resolution process.

- Regional/global performance monitoring – This section provides guidance on providing a respository for exchange actual SATCOM performance and problem report/resolutions at a regional and global level.

  - ATSP or aeronautical station data collection and analysis

  - Problem reporting and resolution

  - Regional/global performance monitoring

- Regional/State-specific information

  - Current uses of SATCOM voice

- SV7-0247-ML – COMMENT.   It would be interesting to add in the Guidance Material a world map (or at least a table) detailing the rules that are currently in used in the various Flight Information Regions:
	1)  What is the primary means of communication: voice or datalink. For example: in the NAT region datalink is primary, whereas in China region voice is the primary means of communication.
	2)  Is it mandatory to bring 1HF or 2 HF to fly in this region/state.
SUGGESTED CHANGE:  Add such a world map or table that every ANSP could complete. This could appear in Chapter 2.
15-Sep-11-IR-SVTF/2 – Could be part of Region/State-specific appendix?
23-Sep-11-TK – TK – I would have concerns with putting dynamically changing material in the main body of the document; however, an Appendix of such information is established here to gather this information.  Make Apx C for now.

  - Region/State contact information

- SV4-0137-PR – COMMENT:  SAT Voice may well be the medium of the future, but it’s very difficult to get access let alone for the controllers to determine who they can call and via what ;>)
The global manual should include contact details for those folks controlling access to INMARSAT/MTSAT/MTSAT and whatever SAT that we as ANSP can call to get access.
SUGGESTED CHANGE:  
22-Jul-11-TK – Maybe a new Appendix, equivalent to what we have in GOLD, Appendix E
14-Sep-11-IR-SVTF/2 – Maybe will be addressed by the phone number data based (28-day update).  See para 3.1.5.2, letter c).
23-Sep-11-TK – Make Apx C for now.

  - Planning for SATCOM voice implementation

- SV-0255-AH – COMMENT: Document has no timeline or cut-off to define “current” service from “future” services. For example, there is mention of Inmarsat SBB and IridiumNext, but no method to identify those systems in the flight plan, and no specifics on the use of VoIP dialing.. It is not clear if the guidance material would need to be updated to make a change to an underlying technology. This should help with the caller ID issue as well.
SUGGESTED CHANGE: Add a timeline to the guidance material that indicates which services are available in the first phase, and which services are “future”.
15-Sep-11-IR-SVTF/2 – Could be part of Regional/State-specific Appendix.
23-Sep-11-TK – Make Apx C for now.

- Planning for desirable SATCOM voice features

This section includes guidance on the planning for implementation of SATCOM voice features that may be required for future uses in ATM.

  - 2-channel SATCOM voice system

- SV7-0234-ML – COMMENT: Perhaps could we recommend in the SVGM the availability of at least 2 channels per SATCOM system on-board? Benefits would be obvious in terms of redundancy and operational use (with 2 channels, 2 communications can be established at a time, compared to 1 with HF). 
SUGGESTED CHANGE:  
14-Sep-11-IR-SVTF/2 – Related to safety requirement.  What is the safety requirement?
16-Sep-11-IR-SVTF/2 - Not all meet 2-channel today.  Why would it be a requirement?
Are their any limitations on use of a single channel installation?, e.g., MEL relief, operational applications.  Does this provide a mitigation means to undesirable effects of preemption?  Should guidance address planning for new features? 
Put this in a new “desirable” appendix.
23-Sep-11-TK – Make Apx D for now.

  - Caller identification display

- SV7-0265-GL – COMMENT:  Iridium and Inmarsat systems cannot currently provide CLI and PIN information to the receiving aircraft. (3.1.6.1 (c) SVGM) (Par. 7 (j) AC 20-150A)
SUGGESTED CHANGE:  
15-Sep-11-TK – Issue “Safety requirements”, “Security requirements” and “non-compliance of current systrems.”
23-Sep-11-TK – Move to appendix as “desirable.”

- SV7-0268-GL – COMMENT:  Current aircraft systems cannot comply with the requirement to display CLI / PIN information for the pilots to confirm. (4.3.1 SVGM) (Par. 7 (j) AC 20-150A)
SUGGESTED CHANGE 
15-Sep-11-IR-TK – Issue “non-compliance of current systems.”
23-Sep-11-TK – Revised text to indicate CLI/PIN display to a flight crew is a desirable feature and not required.  Move to Apx D as desirable feature.

  - Conference calling

- SV7-0259-GL – COMMENT:  When receiving an air-to-ground call presently, the systems at most CSPs have one to four inbound phone numbers that will “roll” to any available RO.  To accommodate “Conference Calling” CSPs are considering associating a SATCOM phone number with a specific HF frequency “family group.”  Pilots would contact the RO based on geographic location associated with an HF “family of frequencies.”  Charting currently does not accommodate this effectively. This will most likely require an RO to assign a SATCOM number, Primary and Secondary HF frequencies when entering a new FIR or when changing frequencies. (3.1.5.3 SVGM)
SUGGESTED CHANGE:  
15-Sep-11-TK – Issue “Safety requirements” & “non-compliance of current systrems”.” –
23-Sep-11-TK – From IR-SVTF/2 review:
What is it?  When more than two parties are involved (within practical limits)
Air conference - Radio operator or controller talking to two or more aircraft
Ground conference - Aircraft talking to Radio operator, AOC, or controller or all three or others – can be done by aeronautical station via “phone patch”
What is the purpose?  To facilitate resolution of an operational issue where multiple parties at different locations are involved, e.g., broadcast on HF, ACARS, whatever means are used today
There is no operational requirement for this capability
Some issues – We do it on VHF today, but on SATCOM, it is confusing cause you cut people off due to SATCOM delays
Related to “ruthless” preemption – may mitigate against the effects of interrupting essential or critical communications:  2-channel could potentially mitigate the effects or procedures and proper use of priority levels could potentially mitigate the effects
Resolution rules - use procedures and existing priority scheme/capability to mitigate against the effects of ruthless preemption
Also, resolution rule implies that conference calling is not a requirement.  Will highlight in desirable appendix.  Moved all related comments to new Apx D and deleted item e).  Close.

- SV6-0182-GL – COMMENT:  This means that when calling the SATCOM RO the aircraft will be joining a “Conference Call” in progress.  This technology is not implemented by any CSP at this time.
SUGGESTED CHANGE:  
4-Sep-11-TK – CSP or radio facility?  Or ATS unit?  Is it a requirement?  Or even a guideline?
15-Sep-11-TK – Issue “Performance/safety specifications” and “non-compliance of current systrems.”
23-Sep-11-TK – See comment SV7-0259 for resolution status.  See all comments on 3.1.5.3 e)
SV7-0259
SV6-0182
SV7-0257
SV2-0053
SV2-0040
SV7-0251

- SV7-0257-GL – COMMENT:  “Conference Calling” inbound to the CSP, as directed in 3.1.5.3 (e) of the SVGM, is not accommodated by any CSPs (NAV Canada, ARINC, Shanwick (IAA)).
SUGGESTED CHANGE:  
15-Sep-11-TK – Issue “Performance/safety specifications” and “non-compliance of current systrems.”
23-Sep-11-TK – See comment SV7-0259 for resolution status.  See all comments on 3.1.5.3 e)
SV7-0259
SV6-0182
SV7-0257
SV2-0053
SV2-0040
SV7-0251

- SV2-0053-MM – COMMENT:  This paragraph should be removed entirely as I believe it reduces the Safety Case and also introduces the possibility of mis-interpretation and potential callsign confusion.  To have this facility would need extensive trials and Safety Case reviews. (Comment by Irish Aviation Authority).
Refer to “Accept a new call from a flight crew as a participant of an existing SATCOM voice communication already in progress, e.g., the new caller should be able to hear the transmission already in progress and standby for an opportunity to intervene. ( This is an example of “Conference”, …)”
SUGGESTED CHANGE:  Delete item.
15-Sep-11-TK – Issue “Performance/safety specifications” and “non-compliance of current systrems.”
23-Sep-11-TK – See comment SV7-0259 for resolution status.  See all comments on 3.1.5.3 e)
SV7-0259
SV6-0182
SV7-0257
SV2-0053
SV2-0040
SV7-0251

- SV2-0040-BP – COMMENT:  Is this a “requirement” or is only “desirable”?  If this is a “requirement” (and so, new) then it will need to be examined and defined separately.
SUGGESTED CHANGE:  Remove text.
1-Jun-11-TK – SVTF discussion needed.
15-Sep-11-TK – Issue “Performance/safety specifications” and “non-compliance of current systrems.”
23-Sep-11-TK – See comment SV7-0259 for resolution status.  See all comments on 3.1.5.3 e)
SV7-0259
SV6-0182
SV7-0257
SV2-0053
SV2-0040
SV7-0251

- SV7-0251-AH – COMMENT: Conference calling is not a required feature, is not part of the ICAO SARPs for AMS(R)S, is not part of the DO-210D requirements, is not part of AC 20-150A requirements, and is a potential safety hazard.
SUGGESTED CHANGE: Delete entire clause “e”.
15-Sep-11-TK – Issue “Performance/safety specifications” and “non-compliance of current systrems.”
23-Sep-11-TK – See comment SV7-0259 for resolution status.  See all comments on 3.1.5.3 e)
SV7-0259
SV6-0182
SV7-0257
SV2-0053
SV2-0040
SV7-0251