THE THIRTY-FIFTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/35)

(Sapporo, Hokkaido, Japan, 7-11 November 2011)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

RNP4 Effect on PACOTS

(Presented by Civil Aviation Bureau, Japan)

SUMMARY

RNP4 reduces Oceanic separation. However, RNP4 equipped aircraft must increase position reports by ADS downlink. To clear the cost benefit by RNP-4 is desired.

ENRI examined efficiency by ratio of RNP4 equipped.

- Introduction

  - RNP4 reduces a oceanic separation 50NM to 30NM. It might reduces separation more by CDP or ITP. However, RNP4 equipped  ratio remains about 30 %, the effect of RNP4 does not shown  enough to cost benefit.  When both aligned flight  pair have RNP4 equipped, separation will be reduced. RNP4 operation need to increase ADS downlink frequency from 16 minutes to 10 minutes due to safety. Increasing downlink occurs communication cost. Presentation of cost benefit about RNP4 is demanded by airlines.

  - At IPACG/34, the ratio of RNP4 equipped potentially increases to up to 44%. Because FANS 1/A aircraft have RNP4 matter. ENRI examined oceanic ATC simulation for some traffic pattern. The ratio of RNP4 equipped and traffic volume are changed.

- Discussion

  - ENRI examined benefit by improved RNP4 equipment in current traffic volume. the 4 types of scenarios are  assumed. RNP_A, RNP_B, RNP_C and RNP_D shows ratio of RNP-4 equipment 30% (Currently), 40%, 60% and 75% respectively.

- Fig 1 shows  the number of flight which is changed altitude due to traffic. R591 is heavies route at the day. “ALL” shows potentially number when all flights have RNP-4 equipment.

Fig1. The number of  flight ( Altitude changed)

- On the route of low traffic volume, benefit was small. On the route of high traffic volume, the benefit are shown as follows:

- The number of flight which is changed altitude for keep ATC separation

- The altitude difference from original altitude  was reduced. Especially, the flights  whose altitude is very low are reduced.

- RNP4 effect is limited for most heavier route in current traffic.

- 4～6　flights/30min. is border.

- RNP_C (60%) is effective.

  - For high density routes, more traffic volume are assumed. Fig 2 shows flight profile sample. “13A” shows that traffic volume is current 1.3 times, and ratio of RNP-4 is “RNP_A”. Growing the ratio of RNP-4 keeps economical altitude in spite of growing traffic. 11A and 11C are same as 12C. And 10C is same as 10A.

140E         170E         160W         130W        100W

Fig 2　Flight profile sample for each traffic and ratio of RNP4 equipped

When traffic is increased, RNP4 equipped is covered the increase.

  - Currently, the ratio of RNP-4 equipped is about only 30%. RNP-4 keeps economical altitude in spite of growing traffic.

- Conclusion

  - The meeting is invited to note the information provided.