THE THIRTY-FIFTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/35)

(Sapporo, Hokkaido, Japan, 7-11 November 2011)

Agenda Item 5:  	Air Traffic Management (ATM) Issue

Update on the status of trial operations on acceptance and provision of

turbulence information by CPDLC

(Presented by Civil Aviation Bureau, Japan)

SUMMARY

This paper updates the status of trial operations on acceptance and provision of turbulence information to capable aircraft by CPDLC within Fukuoka FIR.

- Introduction

  - ATMC has provided turbulence information to capable aircrafts by CPDLC as trial operation since February 10th 2010.  During this trial operation, JCAB has provided the turbulence information in both of CPDLC and HF voice communication, and has evaluated the efficiency.

  - JCAB improved ODP system in July of this year. ODP system could not accept turbulence information automatically from aircraft in particular format such as "20nm SW of CUTEE" into the system so far, but it has been able to accept particular format automatically after system improvement.

- Discussion

  - Currently in the Fukuoka FIR, JCAB provide turbulence information by using both methods CPDLC and HF voice communication. And turbulence information is provided from aircraft which fly around location of observation within height difference of ±2,000ft from altitude of observation and will pass within two hours from time of observation. At the IPACG/34, aircraft operator requested expansion of the reporting area to ± 4,000ft.  JCAB will expand the reporting area to ± 4,000ft after finished the internal coordination.

  - Basically JCAB intends to provide the turbulence information via CPDLC only for aircraft which has established CPDLC connection.  In case of difficulty with this service for aircraft which is established CPDLC connection might be provided via HF voice communication in stead of CPDLC as practicable.

  - If this procedure causes no adverse conditions, JCAB will terminate this trial operation and change over to a regular operation by the end of this fiscal year.

  - At the IPACG/33, JCAB and FAA both agreed the necessity and effectiveness on the scheme for providing turbulence information not only Fukuoka FIR but also in the vicinity of FIR boundary. ATMC trying to shares the turbulence information with Anchorage ARTCC and Oakland ARTCC by using verbal communication or FAX.  ATMC wishes to discuss with aircraft operators to enhance delivery procedures for turbulence information more adequately from/to aircraft.  For example the specific area.

- Conclusion

  - The meeting is invited to note the information provided.