THE THIRTY-FIFTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/35)

(Sapporo, Hokkaido, Japan, 7-11 November 2011)

Agenda Item 5:   Air Traffic Management (ATM) Issues

Study on User Preferred Route (UPR) between S.Korea and New Zealand / Fiji / Australia

(Presented by Korean Air)

SUMMARY

This is a copy of a paper presented to IPACG 35 in Sapporo. It provides information on the results of the study on UPR between S.Korea and New Zealand / Fiji / Australia.

- Introduction

  - Since last December, Korean Air (KAL) has been flying UPR routes between Incheon, Korea and Hawaii. We highly appreciate superb support rendered by the Federal Aviation Administration Oakland, the Japanese Civil Aviation Bureau.

  - The Oceania UPR has proven significant benefits to airlines by allowing optimization of flight planning particularly for long haul operations.

  - KAL UPR trial study has been completed in October 2011 based on near 50 flights per week between S. Korea and New Zealand / Fiji / Australia. (see attached UPR tracks studied)

- Discussion

  - The flight routes KAL has been using for flights between Incheon , Korea(RKSI) and New Zealand / Fiji / Australia are Air Traffic Service (ATS) routes. KAL is operating some more flexible route between Korea and Australia with Oakland control center’s support since September 2010.

  - The initial paper trial indicated that fuel saving of around 1,000 tons annually could be expected by UPRs between RKSI and NZAA/NFFN/YSSY/YBBN/YMML/PTRO.  There would be an additional environmental savings of 3,140 tons of carbon dioxide (CO2) emissions.

ㅇ UPR Operational Savings (Annual)

a) Between RKSI and NZAA ( 730 flights, B747 aircraft )

1) 535 ton fuel

2) 1,685 ton carbon dioxide (CO2)

b) Between RKSI and NFFN ( 156 flights, A330 aircraft )

1) 127 ton fuel

2) 400 ton CO2

c) Between RKSI and YSSY/YBBN/YMML ( 834 flights,  B747 aircraft for YSSY )

( 832 flights, A330 aircraft for YBBN/YMML )

1) 338 ton fuel

2) 1064 CO2 emissions

d) Between RKSI and PTRO ( 68 flights, B737 aircraft )

1) 14 ton fuel

2) 44 CO2 emissions

- Recommendation

  - Korean Air thanks to all IPACG members for making UPRs progress this far.

3.2     The meeting is requested to note the KAL’s Oceanic UPR trial result and to consider Incheon, Korea

to be added to the list of approved UPR airports for this Oceania UPR Trial.

(End)