THE THIRTY-FIFTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/35)

(Sapporo, Hokkaido, Japan, 7-11 November 2011)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

RNP4 Benefit on PACOTS Route

(Presented by ENRI)

SUMMARY

RNP4 reduces Oceanic separation. However, RNP4 equipped aircraft must increase position reports by ADS downlink. It is desired to clarify the cost and benefit of RNP4.

ENRI examined benefit by proportion of RNP4 equipped aircraft.

- Introduction

  - RNP4 reduces oceanic separation from 50NM to 30NM. It could reduce separation further by CDP or ITP. Currently, proportion of RNP4 equipped  aircraft remains about 30 %, benefit of RNP4 is not shown sufficiently. When both aircraft of aligned flight  pair have RNP4 equipped, separation can be reduced.

  - RNP4 operation needs to reduce ADS downlink interval from 16 minutes to 10 minutes for safety. Frequent downlink messages increase communication cost. Clarification of cost and benefit of RNP4 is requested by airlines.

  - At IPACG/34, proportion of RNP4 equipped aircraft will potentially increase up to 44%, as FANS 1/A aircraft includes RNP4 capability. ENRI conducted simulation of PACOTS flight for several cases of different proportion of RNP4 equipped aircraft and traffic volume.

- Discussion

  - ENRI examined benefit for cases of increased proportion of RNP4 equipped aircraft in current traffic volume. Four types of scenarios are  assumed. RNP_A, RNP_B, RNP_C and RNP_D show proportion of RNP4 equipped aircraft of 30% (Current), 40%, 60% and 75% respectively.

- Fig 1 shows  the number of flight which have to change altitude to maintain ATC separation with other traffic. R591 is the highest volume route on the day. “ALL” shows fifth type of scenario in which all aircraft are RNP4 equipped.

Fig1. The number of  flight ( Altitude changed)

- On the route of low traffic volume, benefit is small. On the route of high traffic volume, results of the simulation are shown as follows:

- The altitude difference from original altitude  is reduced. Especially, number of flights  with very low altitude is reduced.

- RNP4 benefit is limited for most high traffic volume routes in current traffic volume.

- 4～6　flights/30min. is border.

- RNP_C (60%) shows the highest benefit in four scenarios.

  - For high traffic volume routes, increases traffic volume scenarios are assumed. Fig 2 shows flight profile samples of those scenarios. For example, “13A” shows the case in which traffic volume is 1.3 times higher with “RNP_A” scenario. Increase of the proportion of RNP4 equipped aircraft keeps more economical altitude in spite of traffic increased. 11A and 11C are the same as 12C. And 10C is the same as 10A. So those case are not indicated in Fig 2.

140E         170E         160W         130W        100W

Fig 2　Flight profile sample for different traffic volume and proportion of RNP4 equipped aircraft

  - Currently, the proportion of RNP4 equipped aircraft is about only 30%. Increase of this proportion keeps more economical altitude in spite of traffic increase.

- Conclusion

  - The meeting is invited to note the information provided.