THE THIRTY-FIFTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/35)

(Sapporo, Hokkaido, Japan, 7-11 November 2011)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

PACOTS Track 1 User Preferred Route (UPR) Trial

(Presented by Civil Aviation Bureau, Japan (JCAB) and the Federal Aviation Administration)

SUMMARY

This information paper provides an update on the PACOTS track 1 UPR operational trial.

- Introduction

  - An operational trial of Pacific Organized Track System (PACOTS) Track 1 UPR usage began on 30 June 2011.  A number of operational requirements must be used by the operators when flight planning PACOTS Track 1 UPRs, due to the complexity of the airspace.

- Discussion

  - The two-part paper trial which was conducted in June and November 2010 indicated that impressive annual fuel savings of approximately 10 million kilograms could be realized, even with the necessary operational constraints.  The associated reduction in carbon dioxide (CO2) emissions would be approximately 31.6 million kilograms.

  - The filing requirements for utilizing a PACOTS Track 1 UPR are as follow:

a.	The UPR must remain in the Fukuoka, Anchorage and Oakland Oceanic control areas.

b.	The UPR must utilize a published standard departure routing where applicable.

c.	The UPR must be planned to avoid military special use airspace when active.

d.	When transiting through Fukuoka Flight Information Region (FIR), operators must file one of the following Oceanic Transition Routings:

PEXEL A590 POWAL

ONION OTR5 ADNAP R591 ADGOR

ONION OTR5 KALNA

ONION OTR5 ADNAP OTR7 EMRON

AVBET OTR9 EMRON

AVBET OTR11 LEPKI

e.	When transiting through Anchorage FIR, UPR flights must remain on or south of A590 and use one of the following transition routes:

A590 transition route is PASRO A590 POWAL

R591 transition route is AKISU R591 ASPIN

G344 transition route:

CUTEE G344 CARTO

CUTEE 48N170E 49N180E (or point north of 49N180E

CUTEE 49N170E

CUTEE 50N170E

f.	When transiting through Vancouver UTA/FIR flights must use one of the following transition routes:

To CYVR:

KATCH:	YZP POWOL KEINN CYVR

MANJO:	NAKBI EPTIV POWOL CYVR

NUDGE:	PETPA ITGAV POWOL CYVR

ORNAI:	SIMLU KEPKO YAZ FOCHE CYVR

PRETY:	TAMRU YAZ FOCHE CYVR

ROUKE	VIDKU YAZ FOCHE CYVR

To KSEA/KPDX:

KATCH:		UDMAP TOU JAWBN KSEA

UDMAP TOU KEIKO KPDX

MANJO NAKBI EPTIV:	TOU JAWBN KSEA

TOU KEIKO KPDX

NUDGE PETPA ITGAV:	TOU JAWBN KSEA

TOU KEIKO KPDX

ORNAI SIMLU KEPKO:	TOU JAWBN KSEA

TOU KEIKO KPDX

PRETY TAMRU:	TOU JAWBN KSEA

TOU KEIKO KPDX

ROUKE VIDKU:	DOLFF TOU JAWBN KSEA

DOLFF TOU KEIKO KPDX

WIDEN HONSO HQM: 	OLM KSEA

KEIKO KPDX

Transiting aircraft:

KATCH YZP or YZT, UPR to destination

MANJO NAKBI EPTIV, UPR to destination

NUDGE PETPA ITGAV, UPR to destination

ORNAI SIMLU KEPKO, UPR to destination

PRETY TAMRU SEFIX, UPR to destination

ROUKE VIDKU DOLFF TOU, UPR to destination

WIDEN HONSO HQM, UPR to destination

g.	The UPRs must remain 50NM north of PACOTS Track 2

  - Some aircraft operators initially created some problems when filing PACOTS Track 1 UPRs.  Routes were flight planned with multiple crossings of the Oakland/Anchorage FIR boundary.  However, after some discussions with the involved operators, the trial appears to be running smoothly.

- Conclusion

  - The meeting is encouraged to note the information provided.