# THE THIRTY-FIFTH MEETING OF THE
IFORMAL PACIFIC ATC CO-ORDINATING GROUP
(IPACG/35)

# THE TWENTY-SECOND MEETING OF THE IPACG FANS INTEROPERABILITY TEAM
(IPACG FIT/22)

# (Sapporo, Hokkaido, Japan, 8 November 2011)

Agenda Item 3:	ANSP GOLD Performance Analysis Review

CPDLC and ADS-C Performance for FANS 1/A over HFDL (FOH)

(Presented by the FAA)

Summary

This paper provides observed performance of the HF data link operations conducted within Oakland FIR as part of the FANS 1/A Over HFDL (FOH) evaluation project. The performance is measured against the criteria specified in the Global Operational Data Link Document (GOLD).  This analysis includes performance of the Controller Pilot Data Link Communication (CPDLC) and Automatic Dependent Surveillance – Contract (ADS-C) of HF data link operations.

# Introduction

## This paper provides observed performance of the HF data link operations conducted within Oakland FIR as part of the FANS 1/A over HFDL (FOH) evaluation project. The purpose is to aid in the evaluation of the viability of FOH for some oceanic data link operations.

## The performance data observed from the Controller Pilot Data Link Communication (CPDLC) and Automatic Dependent Surveillance - Contract (ADS-C) systems are measured against the appropriate Required Communication Performance (RCP) and Required Surveillance Performance (RSP) specification to demonstrate that safety objectives which rely on the communications infrastructure can be met by the aircraft and ground systems.

## This paper presents the HF data link performance by month for the Hawaiian Airlines (HAL) fleet of B763 participating in the evaluation.  In addition, Attachment A contains ADS-C performance details by individual airframe.

# DISCUSSION

## The Global Operational Data Link Document (GOLD) provides the guidance material describing the required data points from the FANS 1/A aircraft communications addressing and reporting system (ACARS) messages.  The GOLD also describes the calculation process for the actual communication performance (ACP), actual communication technical performance (ACTP), pilot operational response time (PORT), and surveillance latency.

### Figure 1 presents the ACTP measurement for the HF data link messages sent for the fleet of HAL B763 within the Oakland FIR by month over the last six months.  The numbers of CPDLC messages included in the analysis are shown in the legend of Figure 1.  The ACTP data show that the 95 percent criteria for RCP400 was met in four of the past six months.

Figure 1.	ACTP (RCP 400) – FOH Data Link Performance by Month

### Figure 2 and Figure 3 present the ACP and ADS-C measurements, respectively, for HF data link messages sent for operator HAL fleet of B763 aircraft within the Oakland FIR by month.  The numbers of CPDLC and ADS-C messages used for each measurement are shown in the legend key of the figure.

### Figure 2 shows that the ACTP for the HF data link messages met the 95 criteria for RCP400 in two of the six months presented.  Figure 3 shows that the ADS-C (RSP 400) criteria were met in one of the six months presented.

Figure 2.	ACP (RCP 400) – FOH Data Link Performance by Month

Figure 3.	ADS-C (RSP 400) – FOH Data Link Performance by Month

## Attachment A contains detailed ADS-C performance data by airframe.  These data show that during the aggregate time period from July to September 2011, seven of the seventeen HAL-B763 airframes met the 95 percent ADS-C (RSP 400) criteria.

# Action by the Meeting

## The meeting is invited to:

- Note the information in the paper; and

- Review and comment on the observed performance.

Attachment A.	CPDLC and ADS-C Performance for FANS 1/A over HFDL (FOH)

[Provided as a separate file]

-END-