# THE THIRTY-FIFTH MEETING OF THE
IFORMAL PACIFIC ATC CO-ORDINATING GROUP
(IPACG/35)

# THE TWENTY-SECOND MEETING OF THE IPACG FANS INTEROPERABILITY TEAM
(IPACG FIT/22)

# (Sapporo, Hokkaido, Japan, 8 November 2011)

Agenda Item 3:	ANSP GOLD Performance Analysis Review

CPDLC and ADS-C Performance in the Oakland and Anchorage FIRs

(Presented by the FAA)

Summary

This paper provides observed performance measures as specified in the Global Operational Data Link Document (GOLD) from the operational data collected in Oakland and Anchorage Oceanic airspaces.  This analysis includes performance of the Controller Pilot Data Link Communication (CPDLC) and Automatic Dependent Surveillance – Contract (ADS-C) systems.

# Introduction

## This paper provides observed performance measures from the operational data link systems at Oakland and Anchorage Oceanic Centers.  The purpose of this paper is to present the most recent observed performance of the data link system.

## The performance data observed from the Controller Pilot Data Link Communication (CPDLC) and Automatic Dependent Surveillance - Contract (ADS-C) systems are measured against the appropriate Required Communication Performance (RCP) and Required Surveillance Performance (RSP) specifications to demonstrate that safety objectives which rely on the communications infrastructure can be met by the aircraft and ground systems in the respective airspace.

## This paper presents the data link performance in the Oakland and Anchorage Oceanic airspaces. Performance analysis is presented by media type and by ground earth station (GES). Additionally, a summary of the data link performance by operator is provided.  Attachment A contains additional charts related to the data link performance in Oakland and Anchorage Oceanic airspaces as well as similar analyses for New York Oceanic airspace.

# DISCUSSION

## The Global Operational Data Link Document (GOLD) provides the guidance material describing the required data points from the FANS 1/A aircraft communications addressing and reporting system (ACARS) messages.  The GOLD also describes the calculation process for the actual communication performance (ACP), actual communication technical performance (ACTP), pilot operational response time (PORT), and surveillance latency and specifies the requirements for each performance measure at the 95% and 99.9% levels.

## According to the guidance in the GOLD, the ACP, ACTP and PORT for applicable CPDLC transactions are required to meet RCP240 criteria when sent via satellite and VHF, and are required to meet RCP400 criteria when sent via HF. Similarly, the ADS-C downlink latency is required to meet RSP180 criteria for ADS-C downlink messages sent via satellite and VHF, and is required to meet RSP400 criteria when sent via HF.

## Table 1 outlines the requirements for the performance measures described above at the 95% and 99.9% levels.

Table 1.	Summary of Performance requirements

| Performance Measure | Percent of Messages Required to Meet Criteria | RSP180 Criteria (sec) | RSP400 Criteria (sec) | RCP240 Criteria (sec) | RCP400 Criteria (sec) |
| --- | --- | --- | --- | --- | --- |
| ADS-C Latency | 95.0% | 90 | 300 | -- | -- |
|  | 99.9% | 180 | 400 | -- | -- |
| ACTP | 95.0% | -- | -- | 120 | 260 |
|  | 99.9% | -- | -- | 150 | 310 |
| ACP | 95.0% | -- | -- | 180 | 320 |
|  | 99.9% | -- | -- | 210 | 370 |
| PORT | 95.0% | -- | -- | 60 | 60 |

## Observed Data Link Performance by Media Type

### Figure 1, Figure 2, and Figure 3 present the ACTP, ACP and ADS-C performance by media type for the aggregate period from April to September 2011 for the Oakland FIR. The numbers of messages observed during each month are shown in the legend key of each figure.

### Only the CPDLC transactions in which the same media type was used for both the uplink and downlink portion of the transaction are included in these counts. Approximately 0.6% of the CPDLC transactions in Oakland oceanic airspace during the period from April to September 2011 occurred using mixed media.

### The 95 percent criteria for RSP180 ADS-C and RCP240 ACTP and ACP were easily met for both satellite and VHF. In addition, the 99.9 percent criteria for RCP240 ACTP is met for VHF. The 95% criteria for RCP400 ACP and ACTP and the 95% criteria for RSP400 ADS-C were not met for HF during this period.

Figure 1.	ACTP – Oakland FIR by Media Type

Figure 2.	ACP – Oakland FIR by Media Type

Figure 3.	ADS-C Downlink Latency – Oakland FIR by Media Type

### Figure 14, Figure 25, and Figure 36 present the ACTP, ACP and ADS-C performance by media type for the aggregate period from April to September 2011 for the Anchorage FIR.

### Only the CPDLC transactions in which the same media type was used for both the uplink and downlink portion of the transaction are included in these counts. Approximately 1.6% of the CPDLC transactions in Anchorage oceanic airspace during the period from April to September 2011 occurred using mixed media.

### The 95 percent criteria for RSP180 ADS-C and RCP240 ACTP and ACP were easily met for both satellite and VHF. In addition, the 99.9 percent criteria for RCP240 ACTP is met for VHF. The 95% criteria of 300 seconds for RSP400 ADS-C was not met for HF during this period and please note there were only 5 CPDLC transactions via HF applicable to the RCP analysis during the period from April to September 2011.

Figure 4.	ACTP – Anchorage FIR by Media Type

Figure 5.	ACP – Anchorage FIR by Media Type

Figure 6.	ADS-C Downlink Latency – Anchorage FIR by Media Type

## Observed Data Link Performance by GES

### Figure 7 and Figure 8 show the ADS-C data link performance by ground earth station (GES) designator for all satellite GES observed during the aggregate period from July to September 2011 in the Oakland and Anchorage FIRs. Table 2 provides a summary of the GES designators that appear in this chart and the associated locations and service providers.  The GES designators shaded in gray are provided for reference only and do not appear in the chart.

Table 2.	GES designators

| Satellite | GES Location | SITA | ARINC |
| --- | --- | --- | --- |
| Inmarsat I-3 | Aussaguel, France: | AOW2 AOE2 |  |
|  | Eik, Norway: | AOW3 AOE3 IOR5 | XXE |
|  | Perth, Australia: | POR1 IOR2 |  |
|  | Santa Paula, CA, US: | POR4 | XXC |
| Inmarsat I-4 | Fucino, Italy | EUA1 | XXF |
|  | Paumalu, HI, US | APK1 AME1 | XXH |
| Iridium | Phoenix, AZ, US | IGW1 | IG1 |

Figure 7.	ADS-C Downlink Latency – Oakland FIR by GES – Satellite Only

Figure 8.	ADS-C Downlink Latency – Anchorage FIR by GES – Satellite Only

### All GES meet RSP180 95% criteria in Oakland except for XXE. All GES meet RSP180 95% criteria in Anchorage except for IGW1, XXE and AME1. However, please note that XXE and AME1 have very small message counts in Anchorage FIR.

### Performance for the two main Pacific GES, XXC and POR1, is noticeably better in Oakland FIR than in Anchorage FIR.

## Observed Data Link Performance by Operator

### There are 22 operators that contribute the top 90% of the ADS-C downlink messages in Oakland FIR. All of these operators meet the 95% criteria for RSP180 ADS-C and RCP240 ACTP and ACP. Four of the 22 operators meet the 99.9% criteria for RSP180 ADS-C, 7 of the 22 operators meet the 99.9% criteria for RCP240 ACTP, and 7 of the 22 operators meet the 99.9% criteria for RCP240 ACP.

### There are 15 operators that contribute the top 90% of the ADS-C downlink messages in Anchorage FIR. All of these operators meet the 95% criteria for RSP180 ADS-C and RCP240 ACTP and ACP. Two of the 15 operators meet the 99.9% criteria for RSP180 ADS-C, 4 of the 15 operators meet the 99.9% criteria for RCP240 ACTP, and 5 of the 15 operators meet the 99.9% criteria for RCP240 ACP.

## Attachment A contains detailed further details of the analyses presented in this paper as well as similar analyses for the New York FIR for the purpose of comparison.

# Action by the Meeting

## The meeting is invited to:

- Note the information in the paper; and

- Review and comment on the observed performance.

Attachment A.	CPDLC and ADS-C Datalink Performance Monitoring

[Provided as a separate file]

-END-