THE THIRTY-FIFTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/35)

(Sapporo, Hokkaido, Japan, 7-11 November 2011)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Automatic Dependent Surveillance – Broadcast (ADS-B) In-Trail Procedures (ITP) Operational Flight Trial Update

(Presented by the Federal Aviation Administration)

SUMMARY

The purpose of this information paper is to present an update on the ADS-B In-Trail Procedures (ITP) operational trial in the South Pacific.

- Introduction

  - On 15 August 2011, a one-year operational trial of the ADS-B ITP began in Oakland Oceanic Control Area (OCA).  During the trial, specially trained United Airlines flight crews will utilize ADS-B In avionics to determine whether a climb or descent can be accomplished with reference to aircraft broadcasting ADS-B signals.  When the correct parameters exist, the pilot will initiate a request for an ITP climb/descent with reference to the call sign of ADS-B traffic.

- Background

  - ADS-B ITP has been under development over many years.  The work supporting the development of ITP has ranged from batch simulations to human-in-the-loop experiments and has included avionics and separation standards development.

  - In 2008, the Federal Aviation Administration (FAA) Surveillance and Broadcast Services (SBS) program office established a project for the purpose of performing an operational trial of ADS-B ITP in revenue service.  The objectives of the project were to:

    - Validate the operational performance and economic benefits of ITP; and

    - Develop and validate ADS-B ITP Minimum Operational Performance Specifications (MOPS) material.

  - As a part of this project, the FAA established agreements with United Airlines and Honeywell.  The agreements include the work necessary for the development, certification and installation of onboard systems for 12 United Airlines Boeing 747-400 planes.  These onboard systems calculate the ADS-B ITP criteria and display that information to the pilot.  Honeywell is primarily responsible for the development of the traffic computer and traffic computer software, as well as the ITP display software.  Goodrich is responsible for the Electronic Flight Bag (EFB) hardware and operating system software.  United is responsible for overall coordination and installation of ITP equipment and conducting the flight trial.

  - The FAA has developed controller procedures and safety analyses that are required to support the flight trial.  The controller procedures were developed and tested using the FAA’s Dynamic Simulation (DYSIM) system.  The safety analyses were developed following the FAA’s Safety Management System.

- Description of ITP

  - ADS-B ITP is comprised of a set of six flight level change geometries with each geometry dictated by whether the ITP aircraft desires to climb or descend and its proximate relationship with the other aircraft:

• Leading climb 		• Leading descent

• Following climb 		• Following descent

• Combined climb		• Combined descent

While there is no limit of the total climb authorized in the ADS-B ITP flight level change, the other aircraft cannot be more than 2,000 feet above or below the ADS-B ITP aircraft’s altitude.

  - For ADS-B ITP, the maneuvering (trailing or leading) aircraft obtains the flight identification (ID), altitude, position and ground speed transmitted by proximate ADS-B equipped non-maneuvering (leading or trailing) aircraft.  Based on the ADS-B data from the non-maneuvering, or reference aircraft, a pilot can request an ITP altitude change to air traffic control (ATC).  The controller verifies that the ITP and reference aircraft are same track and that the maximum closing Mach differential is less than or equal to a Mach Number of 0.06. If the controller determines that the requesting aircraft will maintain standard separation minima with all aircraft other than the ITP reference aircraft, a clearance for the climb or descent may be issued.  After re-validating that the ITP initiation criteria are still valid, the maneuvering aircraft may then vertically transition through the altitude of the non-maneuvering aircraft.

  - To perform these procedures, the aircraft desiring to climb or descend must be equipped with an ADS-B transceiver and an appropriate onboard decision support system, both of which would have to be certified for this application.  Aircraft operators choosing to equip in this manner would be able to take advantage of this procedure when operating in proximity to aircraft equipped with a suitable ADS-B transmitter (“ADS-B Out”).

  - Aircraft operators who choose to equip with an ADS-B transceiver and on-board automation will benefit through the ability to perform in-trail maneuvers to achieve more time at optimum altitudes.  This could result in more efficient and predictable flight profiles, thereby saving fuel and, in some cases, allowing operators to make operational decisions such as the opportunity to carry additional high value payload in lieu of additional contingency fuel.  Aircraft operators have also indicated there may be other potential benefits associated with increased cockpit situational awareness resulting from ADS-B In traffic displays.

  - A summary of the ITP criteria are as follows:

      - Maximum of two reference (target) aircraft; +/- 2000 feet from ITP aircraft altitude

      - Reference aircraft can be any combination of ahead of or behind the ITP aircraft

      - ITP aircraft can climb or descend at no less than 300 feet per minute

      - Initiated with no closer than 15 NM and no more than 20 knots of closure

      - The closing Mach number difference must be less than or equal to Mach Number of 0.06

      - ITP aircraft must maintain Mach number in climb

      - Reference aircraft must be level and non-maneuvering

      - ITP aircraft must have certified ITP equipment onboard as well as CPDLC

      - Reference aircraft must have valid ADS-B Out signal

- ADS-B ITP Operational Trial

  - As stated previously, the FAA has begun an operational trial of ADS-B ITP in revenue service.  Details of the operational flight trial are outlined below.

    - Scope:

- The initial trial utilizes equipped United Airlines Boeing 747-400 planes operating between the US West Coast and Australia and on return flights.  The flights are regularly scheduled passenger revenue flights, not test flights.  Expansion to other portions of Oakland OCA may occur.

- Data collection activity is being conducted by both United Airlines and Oakland Air Route Traffic Control Center.  Any significant adverse operational issues that are discovered (such as communication or workload) will result in an immediate suspension of all operational evaluation activity.

- The data collected will be used to enhance the understanding of the economic and operational impact.

- Regular line flight crews and air traffic controllers are being used.  All pilots and controllers that are authorized to participate have completed approved training.

- Initial expectations are a maximum of one or two ITP procedures will be performed per week of operations.

- The operational trial flights are being conducted for a period of one year.

    - This operational trial has additionally been adopted as an initiative within the ASPIRE program. The initiative establishes a cooperative agreement between the FAA, Airservices Australia and Airways Corp New Zealand which will allow the organizations to share data and provide a mechanism for providing mutual support of the operational trial.

    - Objectives:

- Validate that air traffic controllers applying ADS-B ITP find it a useful tool.

- With data collected from United Airlines, validate the impact on contingency fuel loading by dispatch and flight crews when they are aware that the aircraft will be ITP capable.

- From post flight surveys, determine the significance and value of increased situation awareness for the flight crew.

- Determine how often the procedure is requested, how often it is cleared.

- Determine a measurable reduction in fuel burn on flights with the combined benefit of situation awareness and ITP climbs.

f.	Validate SPR assumptions and gather data to support validation of a MOPS currently under development for ADS-B ITP.

- Operational Evaluation Preliminary Results

  - A comprehensive data collection and analysis process was developed for the operational evaluation. This data will be used to validate operational performance and economic benefits of ITP, validate safety requirements and assumptions and monitor operational hazards that could lead to suspension of the operational flight trial. There are seven different types of data being collected during the operational flight evaluation. The seven type of data are:

    - United Airlines computer data – supports analysis of the economic benefits of ITP

    - Dispatch comment sheet – captures comments from dispatchers about fuel planning and consumption from pre-flight planning to arrival

    - Computer Pilot Data Link Communications (CPDLC) messages – captures ITP events and related messages

    - Automatic Dependent Surveillance – Contract (ADS-C) position reports – capture position information of aircraft involved in an ITP event

    - Controller comment sheet – provides an opportunity for controllers to provide feedback on ITP events

    - Pilot comment sheets – capture pilots comments on subjects including fuel planning, situation awareness, operational impacts and comments on human factors aspects of ITP

    - Pilot data sheet – capture information about the change of ITP distance during an ITP event and the duration of the flight level change

  - This data is collected, analyzed and used to address key higher level metrics and hazard tracking. Attachment A includes a table of the key metrics that are being tracked the summary of results from ITPs performed in August 2011.

  - The ITP data contained in the tables involved two ITP equipped United Boeing 747s flying on the same day in the same airspace (one flight operated from LAX-SYD flight and the other flight from SFO-SYD). Most of the ITPs performed that evening were between the two United Airlines aircraft. They were valid operational ITPs performed in non-radar airspace applying the ITP separation standard, but United Airlines intentionally conducted extra climbs and descents to allow the partners an opportunity to gather additional data on ITP.  United Airlines also routed the aircraft so the flight paths would facilitate ITP events.  As a result, the business case numbers were not valid for these series of flights.  All other data is valid and provides insight into the application of ADS-B ITP in the South Pacific (SOPAC) airspace.

- Summary

  - The FAA began an operational evaluation of ADS-B ITP along SOPAC routes in mid 2011.  The FAA established agreements that led to the development and certification of onboard systems that provide the ADS-B ITP criteria and display that information to the pilot. The FAA has also performed all required safety management system processes and analyses.

  - For additional information on the operational evaluation, please contact Mr. Ken Jones at Kenneth.M.Jones@nasa.gov or +1 (757) 864-5013.

- Conclusion

  - The meeting is invited to note the information presented in this paper.

ATTACHMENT A  -  Preliminary Operational Evaluation Results

Operational and Economic Metrics

Safety related metrics

Note: Wake Turbulence Incident Severity of 5 is least severe