# THE THIRTY-FIFTH MEETING OF THE
IFORMAL PACIFIC ATC CO-ORDINATING GROUP
(IPACG/35)

# THE TWENTY-SECOND MEETING OF THE IPACG FANS INTEROPERABILITY TEAM
(IPACG FIT/22)

# (Sapporo, Hokkaido, Japan, 8 November 2011)

Agenda Item 5:	Working Papers

RCP and RSP Planning

(Presented by the FAA)

Summary

This paper proposes that IPACG begin the planning and implementation of a performance-based framework for communications and surveillance within the Central and North Pacific sub-regions.  The framework will complement the existing performance-based navigation framework.  The performance-based communications and surveillance framework will initially apply performance specifications and monitoring per the Global Operational Data Link Document (GOLD) to FANS1/A controller-pilot data link communications (CPDLC) and automatic dependent surveillance contract (ADS-C).

# Introduction

## While the Asia/Pacific Region has an implementation plan for performance-based navigation no regional plan exists for the implementation of performance-based communication and surveillance.

## A number of States in the Region states have reduced separations that are predicated on certain communication, surveillance, and navigation requirements.  With an increasing proliferation of different aircraft FANS 1/A systems, and given current ground system implementations, it is possible that separations may be misapplied because the qualification criteria for communication and surveillance are not formally applied within the Asia/Pacific Region.

# DISCUSSION

## At IPACG/29 FIT/16, the FAA presented a performance-based specification template (Refer to IPACG/29 FIT/16 – WP/03).  The FIT agreed with the concept of developing a template and performance specifications for global use in specifying criteria for communications and surveillance.

## At IPACG/29 FIT/16, the FAA presented performance monitoring by the FAA and Airways New Zealand (Refer to IPACG/29 FIT/16 – WP/04, WP-07 and IP-06).  The FIT agreed to facilitate an already on-going shared effort at data collection, and to anticipate further clarification from the GOLD Ad Hoc Working Group as to specifically what data is to be analyzed.  The FIT felt that the development of an implementation plan was a task best left to the GOLD Ad Hoc Working Group which was representative of multiple regions.

## At IPACG/29 FIT/16, the FAA presented the status of the GOLD (Refer to IPACG/29 FIT/16 – WP/06).  The FIT recommended to IPACG to endorse the Global Operational Data Link Document (GOLD) that will supersede the existing FANS Operation Manual (FOM).  IPACG formally endorsed the GOLD and any future activities.  At IPACG/31, IPACG agreed that the FIT would manage the GOLD implementation and report to the IPACG Plenary.

## The GOLD Ad-Hoc Working Group accomplished its initial task to produce the GOLD, 1st Edition, in June 2010, under the auspices of the North Atlantic Systems Planning Group (NAT SPG) and Asia-Pacific Air Navigation Planning and Implementation Regional Groups (APANPIRG).  In that same month, the NAT SPG and APANPIRG officially adopted the GOLD, which now supersedes region-specific documents – Guidance Material for ATS Data Link Services in North Atlantic Airspace and the FANS 1/A Operations Manual used throughout Asia-Pacific Regions, and parts of the South American and African Indian Ocean Regions.

## The GOLD provides required communication performance (RCP) and surveillance performance specifications (RSP), and guidelines on prescribing specifications to apply to communications and surveillance capabilities in airspace where reduced separations are predicated on certain communication, navigation and surveillance performance requirements.  The GOLD also provides guidelines to States to aid in qualifying components of the operational system to the criteria prescribed by the performance specifications.

## As implementation of reduced separations, in trail and climb/descent procedures, reroutes and other operational capabilities becomes more widespread, the regions of the world will need to plan for and implement a global performance based framework for communications and surveillance.  For example:

- In June 2011, the North Atlantic Region System Planning Group endorsed its RCP and ADS-C Surveillance Performance Based Operations Implementation Plan, which is consistent with GOLD guidelines.  The plan (See Attachment A) is being executed within the NAT Region with an effective implementation date of February 2015.

- In March 2011, the FIT recommended that ISPACG support the development of a performance-based framework for communications and surveillance within the region.  ISPACG agreed with FIT recommendation and encouraged all ISPACG ANSP members to implement system performance monitoring as outlined in the GOLD as an element of their respective SMS programs.

## The performance-based framework is intended to fulfill a States obligation to ensure that the operational system meets requirements for intended uses.  Furthermore, the framework will enable optimum use of technologies, such as Iridium and SwiftBroadband, and implementations, such as I4 Classic Aero and Iridium Next.

## To implement a performance-based communications and surveillance framework, changes will be needed to:

- Type design approval of aircraft, as necessary;

- Master Minimum Equipment List (MMEL) policies;

- Related operational authorizations;

- Regional SUPPs (ICAO Doc 7030 Amendments) and AIPs (or equivalent);

- Flight plan requirements; and

- ATC automation to act appropriately based on communication and surveillance equipment and capability indicators provided in the flight plan.

## It is expected that the performance-based framework would eventually apply to satellite communications (SATCOM) voice for air traffic control per the SATCOM Voice Guidance Material, currently under development by the Inter-Regional SATCOM Voice Task Force.

## A regional – but globally compatible – plan is needed throughout the Asia-Pacific Regions for the implementation of performance-based framework for communication and surveillance.

# Action by the Meeting

## The IPACG FIT is invited to:

- Note the information provided in this paper;

- Cooperate with other regions and sub-regions within the Asia-Pacific Regions; and

- Recommend to IPACG to include in its work program a planning and implementation initiative to effectively implement a performance based-framework in the Central and North Pacific sub-Region.

Attachment A.	NAT RCP and ADS-C Surveillance Performance Based
Operations Implementation Plan

Associated with the NAT data link services in support of RLongSM and RLatSM.

| # | TASKS | COMPLETE BY | STATUS | LEAD | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | GENERAL PROJECT DEVELOPMENT & MANAGEMENT |  |  |  |  |
| 1 | Prepare a draft RCP and ADS-C Surveillance Performance-Based Operations Plan outlining the way forward for consideration by the NAT IMG | NAT IMG/38 and NAT SPG/47 | Approved by NAT SPG/47. | NAT CNSG |  |
| 2 | Identify Key Target Dates on implementing RCP and ADS-C surveillance performance framework and prescribing specifications to support RLongSM, and RLatSM. | NAT IMG/38 and NAT SPG/47 | Pending target dates for associated operations. | NAT IMG | NAT SPG Conclusion 44/11 targets 2015. Target dates for RCP/ADS-C performance specifications need to be in combination with the target dates for RLongSM and RLatSM operational implementation. |
| 3 | Confirm applicable performance specifications that will be used for operational implementation of data link services in support of RLatSM and RLongSM. Detail and validate CRM assumptions against actual performance measurements in accordance with GOLD. | NAT SPG/46 for RLatSM and RLongSM. | Approved by NAT SPG/47 | NAT SARSIG | During trials of RLongSM and RLatSM, specifications are not prescribed, but will provide guidelines against which the actual performance is measured. RCP 240 and surveillance performance 180 are the candidate’s specifications to be prescribed for RLatSM and RLongSM operations. Note: When performance falls below specified levels, operational judgment may be a consideration in determining appropriate actions. |
| 4 | Update operational concepts for implementation of RLatSM, RLongSM supported by associated RCP and surveillance performance specifications. Develop operational concept (of use), including procedures, for data link services using CPDLC and ADS-C to reduce the number and exposure of operational errors and pilot deviations, regardless of whether or not reduced ADS-C based separations are applied. For example, concept of use should detail conformance monitoring, intervention and route re-clearances. Review and comment on material for incorporation in GOLD, NAT Doc 006 and Doc 007. | NAT IMG/40 and NAT SPG/48 | Draft and review through end 2012. | NAT ATMG in coordination with CNSG | ADS-C and CPDLC operational concepts are complete. Review and propose text to be candidate for GOLD amendment. |
|  | DOCUMENTATION |  |  |  |  |
| 5 | Development of the GOLD material in support of reduced longitudinal: -the provisions for data link service (AIC, guidance for AIPs, eligibility requirements etc) -performance specifications -initial qualifications for operations of operators, aircraft and ATC -post implementation monitoring | NAT SPG/46 | Complete | GOLD ad-hoc group | Amendments to GOLD are in work for tasks 3, 5, 6 and 7. Adoption planned at NAT SPG/49. |
| 6 | Develop flight crew and controller contingency procedures in the event of service outage, malfunction or failure that would cause performance to degrade below that required by specifications. Review and comment on material for incorporation in GOLD, NAT Doc 006 and Doc 007. | NAT IMG/40 and NAT SPG/48 | Draft and review through end 2012. | NAT ATMG/ CNSG | Included in NAT Doc 006. Amendments needed to ensure that long duration outages/degradations are also considered. Similar provisions should be included in the GOLD and in NAT Doc 007. |
| 7 | Develop the criteria for resuming data link service, RLatSM, or RLongSM operations after service communication and/or surveillance capabilities are restored to acceptable level of performance. Review and comment on material for incorporation in GOLD, NAT Doc 006 and Doc 007. | NAT IMG/40 and NAT SPG/48 | Draft and review through end 2012. | NAT ATMG/ CNSG | Included in NAT Doc 006. Amendments needed to ensure that long duration outages/degradations are also considered. Similar provisions should be included in the GOLD and NAT Doc 007. |
| 8 | Draft guidance material for the flight plan to define the descriptors for performance specifications, as appropriate, using the new format planned for 2012 implementation. Review and comment on material for incorporation in GOLD. | a) NAT IMG/40 and NAT SPG/48 (as part of the NAT SUPPs PfA) and GOLD b)12th Air Navigation Conference – amend Doc 4444 | Work needs to be confirmed and assigned. Draft and review through end 2012. | NAT CNSG ICAO (Global) | Definition of P descriptors in Item 10a and expansion or redefinition of descriptors for ADS-C. |
| 9 | Draft or update PfA (or revise existing drafts) to the NAT Regional Supplementary Procedures (SUPPs) (Doc 7030) to prescribe the performance specifications for communication and surveillance to support RLatSM and RLongSM. PfA should include criteria for operator eligibility, aircraft equipage, requirements for flight planning, monitoring, alerting and reporting. | NAT IMG/42 and NAT SPG/49 | Draft and review through end 2012. | NAT CNSG NAT ATMG | Dependent on timeline for RLatSM and RLongSM. PfA should be part of PfA for each operational improvement. |
| 10 | Amend AIPs and other State documents to support SUPPs amendment. | Consistent with Task 8 | Consistent with Task 8 | States | Consistent with Task 8 |
| 11 | Implement operational communications performance monitoring capability in ATC automation. | Before the start of operational trials of RLongSM or RLatSM. | Gander – completed Shanwick –completed Reykjavik –4Q/2011 Santa Maria – 2Q/2011 New York – Completed Bodo – TBD Shannon –1Q/2013 | NAT ANSPs | Should be in place prior to effective date of data link mandate and start of RLatSM or RLongSM trials. |
| 12 | Measure actual performance against specifications for feasibility, i.e., ACP, ACTP, PORT, ADS-C latency for operators and aircraft types | Prior to operational implementation | Ongoing | ANSPs/ DLMA/ CNSG/ SARSIG | Collect and analyze data in accordance with GOLD, Apx D. |
|  | AIRWORTHINESS AND OPERATIONAL ELIGIBILITY |  |  |  |  |
| 13 | Provide guidance to State regulators related to aircraft equipage and operator eligibility requirements taking into account the GOLD and appropriate RCP and surveillance performance specifications. Review and amend GOLD, if required. | NAT IMG/40 and NAT SPG/48 | Draft and review through end 2012. | OPSAIR SG | Refer to FAA AC 20-140A and AC 120-70B. Other State material may apply. |
| 14 | Develop or revise State guidance and/or regulations, as necessary. Establish State airworthiness requirements. Establish operational policy/procedures requirements for operational approval. Prepare State inspectors to perform tasks for operational approval. Develop plan to issue operational approval to national operators by [date], to extent possible. Train pilots and, if applicable, dispatchers on RCP and surveillance performance aspects of reduced separation. Develop and distribute operations manuals, pilot bulletins or other appropriate docs containing RCP and communication surveillance performance policy/procedures. | End of 2014 Prior to operational implementation of RLatSM or RLongSM | On-going Need status reports from States | SOG/ States/ ANSPs/ Users | Implementation tasks in this plan need to be completed by NAT SPG/49 (June 2013) to allow time for operational readiness to implement RCP/surveillance performance by 2015. |
|  | POST IMPLEMENTATION TASKS |  |  |  |  |
| 15 | Post-implementation monitoring, analysis and corrective action per GOLD, Apx D and any other necessary monitoring tasks. |  | On-going See related Task 11. | ANSP/ DLMA/ CNSG | When performance falls below specified levels, operational judgment may be a consideration in determining appropriate actions. |
| 16 | Develop a guidance material to clarify the interpretation of RCP and surveillance specification in terms of compliance/non-compliance. | NAT IMG/41 NAT SPG/49 | In progress | CNSG |  |
| 17 | Conduct workshops to raise awareness on RCP and surveillance performance. | NAT SPG/49 |  | ICAO/ States |  |

-END-

Plans for prescribing RCP specifications associated with SATCOM voice will be addressed, as appropriate, pending completion of the SATCOM Voice Guidance Material by the Inter-Regional SATCOM Voice Task Force.