# THE THIRTY-FIFTH MEETING OF THE
IFORMAL PACIFIC ATC CO-ORDINATING GROUP
(IPACG/35)

# THE TWENTY-SECOND MEETING OF THE IPACG FANS INTEROPERABILITY TEAM
(IPACG FIT/22)

# (Sapporo, Hokkaido, Japan, 8 November 2011)

Agenda Item 4:	Working Papers

CPDLC Message Element Usage

(Presented by the FAA)

Summary

This paper provides a summary of the CPDLC message elements used in each of the United States flight information regions (FIRs).  It proposes that the GOLD be amended to collect this type of information to support performance analysis and development of standards for next generation CPDLC implementations.

# Introduction

## One of the current objectives of the ICAO OPLINK Panel is to harmonize and optimize the CPDLC message set. The joint SC214/ED78 group was tasked to perform as assessment of the CPDLC message set to reduce it to CPDLC messages that will be frequently, safely and operationally used. The NAT service providers were invited to provide their operational assessment on the current SC214/WG78 CPDLC message set.

## This paper provides the observed usage of the elements in the existing CPDLC message set in the New York (KZNY), Oakland (KZAK) and Anchorage (PAZA) flight information regions (FIR). The archived CPDLC data from the Ocean21 system in each of the three regions was used for this analysis. The 12-month period from July 2010 to June 2011 was examined.

# DISCUSSION

## The following analysis shows the observed usage of uplink and downlink CPDLC in the three US FIR in which data link is currently used: KZNY, KZAK and PAZA. A summary of the overall usage of uplink and downlink CPDLC in each FIR is presented, followed by the usage of the specific message elements in each FIR.

## Figure 1 and Figure 2 show the observed CPDLC message usage by month for the three US FIRs for during the period from July 2010 to June 2011 for uplink and downlink messages, respectively.

Figure 1.	Monthly Uplink CPDLC Message Usage

Figure 2.	Monthly Downlink CPDLC Message Usage

## Figure 3 shows the percent contribution of the top 20 observed uplink message elements in New York FIR during the period from July 2010 to June 2011. These 20 message elements represent approximately 97% of the total uplink CPDLC messages observed within New York FIR during the 12-month period.

Figure 3.	Uplink CPDLC Usage by Message Element – KZNY

## Figure 4 shows the percent contribution of the top 20 observed downlink message elements in New York FIR during the period from July 2010 to June 2011. These 20 message elements represent approximately 98% of the total downlink CPDLC messages observed within New York FIR during the 12-month period.

Figure 4.	Downlink CPDLC Usage by Message Element – KZNY

## Figure 5 shows the percent contribution of the top 20 observed uplink message elements in Oakland FIR during the period from July 2010 to June 2011. These 20 message elements represent approximately 98% of the total uplink CPDLC messages observed within Oakland FIR during the 12-month period.

Figure 5.	Uplink CPDLC Usage by Message Element – KZAK

## Figure 6 shows the percent contribution of the top 20 observed downlink message elements in Oakland FIR during the period from July 2010 to June 2011. These 20 message elements represent approximately 99% of the total downlink CPDLC messages observed within Oakland FIR during the 12-month period.

Figure 6.	Downlink CPDLC Usage by Message Element – KZAK

## Figure 7 shows the percent contribution of the top 20 observed uplink message elements in Anchorage FIR during the period from July 2010 to June 2011. These 20 message elements represent approximately 98% of the total uplink CPDLC messages observed within Anchorage FIR during the 12-month period.

Figure 7.	Uplink CPDLC Usage by Message Element – PAZA

## Figure 8 shows the percent contribution of the top 20 observed downlink message elements in Anchorage FIR during the period from July 2010 to June 2011. These 20 message elements represent approximately 99% of the total downlink CPDLC messages observed within Anchorage FIR during the 12-month period.

Figure 8.	Downlink CPDLC Usage by Message Element – PAZA

## Tables A1, A2 and A3 in the Attachment A show the top 20 uplink and downlink message elements observed in New York, Oakland and Anchorage FIR, respectively, during the 12-month period from July 2010 to June 2011. These tables include the message element text for each message element reference number shown in the charts.

# Action by the Meeting

## The Group is invited to:

- Note the information in this paper;

- Provide comment on the proposal to collect statistics on CPDLC usage by message element; and

- Agree on proposal to amend the GOLD, as appropriate.

Attachment A.	Commonly Used CPDLC Message Elements

Table A1.	Usage of Uplink and Downlink CPDLC by Message Element – KZNY

| Uplink Message Element | Message Reference No. | Frequency Observed | Cumulative Frequency |
| --- | --- | --- | --- |
| [facility designation] | 163 | 14.7% | 14.7% |
| END SERVICE | 161 | 12.9% | 27.6% |
| [free text] | 169 | 11.3% | 38.9% |
| REPORT MAINTAINING [level] | 129 | 10.3% | 49.3% |
| RESUME NORMAL SPEED | 117 | 5.9% | 55.2% |
| NEXT DATA AUTHORITY [facility designation] | 160 | 5.8% | 61.0% |
| CLIMB TO REACH [level] BY [time] | 26 | 5.8% | 66.7% |
| WHEN CAN YOU ACCEPT [level] | 148 | 3.9% | 70.6% |
| REPORT BACK ON ROUTE | 127 | 3.5% | 74.1% |
| CLEARED TO DEVIATE UP TO [specified distance] [direction] OF ROUTE | 82 | 3.5% | 77.6% |
| CLIMB TO [level] | 20 | 3.3% | 80.8% |
| CLEARED [route clearance] | 80 | 2.9% | 83.8% |
| MAINTAIN [speed] | 106 | 2.8% | 86.5% |
| MAINTAIN [level] | 19 | 2.8% | 89.3% |
| DUE TO [traffic type] TRAFFIC | 166 | 2.4% | 91.7% |
| REJOIN ROUTE BY [time] | 69 | 1.8% | 93.5% |
| UNABLE | 0 | 1.8% | 95.3% |
| ROGER | 3 | 0.7% | 96.0% |
| DESCEND TO REACH [level] BY [time] | 28 | 0.5% | 96.5% |
| AT [time] CLIMB TO [level] | 21 | 0.4% | 97.0% |

| Downlink Message Element | Message Reference No. | Frequency Observed | Cumulative Frequency |
| --- | --- | --- | --- |
| WILCO | 0 | 24.1% | 24.1% |
| [version number] | 73 | 15.1% | 39.2% |
| WE CAN ACCEPT [level] AT [time] | 81 | 13.4% | 52.6% |
| MAINTAINING [level] | 37 | 10.3% | 62.9% |
| [free text] | 67 | 10.0% | 72.9% |
| ROGER | 3 | 5.3% | 78.2% |
| REQUEST CLIMB TO [level] | 9 | 4.5% | 82.7% |
| REQUEST [level] | 6 | 3.7% | 86.4% |
| REQUEST WEATHER DEVIATION UP TO [specified distance] [direction] OF ROUTE | 27 | 2.7% | 89.2% |
| DUE TO AIRCRAFT PERFORMANCE | 66 | 2.0% | 91.1% |
| BACK ON ROUTE | 41 | 1.7% | 92.9% |
| POSITION REPORT [position report] | 48 | 1.4% | 94.3% |
| DUE TO WEATHER | 65 | 1.3% | 95.6% |
| ERROR [error information] | 62 | 1.1% | 96.7% |
| REQUEST [speed] | 18 | 0.5% | 97.3% |
| STANDBY | 2 | 0.4% | 97.6% |
| [facility designation] | 64 | 0.2% | 97.9% |
| WHEN CAN WE EXPECT HIGHER LEVEL | 53 | 0.2% | 98.1% |
| DEVIATING UP TO [specified distance] [direction] OF ROUTE | 80 | 0.2% | 98.3% |
| REQUEST [clearance type] CLEARANCE | 25 | 0.2% | 98.5% |

Table A2.	Usage of Uplink and Downlink CPDLC by Message Element – KZAK

| Uplink Message Element | Message Reference No. | Frequency Observed | Cumulative Frequency |
| --- | --- | --- | --- |
| REPORT MAINTAINING [level] | 129 | 16.3% | 16.3% |
| CLIMB TO [level] | 20 | 14.6% | 30.9% |
| [facility designation] | 163 | 13.6% | 44.5% |
| END SERVICE | 161 | 12.2% | 56.8% |
| RESUME NORMAL SPEED | 117 | 11.7% | 68.4% |
| [free text] | 169 | 6.1% | 74.6% |
| REPORT BACK ON ROUTE | 127 | 4.2% | 78.8% |
| CLEARED TO DEVIATE UP TO [specified distance] [direction] OF ROUTE | 82 | 4.0% | 82.8% |
| NEXT DATA AUTHORITY [facility designation] | 160 | 3.8% | 86.6% |
| WHEN CAN YOU ACCEPT [level] | 148 | 2.2% | 88.8% |
| UNABLE | 0 | 1.9% | 90.7% |
| DUE TO [traffic type] TRAFFIC | 166 | 1.9% | 92.6% |
| CLIMB TO REACH [level] BY [time] | 26 | 1.3% | 93.9% |
| MAINTAIN BLOCK [level] TO [level] | 30 | 1.1% | 95.0% |
| AT [time] CONTACT [unit name] [frequency] | 120 | 0.7% | 95.7% |
| MAINTAIN [level] | 19 | 0.7% | 96.4% |
| ROGER | 3 | 0.4% | 96.7% |
| CLEARED [route clearance] | 80 | 0.3% | 97.1% |
| REQUEST POSITION REPORT | 147 | 0.3% | 97.4% |
| STANDBY | 1 | 0.3% | 97.7% |

| Downlink Message Element | Message Reference No. | Frequency Observed | Cumulative Frequency |
| --- | --- | --- | --- |
| WILCO | 0 | 27.3% | 27.3% |
| MAINTAINING [level] | 37 | 12.4% | 39.7% |
| REQUEST CLIMB TO [level] | 9 | 11.1% | 50.8% |
| [version number] | 73 | 10.3% | 61.1% |
| WE CAN ACCEPT [level] AT [time] | 81 | 9.5% | 70.6% |
| POSITION REPORT [position report] | 48 | 7.3% | 77.9% |
| [free text] | 67 | 5.4% | 83.2% |
| ROGER | 3 | 3.3% | 86.6% |
| REQUEST WEATHER DEVIATION UP TO [specified distance] [direction] OF ROUTE | 27 | 2.8% | 89.3% |
| DUE TO AIRCRAFT PERFORMANCE | 66 | 2.5% | 91.8% |
| BACK ON ROUTE | 41 | 2.0% | 93.8% |
| DUE TO WEATHER | 65 | 1.1% | 95.0% |
| REQUEST [level] | 6 | 0.9% | 95.8% |
| REQUEST BLOCK [level] TO [level] | 7 | 0.8% | 96.7% |
| DEVIATING UP TO [specified distance] [direction] OF ROUTE | 80 | 0.7% | 97.4% |
| ERROR [error information] | 62 | 0.7% | 98.0% |
| CLIMBING TO [level] | 29 | 0.3% | 98.4% |
| REQUEST DESCENT TO [level] | 10 | 0.2% | 98.5% |
| REQUEST CRUISE CLIMB TO [level] | 8 | 0.2% | 98.7% |
| REQUEST OFFSET [specified distance] [direction] OF ROUTE | 15 | 0.2% | 98.9% |

Table A3.	Usage of Uplink and Downlink CPDLC by Message Element – PAZA

| Uplink Message Element | Message Reference No. | Frequency Observed | Cumulative Frequency |
| --- | --- | --- | --- |
| [facility designation] | 163 | 16.7% | 16.7% |
| END SERVICE | 161 | 15.1% | 31.9% |
| NEXT DATA AUTHORITY [facility designation] | 160 | 15.0% | 46.9% |
| [free text] | 169 | 10.1% | 57.0% |
| RESUME NORMAL SPEED | 117 | 9.7% | 66.6% |
| REPORT MAINTAINING [level] | 129 | 8.5% | 75.1% |
| CLIMB TO [level] | 20 | 7.2% | 82.4% |
| SQUAWK [code] | 123 | 6.3% | 88.6% |
| UNABLE | 0 | 2.2% | 90.8% |
| DUE TO [traffic type] TRAFFIC | 166 | 1.9% | 92.7% |
| REPORT [speed type] [speed type] [speed type] SPEED | 134 | 1.0% | 93.7% |
| WHEN CAN YOU ACCEPT [level] | 148 | 0.8% | 94.5% |
| CLIMB TO REACH [level] BY [time] | 26 | 0.8% | 95.3% |
| MAINTAIN [speed] | 106 | 0.5% | 95.8% |
| DO NOT EXCEED [speed] | 116 | 0.5% | 96.3% |
| STANDBY | 1 | 0.4% | 96.7% |
| ROGER | 3 | 0.4% | 97.1% |
| RADAR SERVICE TERMINATED | 154 | 0.4% | 97.5% |
| PROCEED DIRECT TO [position] | 74 | 0.3% | 97.8% |
| MONITOR [unit name] [frequency] | 121 | 0.3% | 98.1% |

| Downlink Message Element | Message Reference No. | Frequency Observed | Cumulative Frequency |
| --- | --- | --- | --- |
| WILCO | 0 | 26.5% | 26.5% |
| [version number] | 73 | 16.6% | 43.0% |
| WE CAN ACCEPT [level] AT [time] | 81 | 15.5% | 58.6% |
| REQUEST CLIMB TO [level] | 9 | 8.9% | 67.4% |
| MAINTAINING [level] | 37 | 8.6% | 76.0% |
| ROGER | 3 | 7.4% | 83.4% |
| POSITION REPORT [position report] | 48 | 6.5% | 89.9% |
| [free text] | 67 | 3.5% | 93.3% |
| DUE TO AIRCRAFT PERFORMANCE | 66 | 2.1% | 95.4% |
| PRESENT SPEED [speed] | 34 | 0.9% | 96.3% |
| ERROR [error information] | 62 | 0.8% | 97.1% |
| [facility designation] | 64 | 0.6% | 97.7% |
| DEVIATING UP TO [specified distance] [direction] OF ROUTE | 80 | 0.3% | 98.0% |
| REQUEST CRUISE CLIMB TO [level] | 8 | 0.3% | 98.3% |
| NOT CURRENT DATA AUTHORITY | 63 | 0.3% | 98.6% |
| REQUEST [level] | 6 | 0.3% | 98.9% |
| DUE TO WEATHER | 65 | 0.2% | 99.1% |
| REQUEST BLOCK [level] TO [level] | 7 | 0.1% | 99.2% |
| REQUEST VOICE CONTACT | 20 | 0.1% | 99.3% |
| REQUEST [speed] | 18 | 0.1% | 99.4% |

-END-