THE THIRTY-FIFTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/35)

(Sapporo, Hokkaido, Japan, 7-11 November 2011)

Agenda Item 5:  			Air Traffic Management (ATM) Issues

Risk Analysis of the Introduction of 10 Minutes Longitudinal Time Separation on PACOTS Routes

(Presented by Civil Aviation Bureau, Japan)

SUMMARY

This paper provides the midterm report of risk analysis of 10 minutes longitudinal time separation without mandatory application of Mach Number Technique on PACOTS routes within Fukuoka FIR.

- Introduction

  - The safety of the 10 minutes longitudinal time separation without Mach Number Technique (MNT) has already been verified. This separation is considered for implementation on oceanic routes within Fukuoka FIR on the condition that the position reports are obtained at least every 40 minutes[1]. PACOTS routes, however, are not on target yet. As a first step to the introduction of 10 minutes time separation without MNT to PACOTS routes, this paper provides the midterm report of risk analysis of PACOTS routes.

- Discussion

2.1.	A gain-loss distribution analysis is common in the safety assessment of longitudinal time separation. However, it tends to overestimate the risk of collision, so a revised method is applied.

2.2.	The gain-loss distribution is based on the distribution of the loss value. The loss value is defined as the change of the separation of two aircraft between two waypoints.  However, the gain-loss distribution does not consider the effect of the relative speed between two aircraft. If the relative speed of a pair of aircraft is obtained, the uncertainty associated with it can be eliminated from the loss distribution. The revised method takes into account the effect of the relative speed. The details can be found in Reference [2].

- 2.3.	This time, the data is obtained between May 2011 and September 2011. Flight data processing system data (FDPS) is used to extract aircraft flying PACOTS routes, and ADS message and ATO (Actual Time Over Fix) data are used to obtain the actual time at each waypoint. All PACOTS routes going east (Track 1, 2, 3, 4, 8, 11, 12, 14, and 15) are considered, and altogether 13,305 flights are observed. Among them, 3,072 pairs of aircraft applied time-based separation. In order to consider the relative speed, the data of true air speed (TAS) recorded in the FDPS data are used.

- 2.4.	It usually takes more than 40 minutes to fly between 150E and 160E waypoints, so the distribution model is constructed in this segment. Note that the data where the initial time separation is more than 1 hour is not considered in the distribution.

- 2.5	679 pairs of aircraft applied the initial time separation within 1 hour. Prior to the data collection, the ATC was asked to obtain the record of a pair of aircraft applying time-based separation within 30 minutes separation. From these, approximately 100 pairs of aircraft data were extracted. Comparing the obtained manual record and the automatically extracted data by FDPS, both records agree well, so the automatic extraction is used this time.

- 2.6	Fig. 1 shows the relationship between relative TAS and separation loss. Positive TAS indicates that two aircraft are getting closer. According to the figure, there is a correlation between the two parameters. The expected uncertainty can be decreased when the loss distribution is kept around the correlation axis. Although the TAS data recorded in FDPS is based on the flight plan which does not necessarily match the actual speed, the difference of TAS in FDPS and actual TAS is included in the loss distribution as uncertainty.

- 2.7	Fig. 2 shows the obtained loss distribution around the correlation axis and the fitting model. Fig. 3 shows the frequency of the relative speed in each range of the initial time separation. When the initial time separation is 60 minutes or less, the maximum relative TAS is 56 kt. However, when the initial time separation is 30 minutes or less, big relative TAS is observed less often. In the case of the initial time separation being 20 minutes or less, the maximum relative TAS is 17 kt. This infers that ATC tries not to apply big relative TAS when the initial time separation is small. Fig. 4 shows the relative frequency of loss value when the initial time separation is 20 minutes or less. This figure clearly shows the asymmetric distribution of loss value, where the maximum separation loss is 2 minutes while the maximum separation gain is 6 minutes. This is because of the asymmetric distribution of relative TAS. This effect is to be considered in the calculation, which leads to a more accurate result.

- 2.8	This time, only five months data is obtained, which is not enough to the calculation. Therefore, the data will be obtained for another half a year, and the calculation result will be provided at the next IPACG meeting.

Fig. 1	Relationship between relative TAS and separation loss.

Fig. 2	Relative frequency of loss value around the correlation axis and fitting model.

Fig. 3	Relative frequency of the relative true air speed in each range of the initial time separation.

Fig. 4	Relative frequency of loss value when the initial time separation is 20 minutes or less.

- Action by the meeting

  - The meeting is invited to discuss the relevance of the proposed method.

References

[1] “Application of 10 min separation minimum without MNT within Fukuoka FIR,” IP/16, IPACG/32, Honolulu, Hawaii, USA, 10-14 May, 2010.

[2] R. Mori, “Safety Assessment for Reduced Time-Based Separation Minima on Oceanic Routes”, Journal of Mechanical Systems for Transportation and Logistics, Vol. 4, No. 1 (2011), pp.39-49.