CAGE Code 81205

Loading of ATC Clearances into the Flight Management System (FMS)

| DOCUMENT NUMBER: | RELEASE/REVISION: | RELEASE/REVISION DATE: |
| --- | --- | --- |
| D6-84207 | A |  |
|  |  | For Document Release Use |

CONTENT OWNER:

Avionics – Air Traffic Management (66-CB-EA11)

All revisions to this document must be approved by the content owner before release.

Document Information

| Original Release Date |  | Contract Number (if required) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Limitations |  |  |  |  |  |  |
| Authorization for Release |  |  |  |  |  |  |
| AUTHOR: | Signature on file: Gordon R. A. Sandell |  |  | 66-ZH-S186 |  | 20 August 2009 |
|  |  |  |  | Org. Number |  | Date |
| APPROVAL: | Signature on file: Henry A. Vonjouanne |  |  | 66-ZH-S186 |  | 20 August 2009 |
|  |  |  |  | Org. Number |  | Date |
| DOCUMENT RELEASE: | Ngoc H. Bui |  |  | G-823A |  | August 20, 2009 |
|  |  |  |  | Org. Number |  | Date |

copyright © 2026 Boeing. All rights reserved.

Loading of ATC Clearances into the FMS

Table of Contents

1	Introduction	6

2	Airplane Architectures	7

3	ATC Uplink Display and Processing	9

3.1	CDU-Based System Display and Processing	9

3.2	MFD-Based System Display and Processing	10

3.3	Duplicate Waypoint Resolution	12

3.4	Multi-Element Messages	14

3.5	Route Clearance Display Presentation	14

3.5.1	MCDU-based airplanes (737, 747-400 and -8, 757 and 767	15

3.5.2	Large Format Display Airplanes (777 and 787)	15

4	Loadable Uplinks	16

4.1	Altitude Crossing Clearances	17

4.2	Time Clearances	18

4.3	Route and Route Segment Clearances	19

4.3.1	Airport and Procedure Information	22

4.3.1.1	Procedure Naming	24

4.3.1.2	Eight-Character Approaches	25

4.3.2	Route Information	25

4.3.2.1	Waypoints	25

4.3.2.2	Airways	25

4.3.3	Additional Route Information	26

4.3.3.1	Along Track Waypoints	26

4.3.3.2	Reporting Points	27

4.3.3.3	Intercept Course From	27

4.3.3.4	Hold At Waypoint	28

4.3.3.5	Waypoint Speed/Altitude	28

4.3.3.6	Required Time of Arrival (RTA)	29

4.4	Holding Pattern Clearances	30

4.5	Direct Clearances	31

4.6	Offset Clearances	32

4.7	Terminal Area Procedure Clearances	33

4.8	Combination Clearances	34

5	Additional Guidance	35

5.1	Discontinuities Inserted in the Route	35

5.2	Altitude Constraints	36

6	References	37

List of Figures

Figure 2-1  Federated Architecture for FANS-1	6

Figure 2-2  FANS-1 Architecture on Boeing 777	7

Figure 3-1  Indication of New Uplink	8

Figure 3-2  LOAD Prompt for Loadable Uplink	8

Figure 3-3  EICAS Datablock on Boeing 777 Airplane	9

Figure 3-4  ATC Datablock on Boeing 787 Airplane	10

Figure 3-5  ATC Uplink Display on MFD on Boeing 787	11

Figure 4-1  Loading of um80	17

Figure 4-2  Loading of um73	18

Figure 4-3  Loading of um79	19

Figure 4-4  Loading of um83	20

List of Tables

Table 3.3-1  FMC-Loadable Uplinks for Boeing Airplanes	14

# Introduction

One of the significant benefits of Air Traffic Control (ATC) datalink (using both FANS-1 and ATN technologies) is the ability to integrate the communication and surveillance functions with the navigation function on the airplane, to provide an integrated CNS capability.

This allows ATC clearances issued by the controller to be loaded into the airplane’s Flight Management System (FMS) by the flight crew with only a few key presses, rather than having to type the data in by hand. This significantly reduces the possibility of data entry errors, and greatly reduces the time and workload required to accomplish the task.

However, in order to make use of this capability, ground system automation designers need a good understanding of how the uplinks are processed by the airplane avionics systems, so that the systems they design can construct clearances that the airplane avionics will load in the way the controller intends.

The purpose of this document is therefore to provide guidance to ground system automation designers, so that they will be able to construct clearances that:

(a) pass the loading checks performed by the avionics on Boeing airplanes, and

(b) create flight plan modifications in the avionics on Boeing airplanes that accurately reflect the intended clearance.

This document addresses the avionics systems on the Boeing 737, 747-400, 747-8, 757, 767, 777 and 787. While other airplanes may have very similar systems installed, there may be differences not discussed in this document. More detail on the implementations in the individual airplanes, including known deficiencies, can be found in the individual ATS SR&O documents (ATS Capabilities Document on 787) listed in the References Section.

# Airplane Architectures

There are two basic ATC datalink architectures on Boeing airplanes. The first is the federated architecture. A composite example of this type is shown in Figure 2-1 below. In this type of architecture, the ATC datalink applications are hosted in the Flight Management Computer (FMC), and the ACARS Management Unit (MU) or Communications Management Unit (CMU) is used a router for the VHF, SATCOM (and even HF) data exchange with the ground.

Figure 2-1  Federated Architecture for FANS-1

In this type of architecture, loading of data from the ATC datalink application into the route in the FMC is purely an internal interface.

This type of architecture is used for the avionics systems on the Boeing 737, 747-400, 757 and 767 airplane models.

The other type of architecture used on Boeing airplane models is the integrated architecture used on the Boeing 777 and 787 airplane models. In this type of architecture, the FMC, the ATC datalink applications and the router are software applications hosted on an integrated processor. An example of this (specifically for the Boeing 777) is shown in Figure 2-2 below.

Figure 2-2  FANS-1 Architecture on Boeing 777

Here, the Flight Deck Communication Function in the AIMS cabinet provides the ATC datalink applications, using the router functionality of the Data Communication Management Function. The interface to the Flight Management Computing Function for loading into the FMC’s route is within the AIMS cabinet.

# ATC Uplink Display and Processing

ATC uplinks contain up to five message elements from the message set defined for the application in use (RTCA DO-258A/EUROCAE ED-100A for FANS-1, or RTCA DO-280B/EUROCAE ED-110B for ATN). Any or all of the message elements may be elements that are loadable into the FMC route. If one or more is loadable, then when the message is displayed a button or prompt to allow the message to be loaded will be provided.

## CDU-Based System Display and Processing

On the CDU-based systems (those airplanes employing the federated architecture, the Boeing 737, 747-400, 757 and 767), when an ATC uplink is received by the FMC, an indication is provided on the Engine Indicating and Crew Alerting System (EICAS) display, along with an aural alert. This is shown in Figure 3-3 below.

Figure 3-3  Indication of New Uplink

The flight crew would then normally select ATC and display the new uplink on the Multifunction Control and Display Unit (MCDU). The display would be as in Figure 3-4 below. The exact display on a particular airplane model may be slightly different. This should be regarded as an example only.

Figure 3-4  LOAD Prompt for Loadable Uplink

On selection of the LOAD prompt, the route is modified by applying each of the loadable message elements sequentially (the example in the figure only contains one, which is the most likely scenario) to the active route. The flight crew would then review the modified route graphically on the Navigation Display, and in text form (with performance predictions) on the LEGS page to verify its acceptability. If the uplink does not load completely, an error message indicating a total load failure or a partial failure will be provided in the scratchpad. It will not indicate the specific nature of the error, or if multiple errors were detected. If the result is acceptable to the flight crew, they would accept the clearance and execute the flight plan modification. If it is not, they would reject the clearance, and erase the modification. The recommendation in such cases would normally be to reject the clearance and contact ATC to obtain one that can be executed.

## MFD-Based System Display and Processing

On those airplanes employing an integrated architecture (the Boeing 777 and 787), the ATC datalink displays are presented on a Multi-Function Display (MFD), and crew interaction is through a cursor, rather than using the line select keys on the MCDU.

On these airplanes, when an ATC uplink is received by the ATC datalink application, an indication is provided on the Engine Indicating and Crew Alerting System (EICAS) display, along with an aural alert, similar to those on the federated architectures. However, the uplink is automatically displayed on a separate datablock display used for this purpose. On the 777, this is on the EICAS (as shown in Figure 3-5 below).

Figure 3-5  EICAS Datablock on Boeing 777 Airplane

On the Boeing 787 airplane, the ATC uplinks are displayed on separate Auxiliary displays alongside the Primary Flight Display (PFD) (as shown in Figure 3-6below). This allows the crew to process most ATC uplinks without having to remove one of their existing display formats, and replace it with one dedicated to communications. This should contribute to reductions in crew response times.

Figure 3-6  ATC Datablock on Boeing 787 Airplane

The flight crew can now view the message (if it is not so large than it cannot be viewed in its entirety on the datablock), and accept, reject or cancel it using glareshield-mounted buttons. They may also display the message on one of the MFDs. If it contains one or more of the loadable message elements, there would normally be a “LOAD FMC” command button on the display (as shown in Figure 3-7below).

Figure 3-7  ATC Uplink Display on MFD on Boeing 787

On selecting the “LOAD FMC” command button on the Boeing 777, the uplink is then passed to the FMC, which applies each of the loadable message elements sequentially, just as on the federated architecture airplanes, resulting in either a successful load or an error message (which may involve a partial load or nothing being loaded, depending on the type of error encountered).  The recommendation in such cases would normally be to reject the clearance and contact ATC to obtain one that can be executed.

On the Boeing 787 airplane, the process is a little different. Whenever the message is displayed (even if only on the Auxiliary displays), and it contains one or more of the loadable elements, it is passed to the Flight Management Function, which then applies them to the active route to create a separate datalink flight plan. Only if the uplink loads 100% (with no errors detected) will the “LOAD FMC” command button appear (on both the ATC uplink display on the MFD, and on the FMS displays). If any errors are detected, an error message is provided indicating that the uplink contains invalid data. There are no partial loads on the Boeing 787.

## Duplicate Waypoint Resolution

Many identifiers of navaids and fixes are used more than once in the worldwide navigation databases on Boeing airplanes. If one of these identifiers that has duplicates is used in a route uplink, then the FMC must resolve which of those duplicates to use.

If a duplicate waypoint identifier is contained in the [routeinformation] as a [publishedidentifier], or as part of a [placebearingdistance] or a [placebearingplacebearing] then the appropriate version of that fix, from the Navigation Data Base, will be selected on the basis of the following priority:

1. As the closest to the reference latitude/longitude, if one is included.

2. If it's the first en-route fix, and it matches a fix in the departure procedure

3. If it follows an airway, and it matches a fix on that airway

4. If it is followed by an airway, and it matches a fix on that airway

5. If it's the last en-route fix, and it matches a fix in the arrival or approach procedure

6. The one closest to the preceding flight plan fix

7. The one closest to the origin airport (except on 787, where it is the one closest to the origin airport when on ground, and the one closest to present position when in the air)

If a duplicate waypoint identifier is contained in a [position] parameter in the [routeinformationadditional], then the fix must lie in the route already defined. The waypoint type (navaid or fixname) is first used to limit the duplicates, and then the first waypoint of that identifier and type in the route (unless the reference lat/long is supplied) is used.

If a duplicate is encountered as a [position] parameter in an uplink (not inside a [routeclearance] parameter), then it is resolved as follows:

| Msg# | Uplink Message Element | Duplicate Resolution |
| --- | --- | --- |
| 46 | CROSS position AT level | First in route |
| 47 | CROSS position AT OR ABOVE level | First in route |
| 48 | CROSS position AT OR BELOW level | First in route |
| 49 | CROSS position AT AND MAINTAIN level | First in route |
| 50 | CROSS position BETWEEN level AND level | First in route |
| 51 | CROSS position AT time | First in route |
| 52 | CROSS position AT OR BEFORE time | First in route |
| 53 | CROSS position AT OR AFTER time | First in route |
| 58 | CROSS position AT time AT level | First in route |
| 59 | CROSS position AT OR BEFORE time AT level | First in route |
| 60 | CROSS position AT OR AFTER time AT level | First in route |
| 62 | AT time CROSS position AT AND MAINTAIN level | First in route |
| 74 | PROCEED DIRECT TO position | Closest to airplane position |
| 75 | WHEN ABLE PROCEED DIRECT TO position | Closest to airplane position |
| 77 | AT position PROCEED DIRECT TO position | First position : use first in route Second position : closest to airplane position |
| 79 | CLEARED TO position VIA route clearance | First in route |
| 83 | AT position CLEARED route clearance | First in route |
| 84 | AT position CLEARED procedure name | First in route |
| 91 | HOLD AT position MAINTAIN level INBOUND TRACK deg dir | First in route |
| 92 | HOLD AT position AS PUBLISHED MAINTAIN altitude | First in route |

Table 3.3-1  Duplicate Waypoint Resolution

It should be noted that the means of resolving duplicate waypoints for ATC datalink is rather more sophisticated than that used for AOC datalink flight plans. It is therefore possible, if an AOC uplink is used to provide a desired route change, that the ATC clearance will be interpreted a little differently. After loading the uplink, the flight crew is required to verify that the route has loaded correctly. It is recommended that where an operator is using AOC datalink to uplink a route, the optional latitude/longitude be used for fixes that have duplicates in the database, to ensure the correct one is selected by the avionics. Unfortunately, the limitations of the DO-258A message set prevent that being used in route clearances for ATC messages (except for place-bearing-distance waypoints).

## Multi-Element Messages

An ATC uplink may consist of up to 5 of the message elements defined in DO-258A/ED-100A. Any or all of those may be message elements that can be loaded into the FMS route. If the message consists of multiple loadable elements, then each of those loadable elements will be loaded into the FMS route in the order they appear in the uplink. The effect of each will be as described in the sections below. It will not be modified as a result of another element appearing earlier, or by any of the non-loadable message elements.

As an example, consider the following clearance:

CROSS ABC AT FL190,

THEN,

PROCEED DIRECT TO XYZ.

This would be loaded as an altitude crossing constraint at ABC, and a direct leg from present position to XYZ (which could delete ABC and its crossing constraint, if XYZ was already in the route, somewhere past ABC). The non-loadable element (THEN) has no effect whatsoever. And the fact that the clearance to proceed direct to XYZ comes after the crossing constraint does not result in any attempt to comply with the crossing constraint before executing the direct clearance.

Obviously, a clearance that replaces the entire route (such as um73 or um80) would need to be the first loadable element in an uplink, as it would effectively overwrite any preceding ones. Similarly a clearance element that depends on another has to come after it. For example, a crossing clearance depends on the existence of the fix, so a clearance such as PROCEED DIRECT TO would need to come before the crossing clearance for that fix.

Controllers should therefore ensure that the order of clearance elements in the message will result in the intended flight plan being loaded.

As general guidance, use of route clearance elements helps to reduce the potential for unintended loads like the above. As the ground generally does not know precisely what is in the aircraft’s active flight plan, the ground should generally use messages with the [routeclearance] variable rather than piecing together multiple elements. This allows the ground system to be sure that the aircraft system loads the route and constraints as intended.

## Route Clearance Display Presentation

While most CPDLC uplink message elements are displayed in their entirety on the Boeing FANS-1 airplanes, this is not true for those uplink message elements containing the [routeclearance] parameter. These are:

Um73 [predepartureclearance]

Um79 CLEARED TO [position] VIA [routeclearance]

Um80 CLEARED [routeclearance]

Um83 AT [position] CLEARED [routeclearance]

Um85 EXPECT [routeclearance]

Um86 AT [position] EXPECT [routeclearance]

The latter two are, of course, not loadable, as they are not clearances, so they are not considered further here. For the other 4 uplink message elements, what is presented on the display depends on what type of display is used.

### MCDU-based airplanes (737, 747-400 and -8, 757 and 767

On these airplanes, the [routeclearance] parameter will not be displayed or printed. The uplinks are displayed as:

Um73 PRE-DEPARTURE CLEARANCE

Um79 CLEARED TO XXX VIA ROUTE CLEARANCE

Um80 CLEARED ROUTE CLEARANCE

Um83 AT XXX CLEARED ROUTE CLEARANCE

None of the additional parameters in the [predepartureclearance] (equipment codes, departure time, PDC revision, etc.) are displayed, but some (excluding the equipment codes) will be printed if the message is printed.

### Large Format Display Airplanes (777 and 787)

On these airplanes, the display of the route clearance will include all the elements of the [routeinformation] and [routeinformationadditional], so that the flight crew can see the cleared route on the Communication display (rather than just on the FMC display when it has been loaded). The additional information in the [predepartureclearance], again excluding the equipment codes, is also displayed and printed.

# Loadable Uplinks

A number of the ATC clearances in the FANS-1 (RTCA DO-258A/EUROCAE ED-100A) message set may be loaded directly into the Flight Management System (FMS) as part of the route, and executed automatically by flying with LNAV and VNAV engaged. The initial FANS-1 development on the Boeing 747-400 began with 10 of these message being loadable. Subsequent FANS-1 developments have increased this to around 25.

The message elements that are loadable for each of the Boeing airplane types are shown in Table 3.4-2 below.

| Msg# | Uplink Message Element | 737 | 747-400 | 747-8 | 757 767 | 777 | 787 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 46 | CROSS position AT level |  |  | X | X | X | X |
| 47 | CROSS position AT OR ABOVE level |  |  | X | X | X | X |
| 48 | CROSS position AT OR BELOW level |  |  | X | X | X | X |
| 49 | CROSS position AT AND MAINTAIN level |  |  | X |  |  | X |
| 50 | CROSS position BETWEEN level AND level |  |  | X |  |  | X |
| 51 | CROSS position AT time | X | X | X | X | X | X |
| 52 | CROSS position AT OR BEFORE time | X | X | X | X | X | X |
| 53 | CROSS position AT OR AFTER time | X | X | X | X | X | X |
| 58 | CROSS position AT time AT level |  |  | X |  |  | X |
| 59 | CROSS position AT OR BEFORE time AT level |  |  | X |  |  | X |
| 60 | CROSS position AT OR AFTER time AT level |  |  | X |  |  | X |
| 62 | AT time CROSS position AT AND MAINTAIN level |  |  | X |  |  | X |
| 64 | OFFSET distance direction OF ROUTE | X | X | X | X | X | X |
| 67 | PROCEED BACK ON ROUTE |  |  | X |  |  | X |
| 73 | predepartureclearance | X | X | X | X | X | X |
| 74 | PROCEED DIRECT TO position | X | X | X | X | X | X |
| 75 | WHEN ABLE PROCEED DIRECT TO position |  |  | X | X | X | X |
| 77 | AT position PROCEED DIRECT TO position |  | X | X | X | X | X |
| 79 | CLEARED TO position VIA route clearance | X | X | X | X | X | X |
| 80 | CLEARED route clearance | X | X | X | X | X | X |
| 81 | CLEARED procedure name |  |  | X | X | X | X |
| 83 | AT position CLEARED route clearance | X | X | X | X | X | X |
| 84 | AT position CLEARED procedure name |  |  | X |  |  | X |
| 91 | HOLD AT position MAINTAIN level INBOUND TRACK deg dir |  |  | X |  |  | X |
| 92 | HOLD AT position AS PUBLISHED MAINTAIN altitude |  |  | X |  |  | X |

Table 3.4-2  FMC-Loadable Uplinks for Boeing Airplanes

When an ATC clearance containing one or more of these message elements is loaded into the route, then all the loadable elements will be loaded sequentially. Non-loadable elements in the same message will be displayed, but will not be loaded and will not effect processing of the loadable elements in any way. The order is thus important, as a particular message element may be dependent on the existence of a specific fix in the route, and those message elements that replace the entire route (um#73 and #80) will delete any previous changes, so they should obviously appear first. The specific results of each message element are discussed in the paragraphs that follow. Use of route clearance messages helps resolve the risk of unintended loads.

## Altitude Crossing Clearances

The altitude crossing clearance elements that may be loaded directly into the FMS on the various Boeing airplanes are as follows:

| Msg Element | Description | Result | Airplane Implementations |
| --- | --- | --- | --- |
| 46 | CROSS position AT level | AT altitude constraint at the specified waypoint | 747-8, 757/767, 777,787 |
| 47 | CROSS position AT OR ABOVE level | AT/ABOVE altitude constraint at the specified waypoint | 747-8, 757/767, 777,787 |
| 48 | CROSS position AT OR BELOW level | AT/BELOW altitude constraint at the specified waypoint | 747-8, 757/767, 777,787 |
| 49 | CROSS position AT AND MAINTAIN level | AT altitude constraint at the specified waypoint | 747-8, 787 |
| 50 | CROSS position BETWEEN level AND level | Both an AT/ABOVE altitude constraint (for the first value) and AT/BELOW (for the second value) at the specified waypoint | 747-8, 787 |

The altitude (level) specified in the clearance must be in feet (QNH) or flight level for the message element to be loadable into the FMS. It must also be lower than the airplane’s cruise flight level.

The specified position must be a waypoint that exists in the route, and the waypoint type must be as specified for that waypoint in the route. Allowed waypoint types are:

Navaid

Fixname

Airport

Latitudelongitude

Placebearingdistance

If a matching waypoint exists more than once on the route (e.g. if a second waypoint using the same identifier is in the route), then the constraint will be applied to the first one.

## Time Clearances

The time clearance elements that may be loaded directly into the FMS on the various Boeing airplanes are as follows:

| Msg Element | Description | Result | Airplane Implementations |
| --- | --- | --- | --- |
| 51 | CROSS position AT time | RTA (AT) at the specified waypoint | 737, 747-400, 747-8, 757/767, 777,787 |
| 52 | CROSS position AT OR BEFORE time | RTA (AT/BEFORE) at the specified waypoint | 737, 747-400, 747-8, 757/767, 777,787 |
| 53 | CROSS position AT OR AFTER time | RTA (AT/AFTER) at the specified waypoint | 737, 747-400, 747-8, 757/767, 777,787 |

The specified position must be a waypoint that exists in the route, and the waypoint type must be as specified for that waypoint in the route. Allowed waypoint types are:

Navaid

Fixname

Airport

Latitudelongitude

Placebearingdistance

If a matching waypoint exists more than once on the route (e.g. if a second waypoint using the same identifier is in the route), then the RTA will be applied to the first one.

Note: Time constraints can also be uplinked using the RTA variable in route clearances. See Paragraph4.3.3.6 for further information.

## Route and Route Segment Clearances

There are 4 clearance elements that replace all or large segments of the existing route that may be loaded directly into the FMS on the various Boeing airplanes. They are as follows:

| 73 | predepartureclearance |
| --- | --- |
| 79 | CLEARED TO position VIA route clearance |
| 80 | CLEARED routeclearance |
| 83 | AT position CLEARED route clearance |

All four of these message elements are implemented on all Boeing FANS-1 airplane models (737, 747-400, 747-8, 757/ 767, 777 and 787).

The way in which each of these clearances modifies the route in the FMC is a little different. Message elements um80 and um73 will replace the route in its entirety, as shown in Figure 4-8 and Figure 4-9 below.

ORIG

DEST

ABCDE

EFGHI

ZABCD

UVWXY

PQRST

KLMNO

FGHIJ

OPQRS

JKLMN

ZZZZZ

YYYYY

XXXXX

CLEARED XXXXX – YYYYY – ZZZZZ

Replaces flight plan between origin and destination (on ground), or

Between airplane and destination (in air)

Inserts the fixes/ATS routes in the [routeinformation]

Figure 4-8  Loading of um80

The effect of um73 (PREDEPARTURE CLEARANCE) is identical to um80 (CLEARED ROUTE CLEARANCE) when the latter is used on the ground. Obviously, um73 should only be used on the ground. Loading of either um80 when the airplane is on the ground or um73 is shown in Figure 4-9 below.

ORIG

DEST

ABCDE

EFGHI

ZABCD

UVWXY

PQRST

KLMNO

FGHIJ

OPQRS

JKLMN

ZZZZZ

YYYYY

XXXXX

PREDEPARTURE CLEARANCE XXXXX – YYYYY – ZZZZZ

Replaces flight plan between origin and destination

Inserts the fixes/ATS routes in the [routeinformation]

Figure 4-9  Loading of um73

The other route clearance uplinks (um79 and um83) will each only replace a segment of the route. Message element 79 (CLEARED TO position VIA routeclearance) will replace the flight plan between where the airplane is and the specified waypoint, as shown in Figure 4-10 below.

ORIG

DEST

ABCDE

EFGHI

ZABCD

UVWXY

PQRST

KLMNO

FGHIJ

OPQRS

JKLMN

ZZZZZ

YYYYY

XXXXX

CLEARED TO JKLMN VIA XXXXX – YYYYY – ZZZZZ

Replaces flight plan between airplane and JKLMN

Inserts the fixes/ATS routes in the [routeinformation]

Figure 4-10  Loading of um79

Note that Figure 4-10 shows what happens when the specified [position] is a fix in the existing active route. If it is not a fix in the existing active route, then the position and the contents of the [routeinformation] parameter will be inserted, followed by a flight plan discontinuity and the remainder of the existing en-route segment of the route. For the example shown, if the waypoint JKLMN did NOT exist in the active route, then the result of this uplink would be:

XXXXX

YYYYY

ZZZZZ

JKLMN

-- DISCONTINUITY --

PQRST

UVWXY

ZABCD

EFGHI

OPQRS

Message element 83 (AT position CLEARED routeclearance) will replace the flight plan, beginning at the specified position, and continuing right up to the destination, as shown in Figure 4-11 below.

ORIG

DEST

ABCDE

EFGHI

ZABCD

UVWXY

PQRST

KLMNO

FGHIJ

OPQRS

JKLMN

ZZZZZ

YYYYY

XXXXX

AT UVWXY CLEARED XXXXX – YYYYY – ZZZZZ

Replaces flight plan between UVWXY and destination

Inserts the fixes/ATS routes in the [routeinformation]

Figure 4-11  Loading of um83

Again, Figure 4-11 shows what happens when the specified [position] is a fix in the existing active route. If it is not a fix in the existing active route, then the position and the contents of the [routeinformation] parameter will be inserted after the remainder of the existing en-route segment of the route, separated from it by a flight plan discontinuity and. For the example shown, if the waypoint UVWXY did NOT exist in the active route, then the result of this uplink would be:

PQRST

ZABCD

EFGHI

JKLMN

OPQRS

-- DISCONTINUITY --

UVWXY

XXXXX

YYYYY

ZZZZZ

### Airport and Procedure Information

Loading of the Departure and Destination information contained in a Route Clearance parameter is highly dependent on the context in which it is used, and in particular which uplink clearance contains the Route Clearance parameter. It will also depend on whether the clearance is loaded before or after departure.

The result of loading each of these parameters, and the loadability or otherwise of each parameter is addressed in the Table below.

| Parameter Name | Um79 | Um73/um80 | Um83 | Additional Information |
| --- | --- | --- | --- | --- |
| Departure Airport | On ground, it replaces the origin airport, and deletes existing departure runway and procedure. In air – not loadable. | On ground, it becomes the origin airport. In air – not loadable (no effect). | Not loadable. | The Departure Airport parameter is not loadable if the Departure Airport identifier does not match an identifier in the FMS Navigation Database. |
| Destination Airport | If it matches the [position] in um79, then it replaces the destination airport. Otherwise not loadable. | Becomes the destination airport. | Replaces the existing destination airport, and deletes the existing arrival/approach procedures and arrival runway. | The Destination Airport parameter is not loadable if the Destination Airport identifier does not match an identifier in the FMS Navigation Database. |
| Departure Runway | On ground,it replaces the existing departure runway and deletes the existing departure procedure if it is incompatible with the runway. In air – not loadable. | On ground - becomes the departure runway. In air – not loadable. | Not loadable. | The Departure Runway parameter is not loadable if the Runway Direction and Runway Configuration do not match a runway identifier for the applicable origin airport in the FMS Navigation Database. |
| Departure Procedure | On ground, it replaces the existing departure procedure and departure transition if it is compatible with the runway. Otherwise not loadable. In air – not loadable. | On ground, it becomes the departure procedure and departure transition if it is compatible with the runway. Otherwise not loadable. In air – not loadable. | Not loadable. | The Departure Procedure parameter is not loadable if the Procedure Type choice is 0 (arrival) or 1 (approach). The Departure Procedure is not loadable if the Procedure Name does not match a departure procedure identifier, or departure procedure and transition identifiers, for the applicable origin airport in the FMS Navigation Database. |
| Approach Procedure | If the destination airport matches the [position] in um79, and the procedure is compatible with the arrival runway, then it replaces the existing approach procedure and approach transition. | If the procedure is compatible with the arrival runway, then it becomes the approach procedure and approach transition. | If the procedure is compatible with the arrival runway, then it replaces the existing approach procedure and approach transition. | The Approach Procedure parameter is not loadable if the Arrival Procedure choice is 0 (arrival) or 2 (departure). The Approach Procedure is not loadable if the Procedure Name does not match an approach procedure identifier, or approach procedure and transition identifiers, for the applicable destination airport in the FMS Navigation Database. Otherwise, the Approach Procedure is loadable |
| Arrival Runway | If the destination airport matches the [position] in um79, then it replaces the existing arrival runway. | It becomes the arrival runway. | It replaces the existing arrival runway. | The Arrival Runway parameter is not loadable if the Runway Direction and Runway Configuration do not match a runway identifier for the applicable destination airport in the FMS Navigation Database. Otherwise, the Arrival Runway is loadable |
| Arrival Procedure | If the destination airport matches the [position] in um79, and the procedure is compatible with the approach procedure and arrival runway, then it replaces the existing arrival procedure and approach transition. | If the procedure is compatible with the approach procedure and arrival runway, then it becomes the arrival procedure and approach transition. | If the procedure is compatible with the approach procedure and arrival runway, then it replaces the existing arrival procedure and approach transition. | The Arrival Procedure parameter is not loadable if the Arrival Procedure choice is 1 (approach) or 2 (departure). The Arrival Procedure is not loadable if the Procedure Name does not match an arrival procedure identifier, or arrival procedure and transition identifiers, for the applicable destination airport in the FMS Navigation Database. Otherwise, the Arrival Procedure is loadable |
| Airway Intercept | This will open the specified airway. In air, the airway entry point will be the closest fix on that airway to the airplane. On ground, it will be the closest fix on that airway to the origin airport, or, if a departure procedure/transition is specified, the last fix in the procedure/transition. | This will open the specified airway. In air, the airway entry point will be the closest fix on that airway to the airplane. On ground, it will be the closest fix on that airway to the origin airport, or, if a departure procedure/transition is specified, the last fix in the procedure/transition. | This will open the specified airway. The airway entry point will be the closest fix on that airway to the specified position. | If this parameter is used, the first item in the routeinformation] may be an airway (which must intersect this one) or a fix on the airway specified in this parameter. Due to the lack of specificity about where the airway entry occurs, it is recommended that this parameter NOT be used. |

Note that the procedure type for each of the terminal area procedures needs to match the variable. For example, the arrival procedure needs to have a type of “arrival”. If it does not match, it will not load correctly.

The use of all these parameters listed in the table is available on all Boeing models. However, on the B737, prior to software release U11.0, it should be noted that they are not loadable when used after departure as part of um79 or um83. On the B737 airplane, it is recommended that these parameters should not be used in um79 or um83.

Note that to avoid a discontinuity being created in the FMS flight plan when the uplink is loaded,

- If the [routeclearance] contains either an arrival procedure/transition (with or without an approach) or an approach procedure/transition (with no arrival specified), then the last waypoint in the  [routeinformation] must be the same as the first fix in the transition (if specified) or the procedure (if a transition is not specified).

- If the [routeclearance] in an uplink contains both an arrival procedure/transition and an approach procedure/transition, then the approach/transition and arrival must be compatible.

#### Procedure Naming

The procedure names are up to six characters, and correspond to those on the terminal area procedure charts for the respective airport. An additional transition may also be specified. That may be up to five characters in length, and usually corresponds to the name of the last fix (departures) or first fix (arrivals and approaches).

Examples of departure procedures would be:

LACKR1

ELIOT1

Examples of arrival procedures would be:

CHINS6

DUVAL3

Approach procedures (except 8-character ones – see below) are created by combining the approach 3-letter type (e.g. ILS, VOR, GLS) with the runway identifier, such as, for example:

ILS28L

VOR25

NDB03R

#### Eight-Character Approaches

In a number of cases, it has been necessary to create more than one procedure of a particular type to the same runway end. In this case, the FMS on Boeing airplanes uses an 8-character identifier. This is comprised of the approach type (3 letters), followed by an identifier indicating which of the multiple approaches of that type is being referenced, a space and the runway identifier. An example would be:

GLSY 28R

This cannot be contained within the 6 characters allowed for a procedure name. However, the FMS will derive the correct name if the procedure name contains just the first 4 characters, (GLSY in the above example) and the runway arrival is specified (for the above example, it would be 28R).

This feature is available on the 737, 747-8, 757/767, 777 and 787 airplanes.

### Route Information

The [routeinformation] parameter provides a sequence of fixes and ATS routes (airways) that will be loaded sequentially as the en-route segment of the flight plan. The items in the [routeinformation] parameter can be any of the following:

Published Identifier

Latitude Longitude

Place Bearing Place Bearing

Place Bearing Distance

ATS Route Designator

The FMS attempts to load all the [routeinformation] parameters into the route in the order in which they occur in the [routeinformation] parameter.

#### Waypoints

To be loadable, a Published Identifier, or the identifier(s) in  a Place Bearing Distance or Place Bearing Place Bearing must match an identifier in the FMS Navigation Database.

The Published Identifier can be a runway identifier (e.g. RW07) and is loadable if that runway identifier exists at the arrival airport.

Any Place Bearing Distance must have a Distance choice of 0 (Distancenm) to be loadable.

A Distance must have a value less than or equal to 700nm to be loadable.

Waypoints of the type [PlaceBearingPlaceBearing] will be converted to place-bearing-distance waypoints when they are loaded into the FMS, so any [routeinformationadditional] information associated with that waypoint would need to reflect this..

#### Airways

An ATS Route Designator must match an airway identifier in the FMS Navigation Database, for it to be loadable.

An ATS Route Designator may be the first item in the [routeinformation], but only if:

- the airway intercept parameter (see above) is used, and that first airway intersects this second airway, or

- the last fix in the departure procedure/transition is on that airway

Apart from an ATS route designator used as the first item (as described above), the point at which the airplane is to join the airway must be defined by a Published Identifier or another airway (ATS Route Designator) that intersects the second one,  immediately preceding the ATS Route Designator in the [routeinformation]. On 787 only, the point at which the airplane is to join the airway may be defined as a Latitude/Longitude that corresponds to a fix on the airway.

The point at which the airplane is to leave the airway must be defined by

- a Published Identifier immediately following the ATS Route Designator in the [routeinformation]. On 787 only, this may also be a Latitude/Longitude that corresponds to a fix on the airway, or

- another airway (ATS Route Designator) that intersects the first airway immediately following the ATS Route Designator in the [routeinformation], or

- the first fix in the arrival/transition (if specified) or approach/transition, if it lies on the airway, and the airway is the last item in the [routeinformation]

As noted above, the airway entry/exit point should never be a latitude/longitude fix (except on 787). It should never be a place-bearing-distance or place-bearing-place-bearing fix, or an airport. In general, it is recommended that airway entry/exit points should always be a [publishedidentifier].

Note also that the navigation database in the Flight Management System is generally updated every 28 days on the AIRAC cycle. Therefore, it will only contain published data that is also updated on the AIRAC cycle. It will not contain track data (e.g for North Atlantic tracks or PACOTS tracks that is updated daily), so these cannot be used as airways in a route uplink.

### Additional Route Information

The [routeinformationadditional] parameter can be used to add various data, above and beyond the basic lateral routing to the flight plan clearance. The data that can be added includes:

Along Track Waypoints

Reporting Points

Intercept Course From

Hold At Waypoint

Waypoint Speed/Altitude

Required Time of Arrival (RTA)

The rules for using each of these are described below.

#### Along Track Waypoints

Along track waypoints are used to insert constraints at places on the route other than the defined waypoints (e.g. to be at 16000ft 10 miles prior to passing waypoint XXX). This item in the [routeinformationadditional] parameter allows a controller or ground automation system  to insert such waypoints. Up to 8 along-track-waypoints may be included in a route clearance.

| Variable Name | Loading Rules |
| --- | --- |
| [atwalongtrackwaypoint] | When the individual parameters are all loadable (see below), the FMF converts these to a place-bearing-distance (PBD) waypoint and then inserts the resulting PBD into the correct location in the selected route |
| [position] | The [position] must match a fix in the route. It can be - a loadable [publishedidentifier] or [placebearingdistance] in the uplinked [routeinformation] - a waypoint on an airway defined by an [airwayidentifier] in the uplinked [routeinformation] - the [position] portion of element 79 or 83 - a fix in a terminal area procedure. This is not loadable for [latitudelongitude] waypoints. |
| [aTWdistance] | The distance must be in nautical miles (and less than or equal to 700), and the resulting ATW must fall between the [position] and the preceding or following waypoint for the ATW to be loadable. |
| [speed] | The speed choice must be (speedindicated) and the value greater than (or equal to) 100kts to be loadable. For the speed to be loadable, there must be a valid [aTWaltitude] defined . It is then inserted as a speed constraint for the corresponding along track waypoint (PBD) |
| [aTWaltitude] | The [altitude] choice for either altitude in the [aTWaltitudesequence] must be 0 (altitudeqnh), 1 (altitudeqnhmeters), or 6 (altitudeflightlevel) to be loadable. The value for a single [aTWaltitude] or the first [altitude] in the [aTWaltitudeseqence] must be below the current cruise altitude. It is then inserted as an AT, AT OR ABOVE or AT OR BELOW altitude constraint or window altitude constraint for the corresponding along track waypoint (PBD) |

#### Reporting Points

This allows a controller to include in a route a point or a set of points to be used for position reporting purposes.

| Variable Name | Loading Rules |
| --- | --- |
| [reportingpoints] | FMF inserts a latitude/longitude waypoint or series of latitude/longitude waypoints, as defined by the sub-parameters |
| [latitudereportingpoints] [latitudedirection] [latitudedegrees] [longitudereportingpoints] [longitudedirection] [longitudedegrees] | If the specified latitude or longitude intersects the route, then the FMF will insert a latitude/longitude waypoint at the intersection point. For um79 and um83, the intersection of the route with this latitude or longitude must occur in the section of the route created by the uplink, not in the section prior to the specified position (um83) or the section after the specified position (um79). |
| [degreeincrement] | If the [degreeincrement] is included, then the FMF will insert reporting points at intervals along the selected route, as defined by the [degreesincrement] along the direction of flight starting at the intersection point. If the location of one of these points matches an existing fix in the route, then that particular point will not be added to the route. |

#### Intercept Course From

This allows a controller to set up the airplane to fly a specified course after passing one of the fixes in the flight plan. The flight crew will have to define the termination of this flight plan leg. Up to 4 of these may be specified in a route clearance. Due to its unsuitability for oceanic operations, and potential confusion in its use, this construct is not recommended for in-service use.

| Variable Name | Loading Rules |
| --- | --- |
| [interceptcoursefrom] | The FMF inserts the intercept course from waypoint into the correct location in the selected route If, after loading, the pilot does not terminate the intercept course from leg, then it remains an intercept course from leg in the flight plan. If, after loading, the pilot does terminate the intercept course from leg, then the FMF creates a latitude longitude waypoint at the termination point and the intercept course from leg no longer exists. |
| [interceptcoursefromselection] | This must be one of the following: - a loadable [publishedidentifier], [latitudelongitude], [placebearingplacebearing], or [placebearingdistance] in the uplinked [routeinformation], - a waypoint on an airway defined by an [airwayidentifier] in the uplinked [routeinformation], - a fix in a [proceduredeparture], [procedurearrival], or [procedureapproach] included in the uplinked [routeclearance] - the [position] portion of element 79 or 83, |
| [degrees] | This may be either MAG or TRU. |

#### Hold At Waypoint

This allows the controller to include a holding pattern at any of the fixes in the route as part of a route clearance uplink. Up to 8 holding patterns may be specified in a route clearance.

| Variable Name | Loading Rules |
| --- | --- |
| [holdatwaypoint] | This loads a Holding pattern at the specified position using the associated data to define the details of the holding pattern. The FMF has pre-defined defaults for the associated data if it is not provided. |
| [position] | The [position] must match: - A loadable [publishedidentifier],[ latitudelongitude], or [placebearingdistance] in the uplinked [routeinformation] - A waypoint on an airway defined by an [airwayidentifier] in the uplinked [routeinformation] - The [position] portion of element 79 or 83 - A fix in a [proceduredeparture], [procedurearrival], or [procedureapproach] included in the uplinked [routeclearance] This will then be used as the position for the hold in the route |
| [holdatwaypointspeedlow] | This will load as a speed constraint associated with the hold in the route This must be choice 0 (speedindicated) and the [speed] value must be greater than or equal to 100kts. If this parameter is used, then an [altitude] value must also be included. If this parameter is used, then the [holdatwaypointspeedhigh] parameter may not be used. |
| [aTWaltitude] | This will load as an AT, AT OR ABOVE or AT OR BELOW altitude constraint associated with the hold in the route. The [altitude] choice must be: i. 0 (altitudeqnh), ii. 1 (altitudeqnhmeters) iii. 6 (altitudeflightlevel The value must be below the current cruise altitude. |
| [holdatwaypointspeedhigh] | This will load as a speed constraint associated with the hold in the route This must be choice 0 (speedindicated) and the [speed] value must be greater than or equal to 100kts. If this parameter is used, then an [altitude] value must also be included. If this parameter is used, then the [holdatwaypointspeedlow] parameter may not be used. |
| [direction] | This provides the turn direction associated with the hold. The [direction] must be specified as choice 0 (left) or 1 (right). |
| [degrees] | This will be loaded as the inbound course associated with the hold. [degrees] may be specified as MAG or TRU. |
| [EFCtime] | This will provide the Expect Further Clearance (EFC) Time associated with the hold. |
| [legtype] | This provides the Leg Distance or Leg Time for the hold. When the [legtype] is [legdistance] , it must be specified as [legdistanceenglish]. |

#### Waypoint Speed/Altitude

This parameter allows a controller to include altitude or speed/altitude constraints associated with the waypoints in the route. Up to 32 of these may be specified in a route. Note that it is generally not necessary to include any such constraints associated with waypoints in terminal area procedures. Those that are part of published procedures will be loaded automatically when the procedure is loaded.

Note that on the B737 airplane, prior to software release U11.0, this cannot be used for a [position] defined as being of type [latitudelongitude].

| Variable Name | Loading Rules |
| --- | --- |
| [waypointspeedaltitude] | This inserts waypoint speed and altitude or altitude-only constraint in the route. The parameters included are the speed, altitude, and associated position. |
| [position] | The [position] must match: - A loadable [publishedidentifier],[ latitudelongitude], or [placebearingdistance] in the uplinked [routeinformation] - A waypoint on an airway defined by an [airwayidentifier] in the uplinked [routeinformation] - The [position] portion of element 79 or 83 - A fix in a [proceduredeparture], [procedurearrival], or [procedureapproach] included in the uplinked [routeclearance] This will then be used as the position for the constraint |
| [speed] | This will load as a speed constraint at the specified position. This must be choice 0 (speedindicated) and the [speed] value must be greater than or equal to 100kts. If this parameter is used, then an [altitude] value must also be included. |
| [ATWaltitudesequence] | This will load as an AT, AT OR ABOVE or AT OR BELOW altitude constraint at the specified position. The [altitude] choice must be: i. 0 (altitudeqnh), ii. 1 (altitudeqnhmeters) iii. 6 (altitudeflightlevel The value must be below the current cruise altitude. |

#### Required Time of Arrival (RTA)

This allows the controller to specify a Required Time of Arrival (RTA) at a waypoint in the route. Only one RTA may be specified. This should generally be specified at a fix in cruise. On most Boeing airplanes, the RTA is only actively managed in the cruise flight phase.  On 737, RTAs will be actively managed in climb or descent as well.

| Variable Name | Loading Rules |
| --- | --- |
| [RTArequiredtimeofarrival] | This creates an RTA at the specified position in the route, and deletes any existing RTA that has been entered. |
| [position] | The [position] must match: - A loadable [publishedidentifier],[ latitudelongitude], or [placebearingdistance] in the uplinked [routeinformation] - A waypoint on an airway defined by an [airwayidentifier] in the uplinked [routeinformation] - The [position] portion of element 79 or 83 - A fix in a [proceduredeparture], [procedurearrival], or [procedureapproach] included in the uplinked [routeclearance] This will then be used as the position for the RTA |
| [RTAtime]] | This will load as the [RTAtime] (UTC) and [timetolerance] (AT, AT/BEFOR or AT/AFTER) for the RTA. |
| [RTAtolerance] | NOT USED |

Note: Time constraints can also be uplinked using the time crossing constraint messages. See Paragraph 4.2 for further information.

## Holding Pattern Clearances

The holding pattern clearance elements that may be loaded directly into the FMS on the various Boeing airplanes are as follows:

| Msg Element | Description | Result | Airplane Implementations |
| --- | --- | --- | --- |
| 91 | HOLD AT position MAINTAIN level INBOUND TRACK degrees direction TURNS legtype | Inserts a holding pattern as defined in the uplink at the specified waypoint | 747-8, 787 |
| 92 | HOLD AT position AS PUBLISHED MAINTAIN altitude | Inserts a holding pattern as defined in the airplane’s navigation database at the specified waypoint | 747-8, 787 |

The specified position must be a waypoint that exists in the route, and the waypoint type must be as specified for that waypoint in the route. Allowed waypoint types are:

Navaid

Fixname

Airport

Latitudelongitude

Placebearingdistance

If a matching waypoint exists more than once on the route (e.g. if a second waypoint using the same identifier is in the route), then the holding pattern will be inserted at the first one.

For both um91 and um92, the specified altitude must be below the current cruise altitude. If the published details of the en-route hold at the fix specified in um92 includes an altitude, then the altitude in the uplink must match that published value. This will be used as an altitude constraint at the specified fix.

The direction must be [Left] or [Right] in um91. This provides the turn direction associated with the hold.

Degrees may be MAG or TRU in um91. This will be loaded as the inbound course associated with the hold.

For um91, the [legtype] provides the Leg Distance or Leg Time  for the hold. When the [legtype] is [legdistance] , it must be specified as [legdistanceenglish], i.e. in nautical miles.

## Direct Clearances

The direct clearance elements that may be loaded directly into the FMS on the various Boeing airplanes are as follows:

| Msg Element | Description | Result | Airplane Implementations |
| --- | --- | --- | --- |
| 74 | PROCEED DIRECT TO position | Inserts a direct leg from the airplane’s present position to the specified fix in the active route. | 737, 747-400, 747-8, 757/767, 777, 787 |
| 75 | WHEN ABLE PROCEED DIRECT TO position | Same as um74. | 747-8, 757/767, 777, 787 |
| 77 | AT position PROCEED DIRECT TO position | Inserts a leg from the first specified fix to the second specified fix in the active route. | 737, 747-400, 747-8, 757/767, 777, 787 |

For um 74 and 75, the specified position does not have to exist in the route. Allowed waypoint types are:

Navaid

Fixname

Airport

Latitudelongitude

Placebearingdistance

If the fix does already exist in the route, then the flight plan change will effectively be to bypass the intermediate waypoints, which will no longer be available for position reporting purposes. If the fix exists more than once on the route (e.g. if a second waypoint using the same identifier is in the route), then the direct leg will be to the first one. If the fix does not already exist in the route, then it will be followed by a discontinuity, and the remainder of the existing route.

For um77, the first specified position must be a waypoint that exists in the route, and the waypoint type must be as specified for that waypoint in the route. The second specified fix does not have to exist in the route. Allowed waypoint types are:

Navaid

Fixname

Airport

Latitudelongitude

Placebearingdistance

If either or both of the fixes exists more than once on the route (e.g. if a second waypoint using the same identifier is in the route), then the first one will be used in each case. If the second fix already exists in the route, then the effect will be to collapse the route between those two fixes into a single great-circle leg.

## Offset Clearances

The offset clearance elements that may be loaded directly into the FMS on the various Boeing airplanes are as follows:

| Msg Element | Description | Result | Airplane Implementations |
| --- | --- | --- | --- |
| 64 | OFFSET distance direction OF ROUTE | Creates a lateral (parallel) offset in the active route. | 737, 747-400, 747-8, 757/767, 777, 787 |
| 67 | PROCEED BACK ON ROUTE | Sets the lateral (parallel) offset in the active route to zero. | 747-8, 787 |

These two clearances insert (or in the case of um67, delete) lateral (or parallel) offsets in the active route. The offset begins immediately and extends to the point where it is cancelled automatically as a result of being incompatible (e.g. certain leg types in terminal area procedures, such as heading legs cannot have an offset).

The offset distance may be up to 99, and must be specified in nautical miles. Only two choice values for direction (Left and Right) may be used.

Note that lateral offsets will affect ADS reporting. Locations in the predicted route group and the projected intent groups will reflect the offset, and the baseline zero-deviation route used in determining the occurrence of a lateral deviation event will also reflect the offset. In most airplanes, the immediate effect on lateral deviation of entering an offset is that the deviation changes by the offset value (e.g. if the airplane is on-path and a 5 mile offset is entered, it is now 5 miles off the new path). On the 737 and 787 airplanes, the airplane path will reflect the track to reach the offset, so the deviation will be unchanged by entry of an offset.

## Terminal Area Procedure Clearances

The terminal area procedure clearance elements that may be loaded directly into the FMS on the various Boeing airplanes are as follows:

| Msg Element | Description | Result | Airplane Implementations |
| --- | --- | --- | --- |
| 81 | CLEARED procedure name | Inserts (or replaces the existing) terminal area procedure of the specified type in the active route. | 747-8, 757/767, 777, 787 |
| 84 | AT position CLEARED procedure name | Inserts (or replaces the existing) terminal area procedure of the specified type in the active route, with possibility for shortening the procedure or en-route segment. | 747-8,787 |

The procedure name must correspond to a procedure in the navigation database on the airplane. See section 4.3.1.1 for details of the naming conventions used. Due to the limitations of the RTCA DO-258A message set, the procedure names are limited to 6 characters, and since the runway is not included, eight-character approach names (see section 4.3.1.2) cannot be accommodated using these message elements.

Message element um81 provides a straight insertion of a procedure (or replacement of an existing one) of that type. It may be used for SIDs (obviously prior to departure only), in which case the procedure name must match that of a SID at the airplane’s departure airport, or for arrivals or approaches, in which case the procedure name must match that of a procedure at the airplane’s destination airport.

Message element um84 is rather more complex. It may only be used for arrivals or approaches. It will insert (or replace an existing) procedure of the type specified. The following should be borne in mind in using this uplink:

- If the specified position is a fix in the procedure (or the transition), then the procedure (or transition) will be shortened to commence at that point.

- If the procedure is an approach, then the position may lie in the en-route segment, but only if there is no arrival procedure. In this case, any en-route fixes after that will be deleted.

- If the procedure is an approach, then the position may lie in the arrival procedure or the approach procedure/transition.

- If the procedure is an arrival, the position may lie in the en-route segment or the arrival procedure/transition.

- If the procedure is an approach, and the position is in an arrival procedure (or transition), then that arrival procedure must be compatible with the assigned runway.

- If the procedure is an arrival, then that arrival procedure must be compatible with the assigned runway.

## Combination Clearances

Several clearance elements load into the FMS route as a combination of other uplink elements. The effect is the same as using both those other loadable elements.

The combination clearance elements that may be loaded directly into the FMS on the various Boeing airplanes are as follows:

| Msg Element | Description | Result | Airplane Implementations |
| --- | --- | --- | --- |
| 58 | CROSS position AT time AT level | Same as um46 and um51 together. | 747-8, 787 |
| 59 | CROSS position AT OR BEFORE time AT level | Same as um46 and um52 together. | 747-8, 787 |
| 60 | CROSS position AT OR AFTER time AT level | Same as um46 and um53 together. | 747-8, 787 |
| 62 | AT time CROSS position AT AND MAINTAIN level | Same as um46 and um51 together. | 747-8, 787 |

See the individual message elements for details.

# Additional Guidance

There are additional considerations that need to be borne in mind in constructing an uplink that will load cleanly in the FMS and create the flight plan intended.

This section is intended as a repository for this kind of guidance information, and will be expanded as experience shows additional areas that need such explanation.

## Discontinuities Inserted in the Route

Discontinuities are generally used on most airplane models when part of an ATC route uplink has failed to load correctly, due to some incompatibility between the route described in the uplink and the route construction rules of the FMS. This does not happen on 787, as any such incompatibilities are treated as a total inability to load the route, and no route loading is possible.

In addition, discontinuities are inserted in some case to indicate where the route defined by the uplink can be loaded successfully, but is not defined sufficiently completely to provide a complete flight plan definition. These can occur on all models, including 787. The discontinuities associated with each loadable route uplink element are:

| Uplink Message Element | Discontinuity Insertion |
| --- | --- |
| um74 PROCEED DIRECT TO [position] um75 WHEN ABLE PROCEED DIRECT TO [position] | If the [position] does not exist already in the route, a discontinuity will be inserted between the [position] and the remainder of the route. |
| um77 AT [position] PROCEED DIRECT TO [position] | If the second [position] does not exist already in the route, a discontinuity will be inserted between the second [position] and the remainder of the route. |
| um79 CLEARED TO [position] VIA [route clearance] | If the [position] does not exist already in the route, a discontinuity will be inserted between the [position] and the remainder of the route. |
| um83 AT [position] CLEARED [route clearance] | If the [position] does not exist already in the route, a discontinuity will be inserted between the existing route and the [position]. |
| um84 AT [position] CLEARED [procedure name] | If the [position] does not exist in the procedure, a discontinuity will be inserted between the [position] and the procedure. |

Note also that for those message elements using the [route clearance] parameter (i.e. um73, um79, um80 and um83), discontinuities will be inserted between:

- a departure procedure/transition and the en-route segment

- the en-route segment and the arrival/transition

- the arrival and the approach/transition

- the en-route segment and the approach/transition

- a departure procedure/transition and the arrival/transition or approach/transition (if there is no en-route segment)

whenever no common waypoint exists between the respective segments.

Ground system designers and users should attempt to ensure that none of these situations occurs with a route uplink, in order to avoid creating routes with discontinuities. Flight crew procedures in such circumstances may well call for the uplink to be rejected if it creates discontinuities.

## Altitude Constraints

Altitude constraints may be inserted at the various waypoints in the route by means of the crossing constraint uplinks (Section 4.1), or by using the waypoint speed/altitude feature in a route clearance (Section 4.3.3.5). They may also be specified at points along the track between waypoints, by means of the along track waypoint feature in a route clearance (Section 4.3.3.1)

Terminal area procedures (departures, arrivals and approaches) may also include altitude constraints. These will be incorporated in the route automatically, as a result of invoking the procedure. The altitude constraints do not therefore need to be specified in the uplink. If one is specified in the uplink, then it will replace the one specified at that fix in the procedure.

The altitude specified must be below the airplane’s cruise altitude, and may optionally include a speed constraint (though a speed constraint may not be specified without an altitude). For more details, refer to the sections referenced above.

# References

The following documents are referenced by this document and/or provide more detail not contained in this document:

- RTCA DO-258A/EUROCAE ED-100A, Interoperability Requirements for ATS Applications Using ARINC 622 Data Communications

- D926A087, Rev A 737 Air Traffic Services Systems Requirements and Objectives, 3 March 2003

- D926U100, Rev A 747-400 Air Traffic Services Systems Requirements and Objectives - Generation 4, 30 July 2002

- D926T0280-1757/767 Air Traffic Services Systems Requirements and Objectives - Generation 3, 3 December 2003

- D282W725-05 777 Air Traffic Services Systems Requirements and Objectives - Generation 6, September 14, 2005

- D613Z025-01,  787 Air Traffic Services (ATS) Capabilities Document

- Dxxxxxxx-xx, 747-8 Air Traffic Services (ATS) Capabilities Document

Active Page Record

| Page Numbers | Revision Level | Revision Type (Added, Deleted) |  | Page Numbers | Revision Level | Revision Type (Added, Deleted) |
| --- | --- | --- | --- | --- | --- | --- |
| 1-38 | A |  |  |  |  |  |

Revision Record

| Revision Letter | A |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Changes in This Revision | Added sections 3.5, 3.5.1 and 3.5.2. Revised sections 3.3, 4., 4.3.1, 4.3.2, and 4.3.3.5. Document repaginated.. |  |  |  |  |  |
| Authorization for Release |  |  |  |  |  |  |
| AUTHOR: | Gordon .R. A. Sandell |  | 66-CB-EA11 |  | 18 August 2011 |  |
|  | First Name MI Last Name |  | Org. Number |  | Date |  |
| APPROVAL: | John R. Olsen |  | 66-CB-EA00 |  | 18 August 2011 |  |
|  | First Name MI Last Name |  | Org. Number |  | Date |  |
| DOCUMENT RELEASE: |  |  |  |  |  |  |
|  |  |  | Org. Number |  | Date |  |