THE THIRTY-FIFTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/35)

(Sapporo, Hokkaido, Japan, 7-11 November 2011)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Status Update for Operational Trial of the ADS-C Climb Descend Procedure

(Presented by the Federal Aviation Administration)

SUMMARY

This information paper provides an update on the operational trials for the use of the Automatic Dependent Surveillance – Contract (ADS-C) Climb Descend procedure (CDP).

- Introduction

  - The operational trial of the ADS-C CDP began 15 February 2011 and will continue until 16 February 2012.  To date, the ADS-C CDP has only been applied to four (4) maneuvering and blocking aircraft pairs.  The application was successful in each instance.

- Discussion

  - The ADS-C CDP is limited for use between pairs of Required Navigation Performance 4 (RNP-4) qualified aircraft.  Currently, only 25.5 percent of the aircraft in the Oakland Flight Information Region (FIR) flight plan with RNP-4 equipage.  Only 44 percent of the aircraft in the Oakland FIR use ADS-C.  This has been a limiting factor on the number of ADS-C CDP procedures performed.  Traffic scenarios do present themselves to the controller which would be useful to apply the ADS-C CDP procedure, but equipage is lacking on one or both of the aircraft.

  - In this phase of the trial, controllers must currently apply a long checklist to apply the ADS-C CDP (see attachment 1).  Controller workload also limits when the procedure may be applied.  Unfortunately, when traffic levels are high and controller spare time is limited, those are the times when the greatest opportunity for the use of the procedure is present.

  - Fifteen days of aircraft requests for climbs were analyzed in September 2011.  During those fifteen days, 69 aircraft were advised unable climb that had a spacing of 16 to 30 nautical miles (NM).  While all of these aircraft pairs may not have met all the requirements for the ADS-C CDP procedure, the data indicates that there is potential to apply the procedure more frequently without the limiting factors above.

  - Lack of aircraft certification and equipage reduce the overall efficiency of the airspace.  However, as more aircraft operators acquire RNP-4 certification on their aircraft, it is expected that the use of the procedure will increase.

  - While the controller checklist is long, all of the steps to apply the procedure could be automated within the Ocean21 system.  With automation support, the controller workload would be greatly reduced.  This would also lead to increased use of the procedure.

- Recommendations

The meeting is invited to note the information in this paper.

Attachment 1 
ATC ADS-C CDP CHECKLIST

PRELIMINARY SCREENING CRITERIA CHECKS (STEPS 1-3)

Step #	Checklist Item										   Checkmark or

Insert Data

1.	AIRCRAFT CALLSIGNS……………………………....................................................................

2.  	BOTH Blocking and Maneuvering Aircraft must have the “3” 30/30 ADS separation flag set.

- a.  Both Aircraft Level Flight/Aircraft 1,000 Feet Apart/Planned Altitude Change 2,000 Feet or more.

  - Neither Aircraft on WX Dev nor requesting a WX Dev.

  - Both Aircraft RVSM

  - “POS” NOT Displayed on Either Data Block

  - There are no Out of Conformance (ARP) messages for either aircraft in the Sector Queue.

f.   Aircraft Same Direction traffic

FINAL SCREENING CRITERIA CHECKS (STEPS 4-9)

4.  	Initiate ADS DEMAND for both Aircraft.  ENTER TIME that DEMAND request was sent to Maneuvering Aircraft

5.	From ADD Report,

Mach Number of Maneuvering Aircraft…………………………………………………………………….

Mach Number of Blocking Aircraft………………………………………………………...……………….

6.	SAME SPEED OR FASTER AIRCRAFT IN FRONT:  6a, 6b, and 6c Must be Satisfied

6a.  From Conflict Report Window, ACTUAL Longitudinal Distance Between Maneuvering

and Blocking Aircraft AT LEAST 16 MILES

6b.   From ASD, Both Aircraft Same Groundspeed, or Faster Aircraft is in Front

6c.   From Step # 5, Both Aircraft Same Mach Number, or FASTER Mach AIRCRAFT IN FRONT

7.	OVERTAKE SITUATION:  7a, 7b, and 7c Must be Satisfied

7a.  From Conflict Report Window, ACTUAL Longitudinal Distance Between Maneuvering

And Blocking Aircraft AT LEAST 26 MILES

7b.  From ASD, Trailing Aircraft Groundspeed Must NOT be More Than 10 Knots Faster

7c.  From Step # 5, Trailing Aircraft is NOT More Than .02 Mach Faster

8.	Build Clearance utilizing MOPS Message 26 or 28, "CLIMB/DESCEND TO REACH (level) BY (time).

Probe the Pending Clearance.  Ensure that Time Inserted in Clearance is within 15 Minutes of Time

Inserted in Step # 4.

8a. Append Free-Text Advisory from the Pre Formatted messages “ADS-C CDP PROCEDURE IS BEING APPLIED BY ATC”.

9.	Check the 2nd Profile Conflicts of the Maneuvering Aircraft.  IF THERE ARE ACTUAL OR IMMINENT

CONFLICTS WITH OTHER AIRCRAFT, DO NOT EXECUTE PROCEDURE