THE THIRTY-SIXTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/36)

(San Diego, California, USA, 14-18 May 2012)

Agenda Item 5:  			Air Traffic Management (ATM) Issues

Safety Assessment of the Introduction of 10 Minutes Longitudinal Time Separation on PACOTS Routes

(Presented by JCAB and ENRI)

SUMMARY

This paper provides the final report of safety assessment of 10 minutes longitudinal time separation without mandatory application of Mach Number Technique on PACOTS routes within Fukuoka FIR.

- Introduction

  - 10 minutes longitudinal time separation without Mach Number Technique (MNT) is planned to be introduced on all routes in the Pacific Oceanic airspace of Fukuoka FIR. However, the safety of this separation has been verified only on the condition that position reports are obtained at least every 40 minutes[1], but not all Pacific Organized Track System (PACOTS) routes meet this requirement. This paper provides the final result of safety assessment of the implementation of 10 minutes longitudinal time separation without MNT on PACOTS routes.

- Discussion

2.1.	A gain-loss distribution analysis is common in the safety assessment of longitudinal time separation. However, it tends to overestimate the risk of collision, so a revised method is applied. The revised method considers the relative speed effect for a pair of aircraft. The method itself has been reviewed and discussed in the previous IPACG meeting[2] and ICAO RASMAG meeting[3]. More details can be found in References [3] and [4], and Appendix A.

- 2.2.	This time, data is obtained between May 2011 and March 2012. Flight data processing system data (FDPS) is used to extract aircraft flying PACOTS routes and true air speed (TAS) information. ADS message and ATO (Actual Time Over Fix) data are used to obtain the actual time at each waypoint. The flight time of westbound routes and eastbound routes between waypoints differ, so the risk of collision is calculated separately. The PACOTS route segments considered are summarized in Table 1 and shown in Fig. 1. Other PACOTS route segments are not considered here, because the flight time between the waypoints is less than 40 minutes.

- Table 1	PACOTS route segments considered.

| - Route segments | - 150E160E | - 160E150E | - DOVAGUKATA - 140E150E - UKATA150E | - 150E UKATA - 150E140E - UKATABIXAK |
| --- | --- | --- | --- | --- |
| - Direction | - Eastbound | - Westbound | - Eastbound | - Westbound |
| - Track | - 1, 2, 3, 4, 11, 12, 14, 15, UPR | - A, B, C, D, E,F, G, H, I, J, K, L, M, UPR | - 14, 15 | - H, I, J, K |
| - Number of flights | - 33,973 | - 12,888 | - 2,866 | - 2,419 |
| - Number of pairs applying time-based separation | - 7,006 | - 2,157 | - 208 | - 110 |
| - Number of pairs within 30 minutes initial time separation | - 485 | - 75 | - 24 | - 2 |
| - Ratio of aircraft installed ADS-C | - 83.1 % | - 82.7 % | - 91.8 % | - 93.4 % |
| - Average flight time | - 54.49 min | - 68.75 min | - 50.43 min | - 67.12 min |

- Fig. 1	PACOTS route segements considered.

- 2.3.	Once the distribution is determined, only aircraft pairs within 30 minutes initial time separation are considered. The calculated risk of collision is shown in Table 2. The risk of collision on both directions satisfies the target level of safety (TLS).

- Table 2	Calculated risk of collision.

|  | - Risk of collision |
| --- | --- |
| - Eastbound PACOTS | - 0.83E–9 |
| - Westbound PACOTS | - 3.79E–9 |
| - PACOTS total | - 1.79E–9 |
| - TLS | - 5.00E–9 |

- 2.4.	Although the risk of collision satisfies the TLS for both directions, the margin is smaller on westbound PACOTS routes. The risk of collision depends on the traffic volume and navigation performance mainly, so it might exceed the TLS on westbound routes in the near future. The reason why the risk of collision on westbound routes is higher is that the flight time between waypoints is longer. In general, the longer the flight time of segment is, the higher the risk of collision is. Therefore, the maximum period of the position report is to be set on PACOTS routes, similarly to the way it is set now (at present, 40 minutes). Fig. 2 shows the relative frequency of the position report period on both directions when the initial time separation is 30 minutes or less. The maximum position report period on eastbound routes is 71 minutes, while it is 89 minutes on westbound routes. Table 3 shows the aircraft ratio which meets the given position report interval condition and the risk of collision in each position report interval. The risk of collision is provided on eastbound routes only due to insufficient number of data on westbound routes. The table clearly shows that the longer the flight time of the segment is, the higher the risk of collision is. There is a trade-off between the risk of collision and the number of aircraft which can apply 10 minutes time-based separation, so the criteria should be determined considering the safety and the convenience.

- Fig. 2	Relative frequency of position report period when the initial time separation is 30 minutes or less.

- Table 3	The aircraft ratio which meets the given condition and the risk of collision.

| - Maximum report interval | - Eastbound PACOTS |  | - Westbound PACOTS |
| --- | --- | --- | --- |
|  | - Ratio | - Risk of collision | - Ratio |
| - 45 minutes | - 9.7 % | - 0.001E–9 | - 0 % |
| - 50 minutes | - 34.6 % | - 0.017E–9 | - 0 % |
| - 55 minutes | - 60.8 % | - 0.086E–9 | - 0 % |
| - 60 minutes | - 86.6 % | - 0.310E–9 | - 16.0 % |
| - 65 minutes | - 96.9 % | - 0.850E–9 | - 46.7 % |
| - 70 minutes | - 99.6 % | - 0.830E–9 | - 80.0 % |

- Conclusions

- 3.1.	According to the safety assessment, 10 minutes longitudinal time separation without MNT can be introduced on PACOTS routes with the condition of maximum position interval. However, the risk of collision depends on many factors such as traffic volume, weather condition, and navigation performance. Therefore, after the implementation of the new procedure, the risk of collision has to be monitored continuously.

- Action by the meeting

  - The meeting is invited to discuss the introduction of 10 minutes time separation on PACOTS routes and consider the interval of position reports.

References

[1] “Application of 10 min separation minimum without MNT within Fukuoka FIR,” IP/16, IPACG/32, Honolulu, Hawaii, USA, 10-14 May, 2010.

[2] “Risk Analysis of 10 Minutes Longitudinal Time Separation on PACOTS Routes,” WP/08, IPACG/35, Sapporo, Japan, 7-11 November, 2011.

[3] “Refined Calculation Method for Risk Analysis of Longitudinal Time Separation,” WP/03, ICAO RASMAG/16, Bangkok, Thailand, 20-24 February, 2012.

[4] R. Mori, “Safety Assessment for Reduced Time-Based Separation Minima on Oceanic Routes”, Journal of Mechanical Systems for Transportation and Logistics, Vol. 4, No. 1 (2011), pp. 39-49.

Appendix A

This appendix provides a detailed description of the refined calculation method for longitudinal time separation.

The basic calculation concept is the same as the one in EMA handbook[1]. The expression for the longitudinal collision risk model used in assessing the safety of operations is:

(1)

The parameters are summarized in Table A-1. Three parameters (,, and ) must be set.

Table A-1	Parameters in the calculation of the collision risk.

|  | Explanation | Value used | Source of the values |
| --- | --- | --- | --- |
|  | Lateral overlap probability | 0.2 | EMA handbook [1] |
|  | Vertical overlap probability | 0.5380 | ICAO SASP safety assessment [2] |
|  | Length of a typical aircraft | 0.040 NM | B777-300ER |
|  | Wingspan of a typical aircraft | 0.035 NM | B777-300ER |
|  | Height of a typical aircraft | 0.010 NM | B777-300ER |
|  | Average relative velocity between two aircraft necessary to infringe the planned longitudinal spacing | 100 kt | EMA handbook [1] |
|  | Average relative velocity between two aircraft, across track | 1 kt | EMA handbook [1] |
|  | Average relative velocity between two aircraft, vertically | 1.5 kt | ICAO SASP safety assessment [2] |
|  | Average time to fly the segment | Estimated by data |  |
|  | Proportion of aircraft pairs with initial separation t | Estimated by data |  |
|  | Probability of the loss of longitudinal separation for t minutes or more | Estimated by data |  |

In order to estimate the parameters, a pair of aircraft is extracted based on the following conditions.

- The pair flies on the same flight level and in the same route segment, and maintains the same flight level along the entire segment.

- The pair does not apply distance-based separation nor mach number technique.

- The initial time separation is at least 15 minutes.

T is easily calculated from the obtained pairs of aircraft. However,  is difficult to estimate, because the data is obtained based on the 15 minutes time-based separation, while the risk of collision must be estimated under 10 minutes separation. The obtained initial time separation () is shown as blue bars in Fig. A-1. This time,  is estimated from  based on the following expression as it appears in Ref. 3. It is shown as red bars in Fig. A-1.

(2)

The larger the initial time separation is, the less risk of collision is. Therefore, aircraft pairs with more than 20 minutes initial time separation are not considered, where the risk of collision is assumed to be zero.

Fig. A-1	 vs. initial time separation.

Next, the relationship between the relative TAS and the separation loss time is verified. Note that the relationship tends to be weak when the initial time separation is big. Below, only the data when the initial time separation is 30 minutes or less is used, and the number of the data sets is defined as n. i th aircraft loss time  and the relative TAS  are assumed to have the following linear relationship.

(3)

a and b are the regression parameters and  is the error term. These two parameters are optimized to minimize the error by least mean square method.

(4)

When the parameters are optimized, the remaining error is obtained, which is defined as the refined loss time () which excludes the effect of the relative TAS.

(5)

The refined loss time is obtained as shown in Fig. A-2. Note that the refined loss time is discretized by 1 minute. The refined loss time is fitted by double exponential distribution function defined as follows:

(6)

where  and  are the parameters of the model. These two parameters are set by the maximum likelihood estimation method, i.e. to maximize the following equation.

(7)

Fig. A-2	Refined loss distribution and fitted model.

The obtained fitted model is defined as .

In order to calculate the risk of collision, not only the refined loss time but also the relative TAS must be considered. Therefore, the relative frequency of the relative TAS is also required. According to our pre-testing, the distribution of the relative TAS is affected by the initial time separation. Therefore, the relative frequency functions of the relative TAS where the initial time separation is 20 minutes or less () is considered as shown in Fig. A-3. The figure clearly shows that the distribution is asymmetric. Positive TAS indicates that two aircraft are getting closer. ATC avoids applying large relative TAS when the initial time separation is small.

Fig. A-3	Relative frequency of the relative TAS where the initial time separation is 20 minutes or less.

Finally,  considering the relative TAS is calculated based on the following equation.

(8)

is the probability that the separation loss is greater than t. However, the initial time separation t is discretized by 1 minute, so it is assumed that the initial time separation of t is a uniform distribution between  and .

Reference

[1] Asia/Pacific Region En-route Monitoring Agency (EMA) Handbook, International Civil Aviation Organization Asia and Pacific Office. (online),  available from < http://www.bangkok.icao.int/edocs/EMA_handbook_ver2.pdf >, (accessed 2012-1-16).

[2] Fujita, M., Nagaoka, S., and Amai, O., Safety Assessment prior to Implementation of 50 NM Longitudinal Separation Minimum in R220 and R580, ICAO SASP-WG/WHL/9 WP/14, (2006).

[3] Analysis and recommendation of the potential for the reduction of longitudinal separation in Pacific Ocean environment, Report No. DOT/FAA/CT-TN85/39, (1986).