THE THIRTY-SIXTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/36)

(San Diego, California, USA, 14-18 May 2012)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Effects of Major Eruption of the Sun in the Cross-Polar Region

(Presented by the Federal Aviation Administration)

SUMMARY

This information paper provides an update on the major eruption of the sun in the Cross Polar Region during the months of January and March of 2012.

- Introduction

  - During the months of January and March of 2012, there were multiple geomagnetic storms, which affected flights transiting the Cross-Polar region.   With these storms, the potential for high frequency (HF) communication problems and an increased exposure to solar radiation for flight crews and passengers exists.  These events can have negative affects to both aircraft operators and air navigation service providers (ANSP) in the region.  Primarily, the area affected was north of 75 degrees latitude including routes transiting Cross Polar Fixes ORVIT, NIKIN, PINAG, RAMEL, NALIM, and DEVID.  See Attachment A for Arctic FIR Map.

- Discussion

  - The following is a narrative description of two separate National Oceanic and Atmospheric Administration (NOAA) Space Weather Advisory’s that were issued during the increased solar activity in late January and early March of 2012.

    - January Solar Event: Category R2 (Moderate) radio blackouts were observed on 23 and 27 January due to flare activity from active sunspot Region 1402. A category S1 (Minor) solar radiation storm was observed 23 January through 29 January.  A category S2 (Moderate) was observed on 23-25 and 27-29 January, and a category S3 (Strong) was observed on 23-24 January due to solar activity from active sunspot Region 1402. A category G1 (Minor) geomagnetic storm occurred on 24 January due to a Coronal Mass Ejection (CME).  See Attachment B for NOAA Space Weather Scales.

    - March Solar Event:  A category G2 (Moderate) geomagnetic storm was observed on 7 March, in response to the arrival of a CME associated with the X1.2b (X-class flares are large; they are major events that can trigger planet-wide radio blackouts and long-lasting radiation storms) solar flare on 5 March. A category G1 (Minor) to G3 (Strong) geomagnetic storm was observed from 8-10 March, in response to the arrival of a CME, associated with the X5.3B solar flare on 7 March. A category S1 (Minor) to S3 (Strong) solar radiation storm was observed 7-12 March due to solar activity from active sunspot Region 1429.  Category R3 (Strong) radio blackouts were observed on 5 and 7 March due to flare activity from sunspot Regions 1429 and 1430.  See Attachment B for NOAA Space Weather Scales.

2.2	The following are some of the effects that ATC experienced during the solar events in January and March 2012.

2.2.1 	For a six-day period during the March event, sporadic HF communications prompted Edmonton Area Control Center, ACC, (CZEG), to request Anchorage Air Route Traffic Control Center, ARTCC, (ZAN) Traffic Management to change the separation parameters in the Track Advisory (TA) Program.  This change, from 10 minutes to 15 minutes, helped mitigate potential compacted demand in event of lost communications.  This also allowed CZEG a better opportunity to increase the longitudinal separation used during this period. Lessons learned from the January solar event may have prompted this action.

2.2.2	A noticeable shift in traffic occurred during both solar events.  As an apparent result of the solar activity, operators’ flight planned via routes that transited the southern portion of the Arctic region.  ZAN Traffic Management Units (TMU) also observed increased traffic numbers utilizing the Russian Trans East (RTE) route structure.  See Attachment C for Solar Event Traffic Count.

2.2.3	The 23-29 January solar activities influenced the majority of air traffic to file routes no further north than the RTE fix ORVIT. ORVIT traffic normally averages seven aircraft per day, but the number increased to an average of 17 per day during the events. The highest traffic count occurred on 27 January, with 26 aircraft utilizing the RTE fix ORVIT.

2.2.4	The 7-8 March solar activity forced air traffic even further south.  An increased number of flights utilized routes that transited PILUN and LISKI.  Although, there was still a small contingent of operators that flew routes over RAMEL and NALIM.  Both PILUN and LISKI saw a dramatic increase in traffic between 7-14 March.  PILUN normally averages two aircraft per day, but this number increased to a high of 17 on 7 March.  LISKI traffic also saw an increase in traffic, with its high also occurring on 7 March.

2.2.5	During the solar events, both the Canadian National Operations Centre (NOC) and FAA Air Traffic Control System Command Center (ATCSCC) assisted in the dissemination of information concerning potential adverse conditions in the polar region.

2.2.6	The increased demand over the southern Polar and RTE fixes did not create an unmanageable scenario for ATC at either CZEG or ZAN.

2.3	The following are some of the concerns communicated/experienced by the affected air carriers during the January and March solar events.

2.3.1	Solar radiation exposure concerns for crew and passengers prompted operators to route flights south of their preferred and most cost effective routes.

2.3.2	The increase in separation parameters set forth in the Track Advisory program reduced the flexibility for operators filed via the affected Cross-Polar and RTE routes.

2.3.3	Limited reliable HF communications forced aircraft operators to use other communication methods to make mandatory position reports and altitude change requests. SAT phone use was increased during these solar events, and at times, communications were impossible.

2.3.4	Traffic congestion on the most popular tracks limited the availability of preferred altitudes for aircraft operators.

- Conclusion

  - The meeting is invited to note the information provided.