THE THIRTY-SIXTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/36)

(San Diego, California, USA, 14-18 May 2012)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Dynamic Airborne Reroute Procedures (DARP) Operations

(Presented by Civil Aviation Bureau, Japan and the Federal Aviation Administration)

SUMMARY

This information paper provides an update on the operational DARP trial between Oakland Air Route Traffic Control Center (ARTCC) and Fukuoka Air Traffic Management Center (ATMC).

- Introduction

  - Aircraft operators have been using DARP unrestricted in the South Pacific between Oakland Flight Information Region (FIR) and Auckland FIR.  Aircraft operators may now also DARP within and through the Nadi FIR.

  - On 30 April 2011, FAA and JCAB began an operational trial for use of DARP for flights between Japan and Hawaii, with published procedures to test its application.  When utilized, the DARPs have been very successful.

- Discussion

  - The FAA and JCAB have been in discussions regarding the expansion of westbound DARPs for several months.  During that time, several operators have expressed interest in participating in an initial demonstration flight and subsequent operational trials.

  - The requirements established for DARP usage on flights between Hawaii and Japan would be utilized for this expended operation.  They are:

    - Operational Controller Pilot Data Link Communications (CPDLC) is required for aircraft requesting airborne DARP reroutes.

    - A DARP request must be made within Oakland Oceanic Control Area and at least one (1) hour prior to crossing the Oakland/Fukuoka common boundary.  This is necessary to permit adequate time for clearance delivery and Air Traffic Service Inter-Facility Data Communications (AIDC) messaging to take place.

    - The downlink CPDLC reroute request should be made by the flight crew at least 20 minutes before the divergence waypoint to allow processing time by controller and flight crew.

    - Oakland ARTCC has published the DARP operational requirements to the operators via a KZAK International Notice to Airmen (NOTAM).

    - Eligible flights would need to be on routes that remain entirely within the Fukuoka and Oakland Oceanic Control Areas.

  - Under the current DARP trial between the Oakland and Fukuoka FIRs, all DARP clearances must be issued in the Oakland FIR.  JCAB is developing the capability to initiate DARP clearances in the Fukuoka FIR.  JCAB DARP capability is expected to be completed near the end of 2012.

  - Fukuoka ATMC and Oakland ARTCC will consider operator requests to trial DARPs between different city pairs other than Hawaii to Japan.  Operators wishing to trial DARP procedures between other city pairs with destinations in Japan should coordinate the trials with Dennis Addison (dennis.addison@faa.gov) and Kenji Okuba (okuba-k07xh@cab.mlit.go.jp).

- Conclusion

  - The meeting is invited to discuss the paper; and

  - Note the efficiency gains of the DARP.