THE THIRTY-SIXTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/36)

(San Diego, California, USA, 14-18 May 2012)

Agenda Item 5:  	Air Traffic Management (ATM) issues

PACOTS for NOPAC Routes

(Presented by ENRI)

SUMMARY

Eastbound TRACKs of PACOTS are restricted in the airspace bounded by the ATMC North Pacific Composite Route System (NOPAC). Alternative PACOTS condition by restriction relaxed is examined. ATC operational problem is also examined.

- Introduction

  - At the IPACG/33, Anchorage ARTCC proposed three models about flight route of NOPAC. Model 1 is to change the flight direction to bi-direction. Model 2 is to use for UPR south from A590. Model 3 is to abolish the NOPAC restriction.

  - ENRI made new route network and analyzed distribution of UPR by three models. At the IPACG/34, ENRI shows possibility of model 1 and model 2 depend on weather condition.

  - At the IPACG/35, ATMC announced to propose an alternative route structure with ENRI at IPACG/36. ENRI examined engaged eastbound PACOTS by alternative route network. The proposed rule is improved model 2.

- Discussion

  - Eastbound route of NOPAC route is A590, R591 and G344 from north. Currently, Eastbound TRACKs of PACOTS are computed south of A590. In Fukuoka FIR, track does not branch from A590 or R591. In Anchorage FIR, track is available to branch form NOPAC only directed for south.

  - Tendency of new PACOTS and efficiency is depend on location of original PACOTS. It also depend on weather condition. The location is categorized to four type by the start gateway of TRACK 1.

Categorize:

- “A590, R591” means that TRACK1 is same as A590 or R591 in Fukuoka FIR

- “G344” means that TRACK 1 is same as G344 in Fukuoka FIR

- “KALNA” means that TRACK1 starts from KALNA but branch off  from G344 at KALNA

- “EMRON, LEPKI” means TRACK2 starts from EMRON or LEPKI.

The Start Gateway (or ROUTE) of TRACK 1 in FY2011 is shown as Figure 1.

Figure 1 Start Gateway (or ROUTE) of TRACK 1

- Route is tendency to north in summer and it is tendency to south in winter.

  - When TRACK 1 starts from EMRON or LEPKI (17% in annually), proposed route structure provide same PACOTS with current structure. When TRACK 1 starts from north from EMRON, proposed route structure is expected to improve flight time and fuel consumption.

  - PACOTS routes (TRACK 1, 2 and 3) by current route network are computed. PACOTS routes by proposed route network are also computed. Flight time, fuel consumption and route location are shown in Table 1. Bold letter means benefit as time is more than 5 min or benefit as fuel consumption is more than 1,000 lbs for B747-400.

Table 1 Comparison between current network with proposed network

| TRK1 |  | Current |  |  | Proposed |  |  | Benefit |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DAY |  | Gate | Time | Fuel | Gate | Time | Fuel | Time | Fuel |
| A590 | TRK1 | A590 | 7:54 | 176801 | A590 | 7:50 | 175502 | 4min | 1299lbs |
| DAY1 | TRK2 | KALNA | 8:44 | 191940 | R591 | 8:41 | 191004 | 3min | 936lbs |
|  | TRK3 | LEPKI | 9:32 | 206563 | LEPKI | 9:32 | 206563 | 0min | 0lbs |
| R591 | TRK1 | R591 | 8:02 | 179427 | A590 | 7:57 | 177730 | 5min | 1697lbs |
| DAY2 | TRK2 | G344 | 8:47 | 193191 | R591 | 8:44 | 192271 | 3min | 920lbs |
|  | TRK3 | LEPKI | 9:29 | 205913 | KALNA | 9:29 | 205913 | 0min | 0lbs |
| R591 | TRK1 | R591 | 8:06 | 179613 | POXED | 8:00 | 177758 | 6min | 1855lbs |
| DAY3 | TRK2 | KALNA | 8:37 | 189417 | KALNA | 8:36 | 189330 | 1min | 87lbs |
|  | TRK3 | LEPKI | 9:18 | 201814 | LEPKI | 9:18 | 201814 | 0min | 0lbs |
| G344 | TRK1 | R591 | 8:05 | 180539 | A590 | 8:03 | 179537 | 2min | 1002lbs |
| DAY4 | TRK2 | KALNA | 8:35 | 188724 | KALNA | 8:36 | 188668 | ▲1min | 56lbs |
|  | TRK3 | LEPKI | 9:16 | 201059 | LEPKI | 9:17 | 201135 | ▲1min | ▲76lbs |
| DAY |  | Gate | Time | Fuel | Gate | Time | Fuel | Time | Fuel |
| KALNA | TRK1 | KALNA | 7:50 | 174699 | POXED | 7:46 | 173206 | 4min | 1493 lbs |
| DAY5 | TRK2 | EMRON | 8:26 | 185657 | ADGOR | 8:20 | 183785 | 6min | 1872 lbs |
|  | TRK3 | LEPKI | 9:09 | 199058 | EMRON | 9:06 | 197957 | 3min | 1101 lbs |
| KALNA | TRK1 | KALNA | 7:36 | 170946 | A590 | 7:32 | 169863 | 4min | 1083lbs |
| DAY6 | TRK2 | EMRON | 8:10 | 181184 | R591 | 8:03 | 179321 | 7min | 1863lbs |
|  | TRK3 | LEPKI | 8:50 | 193613 | LEPKI | 8:48 | 192858 | 2min | 855lbs |

  - ATC operational problem is worried. Conflict with R591 traffic is expected when TRACK1 is branch from A590 and through R591. Same things for R591 and G344 is also concerned. Figure 2 shows PACOTS by proposed network at DAY4.From north, TRACK 1, 2, 3 and 14 are shown. Red line is TRACK 2. For one day, number of altitude change is not so different. When R591 is not used, the optimum route is changed from R591 to A590. Then traffic of A590 is increased.

Figure 2 Proposed PACOTS at DAY4

  - Flight profile sample is sown in Figure 3. Flight route in NEW is selected for optimum for PACOTS TRACKs and eastbound NOPAC route. NEW_R is exhibit to fly R591 and G344．NEW_R  reduced conflict by crossing TRACK1 and R591. But it increased traffic volume on A590.

ft

140E		170E		160W		    130W	     100W

Figure 3 Flight profile at DAY4

- Conclusion

  - The meeting is invited to note the information provided.