THE THIRTY-SIXTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/36)

(San Diego, California, USA, 14-18 May 2012)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

ATS route modification with decommission of navaids in Fukuoka FIR

(Presented by Civil Aviation Bureau, Japan)

SUMMARY

This information paper presents a summary of ATS route modification planning with decommission of NDB and VOR in Fukuoka FIR.

- Introduction

  - With the development of RNAV routes, JCAB has decided to decommission of navaids for the purpose of optimize maintenance cost.

  - Under circumstance of strict government budget cut, even if navaids composing oceanic ATS route that cannot to be replaced by RNAV route have been selected as facility to be decommissioned with no exception, after consideration of effect to operators and air traffic controllers.

  - All the NDB maintained by JCAB decommissioned in FY2011except terminal NDB for noise abatement use. 36 VORs are planned to be decommissioned by FY2015, 5 VORs were decommissioned in FY2011.

  - Concerning Oceanic routes, Okinawa NDB (OK) and Minami-Daito NDB (MD) were decommissioned in FY2011 and ATS routes R583 and R584 are modified.

  - R583 is modified so as to clear overlap between training area. Aircraft between Okinawa area and Iwo-to/ Hawaii have to detour. However, by the change of connecting point withWestbound PACOTS, the length of flight is shortened.

  - As for R584, route segment between AVLAS and KEITH is also modified to detour training area.

- Discussion

  - Following VORs concerning Oceanic ATS route will be scheduled to be decommissioned.

| - FY2012 | - Chinen VOR (TIC) |
| --- | --- |
| - FY2013 | - Miyakejima VOR (MJE) |
| - FY2014 to - FY2015 | - Choshi VOR (CVC), Daigo VOR (GOC), Minamidaito VOR (MDE), Onjyuku VOR (OJC), Tateyama VOR (PQE) |

  - ATS routes after decommissioned concerning VOR will be modified or revoked to compose adequate route structure based on careful examination of route utilization by operators.

  - JCAB is considering to designate part of west of SALVA on Oceanic ATS route R584 as CDR2 route with decommission of Chinen VOR in FY2012. The segment of CDR2 will be usable while training areas are cold. While training areas are hot, aircraft have to detour via SAKON R595 KEITH.

  - To ease effect of detour, JCAB will keep on consideration and coordination for exploring possibility of implementing UPR and RNP route.

- Conclusion

  - The meeting is invited to note the information provided.