THE THIRTY-SIXTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/36)

(San Diego, California, USA, 14-18 May 2012)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Request for RJAA/NRT Narita Airport Arrival Route Review

(Presented by IATA)

SUMMARY

This informational paper identifies IATA’s concerns with Narita’s published arrival routings. IATA is greatly appreciative of JCAB’s recent effort to increase the number of available slots by optimizing air traffic flows at Tokyo’s New Narita International Airport.  At the same time the slots were optimized, Narita’s arrival routes were significantly modified.  IATA requests a review by the representatives from JCAB on these arrival routes.

- Introduction

  - Effective with the October 20th, 2011 AIRAC cycle update, airline operators were asked by JCAB to change the arrival routes into Narita per AIC 35/11 “Change of Flight Planned Routes.”   The following describes the three significant arrival route changes where United Airlines has noted an operational impact.

- All flights that enter Japanese airspace from Russia via waypoints AKSUN, LUMIN, ANIMO, BISIV, and ODERI) via CHE (Chitose VOR) are no longer allowed to file airway Y10, and instead are required to file airway Y12.  Reference attached (Attachment A)

- Extended/modified routing on airway Y30 SUIGO-DOVER-AKTUS-MELON prior to final approach vectors into Narita.  Reference attached (Attachment B – circled in Red)

- All flights that are arriving on R220 via NANAC, the relocation of waypoints KETAR and MELON.  Reference attached (Attachment C)

- Discussion

  - Prior to these changes above, airline operators were able to file a much shorter and fuel/emission efficient route to Narita.  For instance, the combination of changes A & B above has resulted in 84NM mileage increase as we have to fly near Niigata VOR (GTC) to proceed southbound, and then plan to fly the extended Y30 before given final approach vectors to Narita.  For the KETAR-MELON relocation, (C above), the distance increase is 13NM.

  - With the stated mileage increases above, United Airlines sees an operational impact associated with fuel and CO2 emissions increases as the following;

CHE (Chitose)–RJAA – Mileage Increase  is *84NM*

UAL79 New York-Tokyo/Narita Flight Planned Route & Data

Route: CHE.Y12.HWE.Y122.OBAKO.Y128.KAMOH.Y303.SWAMP.Y30.MELON..NRT

FUEL Increase (Per Flight)                    		+3,100 LBS 		(1,406 KG)

FUEL ANNUAL Increase (Per Flight)		+339,450 LBS		(157,972 KG)

CO2 Emissions Increase (Per Flight)			+9,610 LBS 		(4,359 KG)

CO2 ANNUAL Emissions Increase (Per Flight)	+1,092,295 LBS	(495,457 KG)

Flight Time Increase/Flight				+10 minutes

NANAC–RJAA - Mileage Increase  is *13NM*

UAL7 Houston – Tokyo/Narita Flight Planned Route & Data

Route: NANAC.Y810.KETAR.Y811.MELON..NRT

FUEL Increase (Per Flight)                    		+800 LBS 		(363 KG)

FUEL ANNUAL Increase (Per Flight)		+262,800 LBS		(119,204 KG)

CO2 Emissions Increase (Per Flight)			+2,480 LBS		(1,125 KG)

CO2 ANNUAL Emissions Increase (Per Flight)	+814,680 LBS 		(369,537 KG)

Flight Time Increase/Flight				+2 minutes

  - Dynamic en route winds and Jet Stream position often dictate where North American flights enter Japanese airspace throughout the year.  Airline operators consider Chitose arrival route to Narita an important route option in addition to NOPAC routes, particularly for flights departing from (but not limited to) New York, Washington DC, Chicago, Atlanta, Detroit,  Toronto, Houston and Dallas, showing many operational benefits especially in the winter months.

  - Airline operators are always appreciative of receiving available shortcuts based on actual traffic demand and flows on a day to day basis, but would certainly welcome the opportunity to work with JCAB to coordinate a shorter arrival route option to Narita that airline operators can actually file and fly so the impact of less fuel carried and less emissions can be realized and contribute to a smoother traffic flow to the biggest international gateway to Japan.

- Conclusion

The meeting is invited to

- Note the information provided.

Attachment A

Pink –New Route 520NM

Blue – Previous Route 436NM

Attachment B

Attachment C