THE THIRTY-SIXTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/36)

(San Diego, California, USA, 14-18 May 2012)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Feedback on the advantages of Dynamic Airborne Reroute Procedures (DARP) Operation

(Presented by All Nippon Airways / ANA )

SUMMARY

This information paper introduces the feedback of DARP operation.

- Introduction

  - ANA has positively participated DARP TRIAL between Hawaii and Japan with B777 and B767. And ANA is also initiating DARP verification flight between Japan and West Coast (LAX). Please see the attachment for the detail regarding our effort so far.

- Discussion

  - DARP introduction effect

During the verification and evaluation period between Hawaii and Japan, 64 flights of 271 flights implemented DARP TRIAL, which resulted in the total reduction of fuel consumption by 42400lbs/142min, average reduction of fuel by 663lbs/3min and maximum reduction of fuel by 4200lbs/21min. Other 207 flights were... route had no merit (132 flight), rejected by ATC (3 flight), rejected by Flight Crew (38 flight), others (34 flight - mainly communication error caused by ACARS UPLINK)

2.2.       Challenges to improve DARP executing rate

In order to improve the DARP executing rate, we are currently “Visualizing the effect of DARP and showing the result to both Flight Crews and Flight Dispatchers” and “Alerting both Flight Crews and Flight Dispatchers that they should recognize the potential risk of communication error and the importance to check the communication status each other”.

2.3.       Technical issues

We faced 2 major issues during DARP operation. Please see the attachment for the detail regarding these issues.

2.4.       Summary

Honolulu DARP TRIAL has definitely contributed to fuel save operation and still have potential growth in executing rate. ANA will continue the DARP verification flight with West Coast flights.

2.5　　 Requests

Expansion of DARP TRIAL is strongly recommended (Ex. Between West Coast and Japan Flights).Also Acceleration of DARP capability implementation into FUKUOKA FIR is strongly recommended for further fuel savings.

- Conclusion

  - The meeting is invited to note the information provided.