THE THIRTY-SEVENTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/37)

(Tokyo, Japan, 29 October - 2 November 2012)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

STATUS UPDATE FOR DEVELOPMENT AND IMPLEMENTATION

OF ADS-C CLIMB DESCEND PROCEDURE

(Presented by the Federal Aviation Administration)

SUMMARY

This information paper provides an update on development and implementation of operational trials for the use of the Automatic Dependent Surveillance – Contract (ADS-C) Climb Descend Procedure (CDP).

- Introduction

  - The Federal Aviation Administration (FAA) is analyzing a new oceanic ATC procedure, termed ADS-C CDP, in the operational environment.  ADS-C CDP utilizes existing user equipage and air traffic control (ATC) capabilities to allow more oceanic flights to achieve their preferred vertical profiles.  The procedure would address present day performance gaps in the area of capacity, productivity, and efficiency in the oceanic environment.  Integral to ADS-C CDP, is the use of advanced communication, navigation, and surveillance (CNS) capabilities, such as, ADS-C, Controller-Pilot Data Link Communications (CPDLC), and Required Navigation Performance (RNP).

- Discussion

  - Currently, ADS-C CDP is being demonstrated operationally by manually applying ADS-C CDP requirements without requiring software changes to Ocean21, and is limited to use between RNP 4 authorized aircraft.  Based on a benefits analysis conducted in 2007, it was anticipated the ADS-C CDP could be applied once or twice per day in an operational trial.  To date that has not been the case; although, it is anticipated that with introduction of more RNP 4 authorized aircraft in the Pacific Region, the number of opportunities to use the procedure will increase.

  - The current trial has been extended until 16 February 2013 to gather more data. Additional aircraft data would be beneficial to help validate the application of the procedure.  The FAA is also investigating the use flight test bench simulators to create scenarios for application of the ADS-C CDP to gather analysis data in that way.

  - The FAA is advancing the ADS-C CDP as a global air traffic procedure by leading the effort for endorsement at the ICAO Separation and Airspace Safety Panel (SASP).  The FAA has submitted working papers in support of the procedure and the SASP has agreed to progress the work on the CDP and to develop a requirement to support the SASP Math Subworking Group (MSG) work in relation to the CDP safety assessment.

  - The FAA developed the draft ICAO Circular “Safety Assessment for the Development of Separation Minima and Procedures for a Climb/Descend Procedure (CDP) Using Automatic Dependent Surveillance – Contract (ADS-C)”.  The Circular was developed for ICAO to provide information on the work undertaken by the SASP to establish the separation minima to be used in applying changes of level using the CDP.  It is aimed at a worldwide audience of the civil aviation regulatory authorities and air navigation service providers responsible for implementing this and other separation minima.

  - At the SASP Twenty-first Meeting of the Working Group of the Whole (SASP-WG/WHL/21), 29 October – 09 November 2012, in Cairns, Australia, the FAA will further the current development work of the SASP in relation to advancing the ADS-C CDP as a global procedure.

  - The ADS-C CDP Project, the NAS Lifecycle Planning (NLP) Project Level Agreement (PLA), “TBO – Automatic Dependent Surveillance – Contract (ADS-C) Climb and Descent Procedure (CDP) Automation Implementation, FY12” includes the following artifacts:

- ADS-C Initial Business Case Analysis (August 2007)

- ADS-C In-Trail Procedure Concept Review (September 2007)

- ADS-C In-Trail Procedure for an Operational Trial (October 2008)

- ADS-C In-Trail Procedure Operational Trial ICAO Petition (April 2009)

- Safety Risk Management Document (SRMD) package (November 2010)

- ADS-C CDP FAA Notice (November 2010)

- ADS-C CDP Operational Trial Controller Briefing (December 2010)

- ADS-C CDP NOTAM (February 2011)

- Estimated Benefits of ADS-C CDP in New York Oceanic Airspace (January 2011)

- ADS-C CDP Operational Trial Quick Look Report (August 2011)

- Program Requirements for ADS-C CDP (February 2012)

- ADS-C CDP Benefits Analysis (April 2012)

  - The ADS-C CDP NLP PLA implements the ADS-C CDP within the Advanced Technologies and Oceanic Procedures (ATOP) automation platform in fiscal year 2015.  ADS-C CDP will become an enhancement to ATOP so the entire procedure is automated and will include new conflict probe rules.

- Conclusion

  - The meeting is invited to note the information provided.