THE THIRTY-SEVENTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/37)

(Tokyo, Japan, 29 October - 2November 2012)

Agenda Item 5:  		Air Traffic Management (ATM) Issues

User Preferred Routes in Association with PACOTS Tracks Update

(Presented by Federal Aviation Administration and Civil Aviation Bureau, Japan (JCAB))

SUMMARY

This information paper provides clarification on the available User Preferred Routes that can be flight planned in conjunction with published PACOTS tracks and any associated constraints.

Introduction

  - For the past 12 years User Preferred Routes (UPRs) have been allowed in the Pacific Region.  The usage of UPRs has grown significantly during that time, with trials of UPRs in conjunction with organized tracks ongoing. UPRs in conjunction with PACOTS Tracks began August 2009 allowing aircraft transiting between Japan and Hawaii to file random routes as long as specific restraints were adhered to.  Since then the practice has been expanded throughout the Pacific.

  - The following discussion will serve to clarify the current guidelines for flying random routes in association with the various PACOTS Tracks where UPRs are allowed

Discussion

  - UPR Trials in association with PACOTS Tracks 1, K and the Oceania UPR have been in daily use for over 1 year.  JCAB and the FAA have monitored these Trials for benefits and any operational issues.  To date, no significant issues have been identified.  These three UPR Trials have been moved from a trial status to normal daily operation.  The guidelines for flight planning these UPRs remain unchanged and are published in AIP (Aeronautical Information Publication) Japan and the FAA Pacific Chart Supplement.

  - UPR Flight Planning

    - PACOTS Track UPRs must remain in the fukuoka, anchorage and oakland oceanic control areas.

    - The upr must utilize a published standard departure routing where applicable.

    - Operators must utilize acceptable gateways and fixed routes within Fukuoka FIR, applicable to the particular PACOTS track UPR being flown.  The gateways and fixed routes pertinent to the particular PACOTS track UPR are contained within AIP Japan.

    - Operators must utilize acceptable transition routings when transiting Anchorage FIR.

      - Eastbound transition routes:

A590 transition route:	PASRO A590 POWAL

R591 transition route:	AKISU R591 ASPIN

G344 transition route:	CUTEE G344 CARTO; or

CUTEE 48N170E 49N180E (or point north of 49N180E); or

CUTEE 49N170E; or

CUTEE 50N170E

      - Westbound transition routes:

Join R220 at any named point at or east of NATES

Join R580 at any named point at or east of ONEIL

Join R591 at any named point at or east of AKISU

Join G344 at any named point at or east of CARTO

    - Operators must utilize acceptable gateways and fixed routes within Vancouver, Seattle and Oakland FIRs.  The gateways and fixed routes pertinent to the particular PACOTS track UPR are listed in the Pacific Chart Supplement.

    - The Upr must be planned to avoid military special use airspace when active.

    - The UPR must utilize a published STAR where appropriate.

    - Operators will be informed whenever a condition exists that does not allow the use of UPRs within a particular FIR.  Conditions that may not allow the use of UPRs include large scale military operations and typhoons.

  - PACOTS Track 1 UPR Procedures. In association with PACOTS Track 1 between Asia and North America the following procedures must be used when flight planning UPRs:

    - The UPRs must remain 50 NM north of  PACOTS Track 2.

    - UPR aircraft do not have priority for altitude assignment over aircraft on an existing PACOTS route or NOPAC traffic.

    - The UPR route must not cross the Oakland/Anchorage fir boundary multiple times.

  - PACOTS Track 3 UPR Procedures. In association with pacots track 3 between asia and North America the following procedures must be used when flight planning UPRs:

    - The upr must remain 50nm south of pacots track 2.

    - Upr aircraft do not have priority for altitude assignment over aircraft on an existing pacots route or central east pacific (cep) traffic.

  - PACOTS Track H/I UPR Procedures. In association with PACOTS Track H/I between North America and Southeast Asia the following procedures must be used when flight planning UPRs:

    - The upr must enter oceanic airspace over one of the existing waypoints on the Oakland Oceanic fir boundary along the west coast of North America.

    - Flights entering the Fukuoka fir from the Anchorage fir must be established on a nopac ats route and cross the Anchorage/Fukuoka boundary between 0000 UTC and 0600 UTC.

    - Operators may flight plan on PACOTS Track F and then diverge so as to remain 50 NM south of pacots Track F.

    - Pacots Track H/I upr aircraft do not have priority for altitude assignment over aircraft on an existing pacots, nopac or Central East Pacific (cep) traffic.

  - PACOTS Track K UPR Procedures. In association with PACOTS Track K between North America and Southeast Asia the following procedures must be used when flight planning UPRs:

    - The upr must enter oceanic airspace over one of the existing waypoints on the Oakland Oceanic fir boundary along the west coast of North America.

    - Flights must join a NOPAC ATS Route when transiting the Anchorage FIR.  However, the upr must not utilize ats route r591 and/or g344 if it is designated as an eastbound pacots track.

    - Flights entering the Fukuoka fir from the Anchorage fir must be established on a nopac ats route and cross the Anchorage/Fukuoka boundary between 1400 UTC and 2100 UTC.

    - Operators must flight plan so as to remain 50 NM south of pacots Track J.

    - Pacots Track K upr aircraft do not have priority for altitude assignment over aircraft on an existing pacots, NOPAC or Central East Pacific (cep) traffic.

  - PACOTS Track 11/12/A/B UPR Procedures. In association with PACOTS Track 11/12/A/B between Asia and Hawaii the following procedures must be used when flight planning UPRs:

    - For eastbound flight planning between Hawaii and Asia:

      - UPRs may flight plan on PACOTS Track 3 and then diverge so as to remain at least 50 NM south of PACOTS Track 3.

      - Aircraft on PACOTS tracks between Asia and the continental u.s. have priority over Asia-Hawaii UPR aircraft for altitude assignment.

      - Aircraft on PACOTS tracks between Asia and Hawaii do not have priority over Asia-Hawaii UPR aircraft for altitude assignment.

      - The UPR route must be flight planned to remain laterally north of ATS route A450 or flight plan via A450 between taavr and kaths.

      - The UPR shall be planned to incorporate a published waypoint on the Honolulu Control Facility boundary.

      - The necessary information regarding available Hawaiian gateway fixes will be sent to participating aircraft operators.

      - Operators may contact Oakland ARTCC to be added to the daily publication of available Hawaiian Gateway fixes.

    - For westbound UPR flight planning between Hawaii and Asia:

      - Aircraft on PACOTS tracks between Asia and Hawaii do not have priority over Hawaii-Asia UPR aircraft for altitude assignment.

      - The UPR route must be flight planned to remain laterally north of ATS route A450 or flight plan via A450 between kaths and taavr.

      - Operators may contact Oakland ARTCC to be added to the daily publication of available Hawaiian Gateway fixes.

      - The UPR shall be planned to incorporate a published waypoint on the Honolulu Control Facility boundary.

      - Aircraft departing Hawaii after 2330 UTC must flight plan so as to remain 50NM south of PACOTS Track F.

      - In the Fukuoka FIR, operators must file one of the acceptable fixed routes published in AIP Japan.

- Conclusion

  - The meeting is invited to note the information provided.