FAA Implementation:

http://www.faa.gov/go/fpl2012

Information and status of States:

http://www2.icao.int/en/FITS/Pages/home.aspx

# 2012 Flight Plan

ALTRV	altitude reservation	HUM	Humanitarian

ATFMX	exempt from ATFM	FFR	Fire fighting

FLTCK	Flight Check		HEAD	Head of State

HAZMAT	Hazardous Materials	SAR	Search and Rescue

MEDEVAC	Life-critical medical flight	HOSP	Medical Flight

MARSA	Military assumes responsibility. for separation

NONRVSM	Non-RVSM requesting operations in RVSM airspace

STATE	Military, customs, or police

#### STS/ Special Handling

#### Reasons

FPL 2012 Quick Guide

(FPL-ACID-Flt Rules Flight Type

- AC Type/Wake Cat-Equip./Capability

- Departure EOBT

- Speed  Altitude [sp] Route

- Destination  ETE [sp] Alternate(s)

- Other Information  )

Example:

(FPL-TTT123-IS

-C550/L-SDE1E2HIJ3J5RGW/SD1

-KPWM1225

-N0440F310 BRUWN2 ACK J62

RBV DCT CHS SSI3

-KJAX0119 KCRG

-PBN/A1B1D1 NAV/RNVD1E2A1

REG/N111A SEL/BPAM CODE/A519D9

DLE/RBV0100 ORGN/KHOUARCW

TALT/8ME2)

A1 RNAV 10    (RNP10)

RNAV 5

B1  All

B2  GNSS

B3  DME/DME

B4  VOR/DME

B5 INS or IRS

B6  LORANC

RNAV 2

C1  All

C2  GNSS

C3  DME/DME

C4  DME/DME/IRU

RNAV 1

D1  All

D2  GNSS

D3  DME/DME

D4   DME/DME/IRU

#### PBN/ capabilities (8 max)

L1 RNP 4

RNP 1

O1  All

O2  GNSS

O3  DME/DME

O4  DME/DME/IRU

Approach

S1 RNP APCH

S2 RNP APCH

w/BARO VNAV

AR Approach

T1 RNP AR APCH

With RF

T2 RNP AR APCH

Without RF

FAA still requires NAV/RNVD1E2A1

For RNAV 1

A less than 91 knots IAS

B at least 91   and less than121 knots IAS

C at least 121 and less than 141 knots IAS

D at least 141 and less than 166 knots IAS

E greater than 166 and less than 211 knots IAS

H Helicopters

#### PER/ Performance Cat.

Categories based on Vref if specified, or 1.3Vso, each at maximum certificated landing weight per CFR 97.3

#### Some Key FAA Requirements

Version 2   					 August 27, 2012

If you are eligible for Oceanic 50 NM lateral separation

PBN/A1 or PBN/L1 in Field 18

R in Field 10a

See International NOTAMs and FAA Order 8400.12

If you are eligible for Oceanic 50 NM longitudinal separation

PBN/A1 or PBN/L1 in Field 18

D1 in Field 10b

(J5, J6, or J7) and R in Field 10a

See International NOTAMs and FAA Order 8400.12

If you are eligible for Oceanic 30 NM (30/30) separation

PBN/L1 in Field 18

D1 in Field 10b

(J5, J6, or J7) and R in Field 10a

See International NOTAMs and FAA Order 8400.33

If you are authorized for RNAV 1 Arrivals / Departures

R in Field 10a

PBN/D1, D2, or D4 in Field 18

and

NAV/RNVD1E2A1 in Field 18

See AIP ENR 1.10 sec. 12 and Advisory Circular 90-100A

If you are authorized for RVSM

W in Field 10a

See AIP ENR 1.1 sec. 38

If you are authorized for ADS-B (UAT)

U1 or U2 in Field 10b and SUR/282B in Field 18

CODE/ in Field 18

See Advisory Circular 20-165, TSO-C166b

If you are authorized for ADS-B (1090-ES)

B1 or B2 in Field 10b and SUR/260B in Field 18

CODE/ in Field 18

See Advisory Circular 20-165, TSO-C154c

Remarks

Insert in RMK/ any information instructed to be placed in Remarks or NAS Field 11

Note:

Remember to always file the a/c registration number in Field 18, REG/, when the callsign is not the registration or when planning CPDLC or ADS-C login.

N No capability- include no other entries if filed

Transponder (file no more than one letter)

A Mode A

C Mode A and C

S Mode S, ACID and Altitude

P Mode S, Altitude, no ACID

I Mode S, ACID, no Altitude

X Mode S, no ACID, no Altitude

E Mode S, ACID, Altitude, extended squitter

H Mode S, ACID, Altitude, Enhanced Surveillance

L Mode S, ACID, Altitude, Enhanced Surveillance, extended squitter

ADS-B

B1 1090 MHz out capability, or

B2 1090 MHz out and in capability

U1 UAT out capability, or

U2 UAT out and in capability

V1 VDL Mode 4 out capability, or

V2 VDL Mode 4 out and in capability

ADS-C

D1 ADS-C FANS-1/A, and/or

G1 ADS-C ATN

#### Field 10a                                                           Field 10b                                                         Field 18

N No capabilities

Include no other entries if filed

S Standard

A GBAS Landing Sys.

B LPV (APV w/SBAS)

C LORAN C

D DME

E1-E3 ACARS

E1 FMC WPR

E2 D-FIS

E3 PDC

F ADF

G GNSS

H HF RTF

I INS

J1 CPDLC ATN

J1 VDL Mode 2

J2-J7 CPDLC FANS 1/A

J2 HFDL

J3 VDL Mode A

J4 VDL Mode 2

J5 Satcom Inmarsat

J6 Satcom MTSAT

J7 Satcom Iridium

# Other Information

# (File in this order)

STS/	Special Handling (see list)

PBN/	Performance Based Navigation (see list)

NAV/	Other Navigation Capability (see FAA rqmts)

COM/	Other Comm. Capability

DAT/	Other Data Application or Capabilities

SUR/	Other Surveillance Capability (see FAA rqmts)

DEP/	Non-standard Departure (e.g. MD24)

DEST/	Non-standard Destination (e.g. EMI090021)

DOF/	Date of Flight (e.g. 121123)

REG/	Registration (if not in callsign) (e.g. N123A)

EET/	Estimated Elapsed Times (e.g. KZNY0124)

SEL/	SELCAL (e.g. BPAM)

TYP/	Non-standard AC Type

CODE/	24-bit address (e.g. A519D9)

DLE/	Delay (at a fix) (e.g. EXXON0120)

OPR/	Operator

ORGN/	Flight Plan Originator (e.g. KHOUARCW)

PER/	Performance Category (e.g. A)

ALTN/	Non-standard Alternate(s) (e.g. 61NC)

RALT/	Enroute Alternate(s) (e.g. EINN CYYR KDTW)

TALT/	Take-off Alternate(s) (e.g. KTEB)

RIF/	Route to revised Destination

RMK/	Remarks

K MLS

L ILS

Satcom RTF

M1 Inmarsat

M2 MTSAT

M3 Iridium

O VOR

P1-P9 RCP

Reserved

R PBN

T TACAN

U UHF RTF

V VHF RTF

W RVSM

X MNPS

Y 8.33 kHz VHF

Z Other Cap.

Notes:

- Filing R requires a PBN/ entry in Field 18

- Filing Z requires a NAV/, COM/, or DAT/ entry in Field 18

- Standard equipment is VOR, VHF, and ILS

- File Field in the order shown (e.g. SDGIRW)

Note:

Do not use an oblique stroke except as part of an indicator.  Do not use special characters in Field 18 text.

Note:

- Include items  as applicable for transponder, ADS-B, and ADS-C.   File “N” only  if none of the capabilities are applicable.