THE THIRTY-SEVENTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/37)

(Tokyo, Japan, 29 October - 2 November 2011)

Agenda Item 5:  	Air Traffic Management (ATM) Issues

Operational procedures of ATS data link by using Iridium satellite within Fukuoka FIR

(Presented by Civil Aviation Bureau, Japan)

SUMMARY

This information paper provides the information about the reason why Iridium satellite cannot be used within Fukuoka FIR and the temporary operational procedures of ATS data link by using Iridium satellite within Fukuoka FIR, which has been effective since 18th October, 2012.

- Introduction

  - The trial operation of Iridium satellite in the Fukuoka FIR was implemented for Continental  Airlines, which was from Guam to Nagoya, for the first time in September,2009 based on the agreement of IPACG/36 ,2009.

  - After that the trial had expanded the applicable aircraft and continued for about a year since December,2009 in order to collect data of ADS/CPDLC down link message.

  - JCAB reported that it was premature for JCAB to race to judgment since the number of sample was small in the IPACG/32, 2010.

  - JCAB has started the trial operation since 2011 again for Hawaiian Airlines for the purpose of collecting data. On the other hand, some airlines other than Hawaiian Airlines have been brought to light that they have been using Iridium satellite for data link within Fukuoka FIR.

  - ADS distance based longitudinal separation using ATS data link by Iridium satellite cannot be applied within Fukuoka FIR. JCAB are now considering to make the framework of evaluation of Iridium satellite. Until the end of the evaluation, JCAB has to publish the interim operational procedures in order to divide Iridium-using aircraft from the others.

- Discussion

- Although JCAB continues to evaluate the data link operation based on the FANS-1/A Operations Manual (FOM), JCAB also applies  Global Operational Data Link Document (GOLD) for the safety purposes. JCAB is expected to establish the official evaluation scheme based on GOLD in 2014.

Gateway1

Gateway2

Gateway1

Gateway2

ADS distance based longitudinal separation using ATS data link by Iridium satellite cannot be applied within Fukuoka FIR because data that was submitted from Data Link Center System(DLCS) of Kobe Aeronautical Satellite Center was below the standard of GOLD. That’s why JCAB has decided not to apply the ADS distance based longitudinal separation. See fig1 and 2 below.

Gateway1

Gateway2

Gateway1

Gateway2

Fig 1: FOI performance against RCP240/RSP180

Gateway1

Gateway2

Fig 2: FOI performance against RCP400/RSP400

- JCAB hopes to start implementing the ADS distance based separation for the Iridium-using aircraft after confirming that the Iridium satellite meets the criteria of GOLD . In order to do that , JCAB would like to collect much data as soon as possible.

- Regarding the ATS data link operations using Iridium satellite within Fukuoka FIR, all airline operators are requested to follow the interim procedures issued by AIP-SUP 117/12 below.

  - Aircraft which conducts Air Traffic Service (ATS) data link by using Iridium satellite shall comply with the following operational procedures in addition to GEN3.3.3.7.

2.5.1	Application airspace

Data link airspace over the Pacific Ocean within the Fukuoka FIR.

2.5.2	Application aircraft

Aircraft conducting ATS data link by using Iridium satellite.

    - Operational procedures

Aircraft concerned shall comply with the following procedures.

(1)　 The pilot shall send downlink free text “FOI” (FANS1/A over Iridium) when logging on to the Air Traffic Management Center (ATMC).

(2)　 The pilot shall make a position report via ADS or CPDLC as primary means.

- 2.6	JCAB would appreciate for your cooperation and understanding on collecting data.

- Conclusion

  - The meeting is invited to note the information provided.