THE THIRTY-SEVENTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/37)

(Tokyo, Japan, 29 October - 2 November 2011)

Agenda Item 5:  	Air Traffic Management (ATM) Issue

Status Update on Introduction of 10 Minute Longitudinal Separation

Without Mach Number Technique within Fukuoka FIR

(Presented by JCAB)

SUMMARY

This paper provides an update on introduction of 10 minute longitudinal separation without the mandatory application of Mach Number Technique within Fukuoka FIR.

- Introduction

  - 10 minute longitudinal separation without Mach Number Technique has been applied on NOPAC Routes and other ATS Routes in Fukuoka FIR on the condition that position reports are obtained within 350nm and 370nm respectively, 40 minutes in terms of time both.

  - It is necessary to review the position report which is every 10°longitude when 10 minute longitudinal separation without Mach Number Technique is applied on PACOTS Routes. JCAB and ENRI had analyzed the data from May  2011 to April 2012 and provided the midterm report in IPACG36.

- Discussion

  - The result of safety assessment satisfied the Target Level of Safety (TLS)  applicable to judging the safety on PACOTS Routes. However, the position reporting interval was much wider, compared with the one that are applied on NOPAC Routes and other ATS Routes,

  - Since the data analysis method is different between the analysis on NOPAC Routes and PACOTS Routes.

  - The method is different from “ a gain-loss distribution analysis” on NOPAC Routes, the revised method on PCOTS Routes considers the relative speed effect for a pair of aircraft. This method has been reviewed and discussed in the IPACG32[1] and IPACG35[2] meeting and ICAO RASMAG16 meeting[3].

  - It is conceivable position reporting interval can be set wider by re-analysing NOPAC Routes and other ATS Routes on this method.

  - By re-analysing for NOPAC Routes and other ATS Routes, the same position reporting interval     would be applied within whole Fukuoka FIR

- Conclusion

  - The meeting is invited to note the information provided.

References

[1] “Application of 10 min separation minimum without MNT within Fukuoka FIR,” IP/16, IPACG/32, Honolulu, Hawaii, USA, 10-14 May, 2010.

[2] “Risk Analysis of 10 Minutes Longitudinal Time Separation on PACOTS Routes,” WP/08, IPACG/35, Sapporo, Japan, 7-11 November, 2011.

[3] “Refined Calculation Method for Risk Analysis of Longitudinal Time Separation,” WP/03, ICAO RASMAG/16, Bangkok, Thailand, 20-24 February, 2012.