THE THIRTY-SEVENTH MEETING OF THE

INFORMAL PACIFIC ATC CO-ORDINATING GROUP

(IPACG/37)

(Tokyo, Japan, 29 October - 2 November 2012)

Agenda Item 5:  		Air Traffic Management (ATM) Issues

AUTOMATIC DEPENDENT SURVEILLANCE – BROADCAST (ADS-B)

IN-TRAIL PROCEDURES (ITP) OPERATIONAL

FLIGHT TRIAL PROJECT OVERVIEW

(Presented by the Federal Aviation Administration)

SUMMARY

The purpose of this information paper is to present the U.S. Federal Aviation Administration (FAA) activities associated with the ADS-B In-Trail Procedures (ITP) operational trial being conducted in the Pacific.

- Introduction

  - The U.S. Federal Aviation Administration (FAA) Surveillance and Broadcast Services (SBS) Program oversees and directs the acquisition of a number of surveillance and broadcast services in FAA-specified service volumes in U.S. en route airspace, terminal area airspace, and on airport surfaces.

  - The SBS Program is developing a number of airborne Automatic Dependent Surveillance - Broadcast (ADS-B) applications to provide benefits to operators who choose to equip their aircraft with appropriate avionics, including "ADS-B In" (i.e. the ability to receive, process, and display ADS-B data from surrounding aircraft).  One such airborne ADS-B application being developed is ADS-B In-Trail Procedures (ITP).

  - Aircraft operating in oceanic airspace are, at times, held at non-optimal flight levels due to conflicting traffic either at the desired flight level or at flight levels between the existing flight level and the optimal flight level.  The use of flight level changes enabled by ADS-B ITP can supplement existing oceanic procedures creating greater operational efficiency.

  - The purpose of this paper is to provide an overview of the development of the ADS-B ITP and preliminary results from the operational trial that began in August 2011.

- Background

  - ADS-B ITP is comprised of a set of six flight level change geometries with each geometry dictated by whether the ITP aircraft desires to climb or descend and its proximate relationship with the other aircraft:

• Leading climb 		• Leading descent

• Following climb 		• Following descent

• Combined climb		• Combined descent

While there is no limit on the total climb authorized in the ADS-B ITP flight level change, the other aircraft cannot be more than 2,000 feet above or below the ADS-B ITP aircraft’s altitude.

  - For ADS-B ITP, the maneuvering (trailing or leading) aircraft obtains the flight identification (ID), altitude, position and ground speed transmitted by proximate ADS-B equipped non-manuevering (leading or trailing) aircraft.  Based on the ADS-B data from the non-maneuvering, or reference aircraft, a pilot can request clearance for an ITP altitude change to air traffic control (ATC).  The controller verifies that the ITP and reference aircraft are same direction traffic and that the maximum closing Mach differential is less than or equal to a Mach Number of 0.06.  If the controller determines that the requesting aircraft will maintain standard separation minima with all aircraft other than the ITP reference aircraft, a clearance for the climb or descent may be issued.  After re-validating that the ITP initiation criteria are still valid, the maneuvering aircraft may then vertically transition through the altitude of the non-maneuvering aircraft.

  - To perform an ADS-B ITP, the aircraft desiring to climb or descend must be equipped with an ADS-B transceiver and an appropriate on-board decision support system, both of which have to be certified for this application.  Aircraft operators choosing to equip in this manner would be able to take advantage of this procedure when operating in proximity to aircraft equipped with a suitable ADS-B transmitter (“ADS-B Out”).

  - Aircraft operators who choose to equip with an ADS-B transceiver and on-board automation will benefit from the ability to perform in-trail maneuvers to achieve more time at optimum or operator-preferred altitudes.  This should result in more fuel-efficient and predictable flight profiles, as well as allow operators to better avoid undesirable (due to winds or turbulence) altitudes.  Aircraft operators have also indicated there may be other potential benefits associated with increased cockpit situational awareness resulting from ADS-B In traffic displays.

  - Detailed benefits analyses for ADS-B ITP have focused on savings that could be achieved in the North Atlantic Organized Track System (NATOTS), Pacific Organized Track System (PACOTS) and in the South Pacific (SOPAC) airspace, including flights that operate between the west coast of the United States and Australia or New Zealand.

  - A summary of the ITP criteria are as follows:

      - Maximum of two reference (target) aircraft; +/- 2000 feet from ITP aircraft altitude

      - Reference aircraft can be any combination of ahead of or behind the ITP aircraft

      - ITP aircraft can climb or descend at no less than 300 feet per minute

      - Initiated with no closer than 15 NM and no more than 20 knots of closure.

Note: for the purposes of the initial phase of the operational evaluation the criteria has been modified to be 18 NM.

      - The closing Mach number difference must be less than or equal to Mach Number of 0.06

      - ITP aircraft must maintain Mach number in climb

      - Reference aircraft must be level and non-maneuvering

      - ITP aircraft must have certified ITP equipment onboard as well as CPDLC

      - Reference aircraft must have valid ADS-B Out signal

- FAA ADS-B ITP Project

  - ADS-B ITP has been under development for over eight years. The work supporting the development of ITP has ranged from batch simulations to human-in-the-loop experiments and has included avionics and separation standards development.  Some of these activities included: U.S. National Aeronautics and Space Administration (NASA) report detailing results from a human-in-the-loop experiment that investigated the viability of ADS-B ITP from a cockpit prospective; a NASA report detailing results from a joint Airservices Australia and NASA validation experiment of the ATC procedures associated with ADS-B ITP; ADS-B ITP Safety, Performance and Interoperability Requirements (SPR) Documents DO-312 and ED-159; development and approval of an ICAO Separation and Airspace Safety Panel (SASP) ITP Circular; and inclusion of ITP standards into RTCA DO-317A/ED-194 (Minimum Operational Performance Standards (MOPS) for Aircraft Surveillance Applications (ASA) System), FAA TSO-C195A (Avionics Supporting Automatic Dependent Surveillance – Broadcast (ADS-B)) and FAA AC20-172A (Airworthiness Approval for ADS-B In Systems and Applications).

  - In 2008, the FAA SBS program established a project for the purpose of enabling an operational evaluation of ADS-B ITP by aircraft operating in revenue service. The objectives of the project were to a) validate the operational performance and economic benefits of ITP; and b) develop and validate ADS-B ITP Minimum Operational Performance Specifications (MOPS).

  - As a part of this project, the FAA established agreements with United Airlines and Honeywell to accomplish the work necessary for the development, certification and installation of on-board systems for 12 United Airlines Boeing 747-400s. Goodrich Aerospace was selected by the partners to be the project’s Electronic Flight Bag (EFB) supplier.  These on-board systems calculate the ADS-B ITP criteria and display that information to the pilot.  Honeywell was primarily responsible for the development of the traffic computer and traffic computer software, as well as the ITP display software.  Goodrich Aerospace was responsible for the EFB hardware and operating system software.  United Airlines has been responsible for overall coordination and installation of ITP equipment, obtaining operational approval and conducting the flight evaluation.

  - The entire ITP system was certified for use on a United Boeing 747 in June 2011.  United Airlines subsequently received Operational Approval from FAA Flight Standards to commence ITP operations on 15 August 2011.

  - The FAA developed ATC procedures and conducted the safety analyses that were required to support the flight evaluation.  The ATC procedures were developed and tested using the FAA’s Dynamic Simulation (DYSIM) simulation system.  Safety analyses for the operational flight evaluation were completed following the FAA’s Safety Management System procedures and resulted in an ITP Safety Risk Management Document that was approved by the FAA’s safety regulator in June 2011.

  - FAA En Route and Oceanic Safety and Operations Support authorized Oakland Air Route Traffic Control Center (KZAK) to initiate the operational evaluation in the SOPAC airspace on 15 August 2011.  This authorization was expanded to include the entire Oakland Oceanic Flight Information Region (FIR) in December 2011.

  - The original authorization was scheduled to expire in August of 2012. The authorization was recently amended to permit ADS-B ITP operations in the Oakland Oceanic FIR until August of 2013.

- ADS-B ITP Operational Evaluation

  - As stated previously, the goal of the FAA ITP project is to enable an operational evaluation of ADS-B ITP in revenue service.  Details of the operational flight evaluation are outlined below.

  - Scope:

- The operational evaluation is being conducted using ITP-equipped United Airlines Boeing 747-400’s operating in the Oakland Oceanic FIR.

- There is a comprehensive designated data collection activity for both United Airlines and Oakland ARTCC (ZOA).  Any significant adverse operational issues that are discovered (such as communication or workload) that cannot be safely mitigated will result in an immediate suspension of all operational evaluation activity.

- The data collected is being used to enhance the understanding of the economic, safety and operational impact of ADS-B ITP.

- All pilots and air traffic controllers authorized to participate have completed approved training.

- The operational evaluation flights will be conducted for a period of at least one year

- This operational evaluation has been adopted as an initiative within the Asia and South Pacific Initiative to Reduce Emissions (ASPIRE) program. The ASPIRE program allows partner organizations to share data and provides a mechanism for providing mutual support of the operational evaluation.

  - Objectives

- Validate that air traffic controllers applying ADS-B ITP find it a useful tool.

- With data collected from United Airlines, validate the impact on contingency fuel loading by dispatch and flight crews when they are aware that the aircraft will be ITP capable.

- From post flight surveys, determine the significance and value of increased situation awareness for the flight crew.

- Determine how often the ADS-B ITP is requested, and how often an ITP clearance is issued.

- Determine a measurable reduction in fuel burn on flights with the combined benefit of situation awareness and ITP climbs.

- Validate SPR assumptions and gather data to support validation of the MOPS currently under development for ADS-B ITP.

  - Controller Pilot Data Link Communications (CPDLC) messages are being used during the operational trail.  Until ITP message sets are built into airborne and ground based communication platforms, standardized free text messages (developed and endorsed by ICAO) and concatenated with standard CPDLC vertical request clearances, will be used.

  - The FAA is currently in discussions with the air navigation service providers (ANSPs) for New Zealand and Fiji about expanding the ITP operational evaluation into the Nadi FIR and the Auckland Oceanic FIR in late 2012. The FAA has also held discussions with the Japan Civil Aviation Bureau about the potential for offering ITP in the Fukuoka FIR at some point in the future.

- Operational Evaluation Data Collection Plan and Preliminary Results

  - A comprehensive data collection and analysis process was developed for the operational evaluation.  This data will be used to validate operational performance and economic benefits of ITP, validate safety requirements and assumptions and monitor operational hazards that could lead to suspension of the operational flight trial.

  - There are seven different types of data being collected during the operational flight evaluation. The seven type of data are:

      - United Airlines computer data – supports analysis of the economic benefits of ITP

      - Dispatch comment sheet – captures comments from dispatchers about fuel planning and consumption from pre-flight planning to arrival

      - CPDLC messages – captures ITP events and related messages

      - ADS-C position reports – capture position information of aircraft involved in an ITP event

      - Controller comment sheet – provides an opportunity for controllers to provide feedback on ITP events

      - Pilot comment sheet – captures pilots comments on subjects including fuel planning, situation awareness, operational impacts and comments on human factors aspects of ITP

      - Pilot data sheet – captures information about the change of ITP distance during an ITP event and the duration of the flight level change

5.2        This data is collected, analyzed and used to address key higher level metrics and hazard tracking.  Attachment A includes a summary of the key metrics that are being tracked and the summary of results from ITPs performed on the first day of the ITP operational evaluation.

  - The data contained in the tables involved two ITP equipped United Airlines Boeing 747s flying on the same day in the same airspace (one flight operated from LAX-SYD flight and the other flight from SFO-SYD).  Most of the ITPs performed that evening were between the two United Airlines aircraft.  They were valid operational ITPs performed in non-radar airspace applying the ITP separation standard, but United Airlines intentionally conducted extra climbs and descents to allow the partners an opportunity to gather additional data on ITP.  United Airlines also routed the aircraft so the flight paths would facilitate ITP events.  As a result, the business case numbers were not valid for these series of flights.  All other data is valid and provides insight into the application of ADS-B ITP in the SOPAC airspace.

  - Between September 2011 and July 2012, seven ITP requests were made resulting in three standard climb clearances granted and no ITP climb clearances.  The low numbers of ITP requests and clearances is primarily due to the low number of ITP trained pilots (about 14%) combined with the requirement that both pilots in the control seats be trained in order to make an ITP request.  As the percentage of trained pilots increase, there should be a corresponding increase in ITP requests and clearances granted.

  - Even with only seven ITP requests, there were some significant lessons learned from those requests. A summary of the requests and results are contained in Attachment B.  Note, the events and aircraft identified in the events have been de-identified.

  - In August of 2012 there were eight ITP requests received by ZOA.  Two of these requests resulted in ITP clearances, four resulted in non-ITP clearances and two requests were denied due to operational reasons.  These results are shown in attachments C and D.

  - Attachment C is a summary of the eight requests received in August 2012.

    - Event IDs 114 and 540 resulted in ITP clearances.  Event ID 114 was made with an ITP distance of 21 NM and resulted in a 3000 foot climb.  Event ID 540 was made with an ITP distance of 56 NM and resulted in a 2000 foot climb.

    - Event IDs 1044, 513, 8910 and 986 resulted in non-ITP clearances of 2000 feet each.  Even though ITP was not used for the climb it is possible the increased situation awareness from the ITP system was used to encourage the crews to ask for the altitude changes.

    - Event IDs 4136 and 7110 resulted in no altitude changes.  For event ID 4136, the ADS-C and CPDLC system was temporarily inoperative and an ITP clearance could not be granted.  For event ID 7110, the ITP distance in the request was 17 NM. Although the ADS-B ITP concept is based on an initial ITP distance of 15 NM, for the purposes of the initial phase of the operational evaluation, the criteria has been modified to be 18 NM.  Once the ground automation system used by ZOA (ATOP) has been modified to include ITP clearances, the FAA plans to go back to the 15 NM initial ITP distance.

  - Attachment D is a summary of the resulting “application validation metrics” and “safety measurements” from the two ITP clearances granted in August of 2012.  The safety measurements show that the results are more conservative than the expected measures.

- Conclusion

  - The meeting is invited to note the information provided.

ATTACHMENT A - August 15, 2011 Operational Evaluation Results

Operational and Economic Metrics

Safety related metrics

ATTACHMENT B - Summary of ITP requests from September 2011 through July 2012

| Event | Request | Result | Observation |
| --- | --- | --- | --- |
| 1 | climb 2000 ft (FL320-FL340) | No altitude change | Near ZOA/ZAN boundary; desired altitude not available in ZAN |
| 2 | climb 2000 ft (FL300-FL320) | Cleared for climb of 2000 feet using 30/30 | Controller applied 30/30 separation |
| 3 | climb 2000 ft (FL320-FL340) | No altitude change | Valid reference aircraft not included in ITP request (ADS-B inop on reference aircraft?) |
| 4 | descend 3000 ft (FL390-FL360) | No altitude change | Pilot referenced an aircraft on a nearly parallel track, 100 nm south of their position; believe that flight crew did not use lateral filter |
| 5 | climb 2000 ft (FL330-FL350) | Cleared standard climb to FL340 (1000 foot climb) | Head on/crossing traffic most likely out of range of ITP aircraft |
| 6 | climb 2000 ft (FL290-FL310) | No altitude change | ITP Distance was 15nm; until ATOP is upgraded to support ITP, FAA has instructed controllers to require 18nm ITP Distance in order to approve the procedure |
| 7 | climb 2000 ft (FL340-FL360) | Cleared standard climb | Traffic was 112nm from requesting aircraft; flight crew exercising the ITP system |

ATTACHMENT C - Summary of August 2012 ITP requests

ATTACHMENT D - August 2012 Operational Evaluation Results

— END —